3GPP TSG-RAN WG4 Meeting #109 														    R4-2319170

# Chicago, US, November 13 – 17, 2023

Agenda item:			8.30.2.1.2 Tx requirements

Source:					LG Electronics

Title:	On Tx requirements for NR sidelink evolution

Document for:	Discussion

# Introduction

Based on the WF on NR sidelink evolution [1] and the submitted T-doc [2], we provide proposals on MPR and A-MPR for NR SL-U single carrier operation.

# UE TX requirements for NR SL-U single carrier operation

## SL-U PSSCH/PSCCH MPR

Table 2-1 shows simulation scenarios for SL-U PSSCH/PSCCH MPR.

Table 2-1: SL-U PSSCH/PSCCH MPR/A-MPR simulation scenarios

|  | Scenario | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 5 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 6 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation (Interlace_0) | 7 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 8 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 9 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 10 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 11 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band Full operation | 13 | CP-OFDM | 40 | Bitmap 10 | 15 |
|  | 14 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 15 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 16 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 17 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 18 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 21 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 22 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 23 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 29 | CP-OFDM | 100 | Bitmap 00100 | 30 |
| Wide band Interlaced Operation (Interlace_0) | 30 | CP-OFDM | 40 | Bitmap 10 | 15 |
|  | 31 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 32 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 33 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 34 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 35 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 36 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 37 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 38 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 39 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 40 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 41 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 42 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 43 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 44 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 45 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 46 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Here, for RB set, ‘105RB0’ means that RB starts at RB index ‘0’ and transmitted RB size is 105 RB. And, Bitmap ‘1’ means the corresponding RB set is transmitted, and ‘0’ means the corresponding RB set is not transmitted for wide band operation.

Table 2-2 shows the all possible bitmap of sub-band configuration for wide band operation. The wide band operation can be aggregated with multiple of 20MHz based sub-band. For the SL-U PSSCH/PSCCH MPR simulation, only contiguous RB set bitmaps are considered.

Table 2-2: All possible RB set Bitmap of sub-band configuration

|  | Contiguous RB sets Bit map | Non-contiguous RB sets Bit map |
| --- | --- | --- |
| 20MHz | N/A | N/A |
| 40MHz | 11, 10, 01 | N/A |
| 60MHz | 111, 110, 011, 100, 001, 010 | 101 |
| 80MHz | 1111, 1110, 0111, 1100, 0011, 1000, 0001, 0110, 0100, 0010 | 1001, 1101, 1011, 1010, 0101 |
| 100MHz | 11111, 11110, 01111, 11100, 00111, 11000, 00011, 10000, 00001, 01110, 01100, 00110, 01000, 00010, 00100 | 10001, 11011, 11010, 01011, 11001, 10011, 10101, 10110, 01101, 10100, 00101, 10010, 01001, 01010, 11101, 10111 |

From the simulation results in [2], the PSSCH/PSCCH MPR for SL-U power class 5 can be proposed as Table 2-4.

Table 2-4 Maximum power reduction (MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full2 (dB) | Partial3 (dB) | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full4 (dB) |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 1.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 3.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 2.5 apply. |  |  |  |  |  |  |

Here, the outer RB set configuration and inner RB set configuration are applied with contiguous sub-band configuration only for PSSCH/PSCCH.

- PSSCH/PSCCH MPR/A-MPR

  - Contiguous sub-band configuration only in Table 2-5

- PSFCH & S-SSB MPR/A-MPR

  - Both contiguous sub-band configuration and non-contiguous sub-band configuration in Table 2-5

Table 2-5: Outer/Inner sub-band configuration for SL-U wideband operation

| Wideband operation channel bandwidth (MHz) | Contiguous sub-band configuration |  | Non-contiguous sub-band configuration |  |
| --- | --- | --- | --- | --- |
|  | Outer | Inner | Outer | Inner |
| 40 | 11, 10, 01 | N/A | N/A | N/A |
| 60 | 111, 110, 011, 100, 001 | 010 | 101 | N/A |
| 80 | 1111, 1110, 0111, 1100, 0011, 1000, 0001 | 0110, 0100, 0010 | 1101, 1011, 1010, 0101, 1001 | N/A |
| 100 | 11111, 11110, 01111, 11100, 00111, 11000, 00011, 10000, 00001 | 01110, 01100, 00110, 01000, 00010, 00100 | 11011, 11010, 01011, 11001, 10011, 10101, 10110, 01101, 10100, 00101, 10010, 01001, 11101, 10111, 10001 | 01010 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |  |  |

#### Proposal 1: For SL-U PSSCH/PSCCH MPR, consider the inner sub-band configuration and outer sub-band configuration in Table 2-5.

#### Proposal 2: For SL-U PSSCH/PSCCH MPR, consider Table 2-4 for SL-U UE power class 5.

## SL-U PSSCH/PSCCH A-MPR

For SL-U PSSCH/PSCCH A-MPR of the NS values, the same test scenarios in Table 2-1 are considered.

### 2.2.1 NS_31 (A-MPR)

From the simulation results in [2], Table 2-7 shows the maximum value of simulation results.

Table 2-7 : NS_31-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full(dB) | Partial (dB) |
| CP-OFDM | QPSK | 4.67 | 6.32 | 3.52 | 6.30 |
|  | 16 QAM | 4.68 | 6.32 | 3.52 | 6.30 |
|  | 64 QAM | 4.67 | 6.32 | 3.52 | 6.30 |
|  | 256 QAM | 5.48 | 6.32 | 5.88 | 6.30 |

A-MPR for inner RB set configuration is lower than that for outer RB set allocation for QPSK, 16QAM and 64QAM in case of Full RB allocation.

Based on the results, Table 2-8 can be considered for NS_31 A-MPR for SL-U UE power class 5.

Table 2-8. NS_31 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 4.5 | ≤ 6.5 | Follow SL-U MPR table |
|  | 16 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 2-5 apply. |  |  |  |  |  |  |

#### Proposal 3: For NS_31 SL-U PSSCH/PSCCH A-MPR, consider the inner sub-band configuration and outer sub-band configuration in Table 2-5.

#### Proposal 4: For NS_31 SL-U PSSCH/PSCCH A-MPR, consider Table 2-8 for SL-U UE power class 5.

### 2.2.2 NS_53 (A-MPR)

From the simulation results in [2], Table 2-10 shows the maximum value of simulation results.

Table 2-10 : NS_53-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 8.51 | 11.31 | 5.47 | 8.06 | 3.90 | 6.31 | 2.82 | 4.67 | 3.17 | 3.90 |
|  | 16 QAM | 8.96 | 11.32 | 5.47 | 8.06 | 3.90 | 6.31 | 2.82 | 5.06 | 3.17 | 3.90 |
|  | 64 QAM | 8.96 | 11.31 | 5.47 | 8.06 | 3.90 | 6.31 | 2.82 | 4.67 | 3.17 | 3.90 |
|  | 256 QAM | 8.96 | 11.32 | 5.47 | 8.06 | 5.47 | 6.31 | 5.47 | 5.07 | 5.47 | 5.07 |

Based on the results, Table 2-11 can be considered for NS_53 A-MPR for SL-U UE power class 5.

Table 2-11. NS_53 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

#### Proposal 5: For NS_53 SL-U PSSCH/PSCCH A-MPR, consider Table 2-11 for SL-U UE power class 5.

### 2.2.3 NS_58 (A-MPR)

From the simulation results in [2], Table 2-13 shows the maximum value of simulation results.

Table 2-13 : NS_58-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full(dB) | Partial (dB) |
| CP-OFDM | QPSK | 2.81 | 2.83 | 2.47 | 1.20 |
|  | 16 QAM | 2.81 | 2.83 | 2.47 | 1.20 |
|  | 64 QAM | 3.18 | 2.83 | 3.16 | 2.47 |
|  | 256 QAM | 5.48 | 5.09 | 5.88 | 5.06 |

Based on the results, Table 2-14 can be considered for NS_58 A-MPR for SL-U UE power class 5.

Table 2-14. NS_58 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |
|  |  | Full (dB)2 | Partial (dB)3 | Full (dB) 2 | Partial (dB) 3 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 2-5 apply. |  |  |  |  |  |

#### Proposal 6: For NS_58 SL-U PSSCH/PSCCH A-MPR, consider the inner sub-band configuration and outer sub-band configuration in Table 2-5.

#### Proposal 7: For NS_58 SL-U PSSCH/PSCCH A-MPR, consider Table 2-14 for SL-U UE power class 5.

### 2.2.4 NS_60 (A-MPR)

From the simulation results in [2], Table 2-16 shows the maximum value of simulation results.

Table 2-16 : NS_60-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 5.89 | 8.07 | 2.48 | 5.07 | 3.90 | 4.28 | 2.82 | 3.53 | 3.17 | 3.90 |
|  | 16 QAM | 5.89 | 8.51 | 2.48 | 5.07 | 3.90 | 4.28 | 2.82 | 3.53 | 3.17 | 3.90 |
|  | 64 QAM | 5.89 | 8.52 | 3.16 | 5.07 | 3.90 | 4.28 | 2.82 | 3.53 | 3.17 | 3.90 |
|  | 256 QAM | 5.89 | 8.52 | 5.47 | 5.07 | 5.88 | 5.48 | 5.47 | 5.07 | 5.47 | 5.06 |

Based on the results, Table 2-17 can be considered for NS_60 A-MPR for SL-U UE power class 5.

Table 2-17. NS_60 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

#### Proposal 8: For NS_60 SL-U PSSCH/PSCCH A-MPR, consider Table 2-17 for SL-U UE power class 5.

### 2.2.5 NS_61 (A-MPR)

From the simulation results in [2], Table 2-19 shows the maximum value of simulation results considering Full/Partial RB allocation.

Table 2-19 : NS_61-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 6.74 | 9.44 | 4.67 | 5.89 | 5.06 | 5.47 | 4.67 | 5.47 | 4.67 | 5.47 |
|  | 16 QAM | 6.74 | 9.43 | 4.67 | 5.89 | 5.07 | 5.47 | 4.67 | 5.47 | 4.67 | 5.47 |
|  | 64 QAM | 6.74 | 9.44 | 4.67 | 6.30 | 5.07 | 5.47 | 4.67 | 5.47 | 4.67 | 5.47 |
|  | 256 QAM | 6.74 | 9.43 | 5.47 | 6.31 | 5.47 | 5.47 | 5.47 | 5.47 | 5.47 | 5.47 |

For NS_61, the maximum output power is limited with 14dBm. Therefore, basically, A-MPR should be met with the following equation considering power class 5, 20dBm.

- NS_61 A-MPR = max{Table 2-19 + implementation margin, 6 dB}.

Based on the results, Table 2-20 can be considered for NS_61 A-MPR for SL-U UE power class 5.

Table 2-20. NS_61 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

#### Proposal 9: For NS_61 SL-U PSSCH/PSCCH A-MPR, consider Table 2-20 for SL-U UE power class 5.

Regarding that NS_61 applies to n96 same as NS_53 and NS_60, and NS_53 and NS_60 support channel bandwidth of {20MHz, 40MHz, 60MHz, 80MHz, 100MHz}, NS_61 can apply to channel bandwidth of 100MHz on top of NR-U NS_61 channel bandwidth of {20MHz, 40MHz, 60MHz, 80MHz}.

#### Proposal 9A: For NS_61 SL-U PSSCH/PSCCH A-MPR, consider the channel bandwidth of 100MHz on top of NR-U NS_61 channel bandwidth of {20MHz, 40MHz, 60MHz, 80MHz}.

## SL-U PSFCH  MPR

The following simulation assumption was captured from WF[1] .

Table 2.21: SL-U UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - Power per RB is same in PSFCH for all users - Total power is 20dBm - Single RB-set [and multiple RB-sets] will be considered based on RAN1 decision. - For single RB-set, RAN4 consider interlacing RBs for PSFCH - For multiple RB-sets, RAN4 consider both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of interlaced transmission should meet at least 80% of channel bandwidth in a single RB-set i.e. [, Ngap = RBend – RBstart ]. - RB allocation method as NR SL legacy RB allocation is also considered |

RAN1 agreement on PSFCH transmission

Agreement In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”, regarding common interlace, index of common interlace is (pre-)configured per resource pool. Agreement Regarding PSFCH transmission, - For “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)” - Cyclic shift on each of K3 dedicated PRB(s) is the same - Cyclic shift on each PRB of common interlace is up to UE implementation - For “Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace” - Support PRB-level cyclic shift hopping as in NR-U to reduce PAPR - The power for a PSFCH transmission is equally allocated over the occupied PRBs Working assumption - In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”: - Assume the UE transmits N PSFCH - Denote the final Tx power on one common PRB is P_common - Denote the final Tx power on one dedicated PRB is P_dedicated - P_common <= P_dedicated - (pre-)configure an offset between P_common and P_dedicated

Table 2.22 shows simulation scenarios based on Alt 1-1b. Here, each PSFCH transmission power is assumed as equal power.

Table 2.22: SL-U PSFCH MPR/A-MPR simulation scenarios

| Sub-band RB sets | Scenario | Bitmap | PSFCH RB location index | SCS(kHz) |
| --- | --- | --- | --- | --- |
| 1 (20MHz) | 1 | 1 | {0 10 20 30 40 50 60 70 80 90 100 104} | 15 |
|  | 2 | 1 | {0 5 10 15 20 25 30 35 40 45 49} | 30 |
| 2 (40MHz) | 3 | 11 | {0 10 20 30 40 50 60 70 80 90 100 104 111 121 131 141 151 161 171 181 191 201 211 215} | 15 |
|  | 4 | 10 | {0 10 20 30 40 50 60 70 80 90 100 104} | 15 |
| 3 (60MHz) | 5 | 111 | {0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 6 | 110 | {0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 7 | 100 | {0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 8 | 010 | {56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 9 | 101 | {0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161} | 30 |
| 4 (80MHz) | 10 | 1111 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 11 | 1110 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 12 | 1100 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 13 | 1000 | { 0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 14 | 0110 | { 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 15 | 0100 | { 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 16 | 1101 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 17 | 1010 | { 0 5 10 15 20 25 30 35 40 45 49 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 18 | 1001 | { 0 5 10 15 20 25 30 35 40 45 49 167 172 177 182 187 192 197 202 207 212 216} | 30 |
| 5 (100MHz) | 19 | 11111 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 20 | 11110 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 21 | 11100 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 22 | 11000 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 23 | 10000 | { 0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 24 | 01110 | { 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 25 | 01100 | { 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 26 | 01000 | { 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 27 | 00100 | { 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 28 | 11011 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 29 | 11010 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 30 | 11001 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 31 | 10101 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 32 | 10110 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 33 | 10100 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 34 | 10010 | { 0 5 10 15 20 25 30 35 40 45 49 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 35 | 10001 | { 0 5 10 15 20 25 30 35 40 45 49 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 36 | 01010 | { 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 37 | 11101 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 223 228 233 238 243 248 253 258 263 268 272} | 30 |

Here, for PSFCH RB location index, ‘0’ means that RB index = 0. And, Bitmap ‘1’ means the corresponding RB set is transmitted, and ‘0’ means the corresponding RB set is not transmitted for wide band operation.

Table 2-23 and Figure 2-7 show the MPR simulation results for the scenarios.

Table 2-23: PSFCH MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 0.92 | 1.16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 1.76 | 0.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 2.55 | 1.81 | 1.14 | 0.79 | 2.59 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 2.24 | 2.12 | 1.79 | 1.05 | 1.63 | 0.83 | 2.83 | 2.60 | 2.74 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 1.85 | 2.31 | 2.54 | 1.82 | 1.10 | 1.45 | 1.60 | 0.63 | 0.93 | 2.68 | 2.86 | 2.98 | 2.87 | 2.65 | 2.68 | 2.53 | 2.75 | 2.40 | 3.06 |

Figure 2-7: PSFCH MPR simulation results for SL-U power class 5

Table 2-24 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-24 : PSFCH MPR simulation results for SL-U power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration5 | Inner RB set configuration5 |
| Contiguous sub-band RB sets | 2.55 | 1.63 |
| Non-contiguous sub-band RB sets | 3.06 | 2.40 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH MPR for SL-U power class 5 can be proposed as Table 2-25 or Table 2-26 based on the simulation results when considering implementation margin.

Table 2-25 PSFCH MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.5 |
| Non-contiguous sub-band RB sets | ≤ 5.0 | ≤ 4.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |

Table 2-26 PSFCH MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 5.0 | ≤ 4.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |

#### Proposal 10: For SL-U PSFCH MPR and A-MPR, consider simulation scenarios in Table 2-22.

#### Proposal 11: For SL-U PSFCH MPR, consider Table 2-25 or Table 2-26 for SL-U UE power class 5.

## SL-U PSFCH  A-MPR

For SL-U PSFCH A-MPR of the NS values, the same test scenarios in Table 2-22 are considered.

### 2.4.1 NS_31 (A-MPR)

For NS_31, Table 2-27 and Figure 2-8 show the A-MPR simulation results for the scenarios.

Table 2-27: NS_31-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 2.97 | 3.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 3.40 | 3.03 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 3.89 | 3.62 | 5.63 | 5.59 | 3.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 3.54 | 3.62 | 3.58 | 5.64 | 2.67 | 5.64 | 3.27 | 3.19 | 3.18 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 2.95 | 3.63 | 3.88 | 3.62 | 5.65 | 2.31 | 2.60 | 5.62 | 5.51 | 3.04 | 3.23 | 3.22 | 3.40 | 3.48 | 3.12 | 3.14 | 3.13 | 3.19 | 3.55 |

Figure 2-7: NS_31-PSFCH A-MPR simulation results for SL-U power class 5

Table 2-28 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-28 : NS_31 PSFCH A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous sub-band RB sets | 5.65 | 5.64 |
| Non-contiguous sub-band RB sets | 3.55 | 3.19 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 2-29 or Table 2-30 based on the simulation results when considering implementation margin.

Table 2-29 NS_31 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 7.5 | ≤ 7.5 |
| Non-contiguous sub-band RB sets | ≤ 5.5 | ≤ 5.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. |  |  |

Table 2-30 NS_31 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. |  |  |

#### Proposal 12: For SL-U NS_31 PSFCH A-MPR, consider Table 2-29 or Table 2-30 for SL-U UE power class 5.

### 2.4.2 NS_53 (A-MPR)

For NS_53, Table 2-31 and Figure 2-9 show the A-MPR simulation results for the scenarios.

Table 2-31: NS_53-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 10.33 | 10.74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 7.29 | 10.32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 5.88 | 7.63 | 10.73 | 10.72 | 7.64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 4.66 | 5.88 | 7.63 | 10.65 | 7.63 | 10.63 | 5.88 | 7.63 | 7.64 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 3.70 | 4.59 | 5.87 | 7.63 | 10.73 | 5.85 | 7.60 | 10.69 | 10.54 | 4.60 | 5.85 | 5.86 | 5.85 | 5.85 | 7.63 | 7.63 | 7.64 | 7.60 | 4.64 |

Figure 2-8: NS_53-PSFCH A-MPR simulation results for SL-U power class 5

Table 2-32 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-32 : NS_53 PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | 10.74 | 7.63 | 5.88 | 4.66 | 3.70 |
| Non-contiguous | N/A | 7.74 | 5.88 | 4.65 | N/A |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 2-33 or Table 2-34 based on the simulation results when considering implementation margin.

Table 2-33 NS_53 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤12.5 | ≤9.5 | ≤8.0 | ≤6.5 | ≤5.5 |
| Non-contiguous | N/A | ≤9.5 | ≤8.0 | ≤6.5 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 2-34 NS_53 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤12.5 | ≤9.5 | ≤8.0 | ≤6.5 | ≤5.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### Proposal 13: For SL-U NS_53 PSFCH A-MPR, consider Table 2-33 or Table 2-34 for SL-U UE power class 5.

### 2.4.3 NS_58 (A-MPR)

For NS_58, Table 2-35 and Figure 2-9 show the A-MPR simulation results for the scenarios.

Table 2-35: NS_58-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 1.73 | 1.16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 2.12 | 1.88 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 2.59 | 2.03 | 1.14 | 0.00 | 1.99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 2.24 | 2.12 | 2.01 | 1.09 | 0.36 | 0.00 | 2.41 | 2.04 | 2.05 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 1.85 | 2.31 | 2.58 | 2.01 | 1.10 | 0.63 | 0.47 | 0.00 | 0.00 | 2.37 | 2.42 | 2.49 | 2.31 | 2.07 | 1.99 | 1.98 | 1.98 | 1.81 | 2.60 |

Figure 2-9: NS_58-PSFCH A-MPR simulation results for SL-U power class 5

Table 2-36 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-36 : NS_58 PSFCH A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration |
| Contiguous sub-band RB sets | 2.59 | 0.63 |
| Non-contiguous sub-band RB sets | 2.60 | 1.81 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 2-37 or Table 2-38 based on the simulation results when considering implementation margin.

Table 2-37 NS_58 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 4.5 | ≤ 2.5 |
| Non-contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |

Table 2-38 NS_58 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |

#### Proposal 14: For SL-U NS_58 PSFCH A-MPR, consider Table 2-37 or Table 2-38 for SL-U UE power class 5.

### 2.4.4 NS_60 (A-MPR)

For NS_60, Table 2-39 and Figure 2-10 show the A-MPR simulation results for the scenarios.

Table 2-39: NS_60-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 7.27 | 7.69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 4.26 | 7.26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 3.96 | 4.63 | 7.68 | 7.67 | 4.64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 2.79 | 2.93 | 4.64 | 7.63 | 4.63 | 7.64 | 3.27 | 4.64 | 4.65 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 3.15 | 2.66 | 2.89 | 4.63 | 7.68 | 2.88 | 4.60 | 7.61 | 7.53 | 3.11 | 2.88 | 3.40 | 3.22 | 3.02 | 4.63 | 4.63 | 4.64 | 4.61 | 3.53 |

Figure 2-10: NS_60-PSFCH A-MPR simulation results for SL-U power class 5

Table 2-40 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-40 : NS_60 PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | 7.69 | 4.64 | 3.96 | 2.79 | 3.15 |
| Non-contiguous | N/A | 4.65 | 3.40 | 3.53 | N/A |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 2-41 or Table 2-42 based on the simulation results when considering implementation margin.

Table 2-41 NS_60 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤9.5 | ≤6.5 | ≤6.0 | ≤5.5 | ≤5.5 |
| Non-contiguous | N/A | ≤6.5 | ≤6.0 | ≤5.0 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 2-42 NS_60 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤9.5 | ≤6.5 | ≤6.0 | ≤5.5 | ≤5.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### Proposal 15: For SL-U NS_60 PSFCH A-MPR, consider Table 2-41 or Table 2-42 for SL-U UE power class 5.

### 2.4.6 NS_61 (A-MPR)

For NS_61, Table 2-43 and Figure 2-11 show the A-MPR simulation results for the scenarios.

Table 2-43: NS_61-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 8.30 | 8.73 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 5.26 | 8.29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 4.57 | 5.63 | 8.69 | 8.68 | 5.64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' | 4.39 | 4.30 | 5.63 | 8.64 | 5.63 | 8.65 | 4.67 | 5.63 | 5.64 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 |
| '100MHz' | 4.19 | 4.47 | 4.53 | 5.63 | 8.69 | 3.87 | 5.59 | 8.65 | 8.53 | 4.39 | 4.64 | 4.60 | 4.74 | 3.87 | 5.63 | 5.63 | 5.63 | 5.60 | 4.57 |

Figure 2-11: NS_61-PSFCH A-MPR simulation results for SL-U power class 5

Table 2-44 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-44 : NS_61 PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | 8.73 | 5.63 | 4.57 | 4.47 | 4.19 |
| Non-contiguous | N/A | 5.64 | 4.74 | 4.57 | N/A |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the PSFCH A-MPR for SL-U power class 5 can be proposed as Table 2-45 or Table 2-46 based on the simulation results when considering implementation margin.

Table 2-45 NS_61 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤10.5 | ≤7.5 | ≤6.5 | ≤6.5 | ≤6.0 |
| Non-contiguous | N/A | ≤7.5 | ≤6.5 | ≤6.5 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 2-46 NS_61 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤10.5 | ≤7.5 | ≤6.5 | ≤6.5 | ≤6.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### Proposal 16: For SL-U NS_61 PSFCH A-MPR, consider Table 2-45 or Table 2-46 for SL-U UE power class 5.

## SL-U S-SSB  MPR

The following simulation assumption was captured in TR [4].

Table 6.1.2-3: SL-U UE’s MPR simulation assumptions for S-SSB transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB set, the 11RB will be repeated N time in a RB set. For the multiple RB-sets, UE may transmit S-SSB repetition in the contiguous multiple RB sets. RBstart: All the possible cases LCRB: 11 RB |

RB allocation in WF[3]

- Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB set, the 11RB will be repeated N time in a RB set. For the multiple RB-sets, RAN4 consider both contiguous RB sets and non-contiguous RB sets.

- N gap from RBstart to RBend of repeated SSB transmission should meet at least 80% of channel bandwidth.in a single RB-set. ( Ngap = RBend – RBstart )

Tx Power for multiple S-SSBs

- The equal power per each S-SSB is assumed for MPR and A-MPR simulation.

Table 2-47 shows simulation scenarios.

Table 2-47: SL-U S-SSB MPR/A-MPR simulation scenarios

| Sub-band RB sets | Scenario | Bitmap | S-SSB {11RBs}xN repeated RB location index | N (Repeated#) | SCS(kHz) |
| --- | --- | --- | --- | --- | --- |
| 1 (20MHz) | 1 | 1 | {0 11 22 33 44 55 66 77 88} | 9 | 15 |
|  | 2 | 1 | {0 23 46 69 92} | 5 | 15 |
|  | 3 | 1 | {0 95} | 2 | 15 |
|  | 4 | 1 | {0 11 22 33} | 4 | 30 |
|  | 5 | 1 | {0 39} | 2 | 30 |
| 2 (40MHz) | 6 | 11 | {0 11 22 33 44 55 66 77 88 111 122 133 144 155 166 177 188 199} | 18 | 15 |
|  | 7 | 11 | {0 23 46 69 92 111 134 157 180 203} | 10 | 15 |
|  | 8 | 11 | {0 94 111 205} | 4 | 15 |
|  | 9 | 10 | {0 11 22 33 44 55 66 77 88} | 9 | 15 |
|  | 10 | 10 | {0 23 46 69 92 } | 5 | 15 |
|  | 11 | 10 | {0 94} | 2 | 15 |
| 3 (60MHz) | 12 | 111 | {0 11 22 33 56 67 78 89 112 123 134 145} | 12 | 30 |
|  | 13 | 111 | {0 38 56 94 112 150} | 6 | 30 |
|  | 14 | 110 | {0 11 22 33 56 67 78 89} | 8 | 30 |
|  | 15 | 110 | {0 38 56 94 } | 4 | 30 |
|  | 16 | 100 | {0 11 22 33} | 4 | 30 |
|  | 17 | 100 | {0 38} | 2 | 30 |
|  | 18 | 010 | {56 67 78 89 } | 4 | 30 |
|  | 19 | 010 | {56 94} | 2 | 30 |
|  | 20 | 101 | {0 11 22 33 112 123 134 145} | 8 | 30 |
|  | 21 | 101 | {0 38 112 150} | 4 | 30 |
| 4 (80MHz) | 22 | 1111 | {0 11 22 33 56 67 78 89 111 122 133 144 167 178 189 200} | 16 | 30 |
|  | 23 | 1111 | {0 38 56 94 111 149 167 205} | 8 | 30 |
|  | 24 | 1110 | {0 11 22 33 56 67 78 89 111 122 133 144} | 12 | 30 |
|  | 25 | 1110 | {0 38 56 94 111 149 } | 6 | 30 |
|  | 26 | 1100 | {0 11 22 33 56 67 78 89} | 8 | 30 |
|  | 27 | 1100 | {0 38 56 94} | 4 |  |
|  | 28 | 1000 | {0 11 22 33} | 4 | 30 |
|  | 29 | 1000 | {0 38} | 2 | 30 |
|  | 30 | 0110 | {56 67 78 89 111 122 133 144} | 8 | 30 |
|  | 31 | 0110 | {56 94 111 49} | 4 | 30 |
|  | 32 | 0100 | {56 67 78 89} | 4 | 30 |
|  | 33 | 0100 | {56 94} | 2 | 30 |
|  | 34 | 1101 | {0 11 22 33 56 67 78 89 167 178 189 200} | 12 | 30 |
|  | 35 | 1101 | {0 38 56 94 167 205} | 6 | 30 |
|  | 36 | 1010 | {0 11 22 33 111 122 133 144} | 8 | 30 |
|  | 37 | 1010 | {0 38 111 149} | 4 | 30 |
|  | 38 | 1001 | {0 11 22 33 167 178 189 200} | 8 | 30 |
|  | 39 | 1001 | {0 38 167 205} | 4 | 30 |
| 5 (100MHz) | 40 | 11111 | {0 11 22 33 56 67 78 89 112 123 134 145 167 178 189 200 223 234 245 256} | 20 | 30 |
|  | 41 | 11111 | {0 38 56 94 112 148 167 205 223 261} | 10 | 30 |
|  | 42 | 11110 | {0 11 22 33 56 67 78 89 112 123 134 145 167 178 189 200} | 16 | 30 |
|  | 43 | 11110 | {0 38 56 94 112 148 167 205} | 8 | 30 |
|  | 44 | 11100 | {0 11 22 33 56 67 78 89 112 123 134 145} | 12 | 30 |
|  | 45 | 11100 | {0 38 56 94 112 148} | 6 | 30 |
|  | 46 | 11000 | {0 11 22 33 56 67 78 89} | 8 | 30 |
|  | 47 | 11000 | {0 38 56 94} | 4 | 30 |
|  | 48 | 10000 | {0 11 22 33} | 4 | 30 |
|  | 49 | 10000 | {0 38} | 2 | 30 |
|  | 50 | 01110 | {56 67 78 89 112 123 134 145 167 178 189 200} | 12 | 30 |
|  | 51 | 01110 | {56 94 112 148 167 205} | 6 | 30 |
|  | 52 | 01100 | {56 67 78 89 112 123 134 145} | 8 | 30 |
|  | 53 | 01100 | {56 94 112 148} | 4 | 30 |
|  | 54 | 01000 | {56 67 78 89} | 4 | 30 |
|  | 55 | 01000 | {56 94} | 2 | 30 |
|  | 56 | 00100 | {112 123 134 145} | 4 | 30 |
|  | 57 | 00100 | {112 148} | 2 | 40 |
|  | 58 | 11011 | {0 11 22 33 56 67 78 89 167 178 189 200 223 234 245 256} | 16 | 30 |
|  | 59 | 11011 | {0 38 56 94 167 205 223 261} | 8 | 30 |
|  | 60 | 11010 | {0 11 22 33 56 67 78 89 167 178 189 200} | 12 | 30 |
|  | 61 | 11010 | {0 38 56 94 167 205} | 6 | 30 |
|  | 62 | 11001 | {0 11 22 33 56 67 78 89 223 234 245 256} | 12 | 30 |
|  | 63 | 11001 | {0 38 56 94 223 261} | 6 | 30 |
|  | 64 | 10101 | {0 11 22 33 112 123 134 145 223 234 245 256} | 12 | 30 |
|  | 65 | 10101 | {0 38 112 148 223 261} | 6 | 30 |
|  | 66 | 10110 | {0 11 22 33 112 123 134 145 167 178 189 200} | 12 | 30 |
|  | 67 | 10110 | {0 38 112 148 167 205} | 6 | 30 |
|  | 68 | 10100 | {0 11 22 33 112 123 134 145} | 8 | 30 |
|  | 69 | 10100 | {0 38 112 148} | 4 | 30 |
|  | 70 | 10010 | {0 11 22 33 167 178 189 200} | 8 | 30 |
|  | 71 | 10010 | {0 38 167 205} | 4 | 30 |
|  | 72 | 10001 | {0 11 22 33 223 234 245 256} | 8 | 30 |
|  | 73 | 10001 | {0 38 223 261} | 4 | 30 |
|  | 74 | 01010 | {56 67 78 89 167 178 189 200} | 8 | 30 |
|  | 75 | 01010 | {56 94 167 205} | 4 | 30 |
|  | 76 | 11101 | {0 11 22 33 56 67 78 89 112 123 134 145 223 234 245 256} | 16 | 30 |
|  | 77 | 11101 | {0 38 56 94 112 148 223 261} | 8 | 30 |

Here, for S-SSB {11RBs}xN repeated RB location index, ‘0’ means that {11RB} start RB index = 0. And, Bitmap ‘1’ means the corresponding RB set is transmitted, and ‘0’ means the corresponding RB set is not transmitted for wide band operation.

Table 2-48 and Figure 2-12 show the MPR simulation results for the scenarios.

Table 2-48: S-SSB MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.24 | 6.62 | 3.98 | 5.69 | 3.92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.17 | 8.63 | 5.56 | 9.02 | 6.95 | 3.94 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.29 | 6.72 | 7.87 | 5.31 | 5.57 | 3.94 | 4.54 | 3.52 | 7.67 | 5.49 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 10.34 | 6.86 | 8.81 | 5.60 | 7.67 | 5.38 | 5.68 | 4.06 | 6.11 | 4.12 | 4.42 | 3.52 | 8.92 | 5.97 | 7.55 | 5.21 | 7.63 | 5.31 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 11.15 | 6.75 | 10.18 | 5.56 | 9.44 | 5.91 | 7.75 | 5.22 | 5.79 | 3.91 | 6.02 | 3.44 | 5.60 | 3.35 | 4.43 | 3.53 | 4.43 | 3.53 | 10.18 | 7.03 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.18 | 6.32 | 8.95 | 6.04 | 8.61 | 4.98 | 9.08 | 4.99 | 7.67 | 5.10 | 7.87 | 5.45 | 7.62 | 5.41 | 6.90 | 4.60 | 10.68 | 5.79 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 2-12: S-SSB MPR simulation results for SL-U power class 5

Table 2-49 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and the number of S-SSB repetition per RB set.

Table 2-49 : S-SSB MPR simulation results for SL-U power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | 11.17 | 6.86 | 6.11 | 4.12 |
| Non-contiguous sub-band RB sets | 10.68 | 7.03 | 6.90 | 4.60 |

The S-SSB MPR for SL-U power class 5 can be proposed as Table 2-50 or Table 2-51 based on the simulation results when considering implementation margin.

Table 2-50 : S-SSB MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 8.5 | ≤ 6.5 |
| Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 9.5 | ≤ 7.0 |
| NOTE 1: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |  |  |

Table 2-51 :  S-SSB MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 9.5 | ≤ 7.0 |
| NOTE 1: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |  |  |

#### Proposal 17: For SL-U S-SSB MPR and A-MPR, consider simulation scenarios in Table 2-47.

#### Proposal 18: For SL-U S-SSB MPR, consider Table 2-50 or Table 2-51 for SL-U UE power class 5.

## SL-U S-SSB  A-MPR

For SL-U S-SSB A-MPR of the NS values, the same test scenarios in Table 2-47 are considered.

### 2.6.1 NS_31 (A-MPR)

For NS_31, Table 2-52 and Figure 2-13 show the A-MPR simulation results for the scenarios.

Table 2-52: NS_31-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.43 | 7.90 | 6.82 | 6.81 | 6.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.05 | 8.92 | 7.45 | 9.20 | 8.17 | 7.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.36 | 7.58 | 8.19 | 6.46 | 6.69 | 7.21 | 4.76 | 7.20 | 8.18 | 6.79 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 9.92 | 7.41 | 8.95 | 6.78 | 7.97 | 6.48 | 6.89 | 7.19 | 5.98 | 4.31 | 4.61 | 7.12 | 9.04 | 6.62 | 7.85 | 6.61 | 7.79 | 6.47 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 10.91 | 7.19 | 9.59 | 6.61 | 9.54 | 6.67 | 7.99 | 6.53 | 6.80 | 7.20 | 6.12 | 4.95 | 5.66 | 4.09 | 4.70 | 7.28 | 4.56 | 7.16 | 9.97 | 7.47 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 8.85 | 6.62 | 9.14 | 6.54 | 8.62 | 6.08 | 8.80 | 5.83 | 8.04 | 6.48 | 7.86 | 6.46 | 8.13 | 6.49 | 7.49 | 6.35 | 10.15 | 6.31 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 2-13: NS_31-S-SSB A-MPR simulation results for SL-U power class 5

Table 2-53 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-53 : NS_31 S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | 11.05 | 7.58 | 6.12 | 7.28 |
| Non-contiguous sub-band RB sets | 10.15 | 7.47 | 7.49 | 6.35 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 2-54 or Table 2-55 based on the simulation results when considering implementation margin.

Table 2-54: NS_31 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 | ≤ 10.0 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

Table 2-55: NS_31 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

#### Proposal 19: For SL-U NS_31 S-SSB A-MPR, consider Table 2-54 or Table 2-55 for SL-U UE power class 5.

### 2.6.2 NS_53 (A-MPR)

For NS_53, Table 2-56 and Figure 2-14 show the A-MPR simulation results for the scenarios.

Table 2-56: NS_53-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.43 | 11.16 | 15.09 | 9.13 | 12.21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.05 | 8.92 | 12.11 | 9.20 | 11.24 | 15.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.36 | 7.58 | 8.19 | 9.19 | 9.12 | 12.22 | 9.09 | 12.22 | 8.18 | 9.12 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 9.92 | 7.32 | 8.95 | 7.43 | 7.99 | 9.19 | 9.19 | 12.23 | 6.36 | 9.21 | 9.22 | 12.13 | 9.03 | 7.47 | 8.09 | 9.10 | 8.31 | 9.20 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 11.13 | 7.20 | 9.76 | 6.37 | 9.55 | 7.44 | 8.15 | 9.21 | 9.24 | 12.28 | 6.04 | 7.38 | 6.28 | 9.18 | 9.22 | 12.19 | 9.13 | 12.22 | 10.26 | 7.48 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.47 | 7.36 | 8.74 | 7.40 | 8.46 | 7.45 | 8.90 | 7.44 | 8.10 | 9.19 | 8.09 | 9.14 | 8.08 | 9.29 | 7.62 | 9.18 | 10.07 | 6.45 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 2-14: NS_53 S-SSB A-MPR simulation results for SL-U power class 5

Table 2-57 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-57 : NS_53 S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | 11.16 | 15.09 | 11.05 | 15.09 | 9.36 | 12.22 | 9.92 | 12.23 | 11.13 | 11.13 |
| Non-contiguous | - | - | - | - | 8.18 | 9.12 | 9.03 | 9.20 | 10.26 | 9.29 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 2-58 or Table 2-59 based on the simulation results when considering implementation margin.

Table 2-58: NS_53 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤17.5 | ≤13.5 | ≤17.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 2-59: NS_53 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤17.5 | ≤13.5 | ≤17.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### Proposal 20: For SL-U NS_53 S-SSB MPR, consider Table 2-58 or Table 2-59 for SL-U UE power class 5.

### 2.6.3 NS_58 (A-MPR)

For NS_58, Table 2-60 and Figure 2-15 show the A-MPR simulation results for the scenarios.

Table 2-60: NS_58-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.43 | 7.90 | 6.82 | 6.81 | 6.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.05 | 8.92 | 7.45 | 9.20 | 8.17 | 7.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.36 | 7.58 | 8.19 | 6.46 | 6.69 | 7.21 | 4.76 | 7.20 | 8.18 | 6.79 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 9.92 | 7.41 | 8.95 | 6.78 | 7.97 | 6.48 | 6.89 | 7.19 | 5.98 | 4.31 | 4.61 | 7.12 | 9.04 | 6.62 | 7.85 | 6.61 | 7.79 | 6.47 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 10.91 | 7.19 | 9.59 | 6.61 | 9.54 | 6.67 | 7.99 | 6.53 | 6.80 | 7.20 | 6.12 | 4.95 | 5.66 | 4.09 | 4.70 | 7.28 | 4.56 | 7.16 | 9.97 | 7.47 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 8.85 | 6.62 | 9.14 | 6.54 | 8.62 | 6.08 | 8.80 | 5.83 | 8.04 | 6.48 | 7.86 | 6.46 | 8.13 | 6.49 | 7.49 | 6.35 | 10.15 | 6.31 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 2-15: NS_58-S-SSB A-MPR simulation results for SL-U power class 5

Table 2-61 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-61 : NS_58 S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | 10.91 | 6.86 | 6.11 | 3.98 |
| Non-contiguous sub-band RB sets | 9.94 | 7.22 | 6.93 | 4.82 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 2-62 or Table 2-63 based on the simulation results when considering implementation margin.

Table 2-62: NS_58 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 8.5 | ≤ 6.5 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 | ≤ 9.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |  |  |

Table 2-63: NS_58 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 9.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |  |  |

#### Proposal 21: For SL-U NS_58 S-SSB A-MPR, consider Table 2-62 or Table 2-63 for SL-U UE power class 5.

### 2.6.4 NS_60 (A-MPR)

For NS_60, Table 2-64 and Figure 2-16 show the A-MPR simulation results for the scenarios.

Table 2-64: NS_60-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.24 | 8.17 | 12.15 | 6.16 | 9.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.31 | 8.63 | 9.09 | 9.02 | 8.17 | 12.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.29 | 7.05 | 7.87 | 6.15 | 6.20 | 9.21 | 6.20 | 9.21 | 7.67 | 6.25 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 10.34 | 6.86 | 8.81 | 5.70 | 7.67 | 6.08 | 6.19 | 9.19 | 6.11 | 6.15 | 6.23 | 9.09 | 8.92 | 6.11 | 7.97 | 6.13 | 7.51 | 6.23 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 10.92 | 6.67 | 9.86 | 5.49 | 9.44 | 5.56 | 7.59 | 6.17 | 6.22 | 9.19 | 5.89 | 4.50 | 5.39 | 6.18 | 6.21 | 9.20 | 6.14 | 9.18 | 10.54 | 7.21 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.60 | 6.16 | 8.72 | 6.14 | 8.26 | 5.82 | 9.16 | 5.30 | 7.74 | 6.16 | 7.76 | 6.05 | 7.72 | 6.12 | 7.20 | 6.22 | 9.94 | 6.23 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 2-16: NS_60 S-SSB A-MPR simulation results for SL-U power class 5

Table 2-65 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-65 : NS_60 S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | 9.24 | 12.15 | 11.31 | 12.15 | 9.29 | 9.21 | 10.34 | 9.19 | 10.92 | 10.92 |
| Non-contiguous | - | - | - | - | 7.67 | 6.25 | 8.92 | 6.23 | 10.54 | 7.21 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 2-66 or Table 2-67 based on the simulation results when considering implementation margin.

Table 2-66: NS_60 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤10.0 | ≤9.0 | ≤11.5 | ≤9.0 | ≤13.0 | ≤10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 2-67: NS_60 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### Proposal 22: For SL-U NS_60 S-SSB A-MPR, consider Table 2-66 or Table 2-67 for SL-U UE power class 5.

### 2.6.5 NS_61 (A-MPR)

For NS_61, Table 2-68 and Figure 2-17 show the A-MPR simulation results for the scenarios.

Table 2-68: NS_61-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ | 9.24 | 9.14 | 13.08 | 7.15 | 10.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ | 11.18 | 9.58 | 10.10 | 9.02 | 9.23 | 13.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ | 9.84 | 7.13 | 8.68 | 7.13 | 7.13 | 10.21 | 7.12 | 10.20 | 8.88 | 7.69 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' | 10.20 | 8.27 | 9.35 | 6.64 | 8.49 | 7.23 | 7.20 | 10.22 | 5.69 | 7.14 | 7.21 | 10.21 | 9.98 | 7.65 | 9.34 | 7.52 | 9.37 | 7.37 |  |  |
| Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |
| '100MHz' | 11.00 | 8.15 | 10.53 | 7.12 | 9.82 | 6.71 | 8.24 | 7.20 | 7.14 | 10.13 | 5.89 | 5.43 | 5.38 | 7.07 | 7.21 | 10.23 | 7.22 | 10.11 | 10.29 | 8.04 |
| Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 |  |  |
| '100MHz' | 9.71 | 7.67 | 9.11 | 7.74 | 9.93 | 7.32 | 9.09 | 6.12 | 9.01 | 7.48 | 8.77 | 7.30 | 9.14 | 7.23 | 8.29 | 7.18 | 10.66 | 7.41 |  |  |

- 20MHz, 40MHz, and 60MHz

- 80MHz

- 100MHz

Figure 2-17: NS_61 S-SSB A-MPR simulation results for SL-U power class 5

Table 2-69 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration.

Table 2-69 : NS_61 S-SSB A-MPR simulation results for SL-U power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | 9.24 | 13.08 | 11.18 | 13.09 | 9.84 | 10.21 | 10.20 | 10.22 | 11.00 | 11.00 |
| Non-contiguous | - | - | - | - | 8.88 | 7.69 | 9.98 | 7.65 | 10.66 | 8.04 |

Considering the inner RB set bitmaps and the outer RB set bitmaps, the S-SSB A-MPR for SL-U power class 5 can be proposed as Table 2-70 or Table 2-71 based on the simulation results when considering implementation margin.

Table 2-70: NS_61 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤15.5 | ≤13.5 | ≤15.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤11.5 | ≤11.0 | ≤12.5 | ≤11.0 | ≤13.0 | ≤11.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 2-71: NS_61 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤15.5 | ≤13.5 | ≤15.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### Proposal 23: For SL-U NS_61 S-SSB MPR, Consider Table 2-70 or Table 2-71 for SL-U UE power class 5.

# Conclusions

Considering the simulation results for MPR and A-MPR for PSSCH/PSCCH, PSFCH and S-SSB and simulation assumption for PSFCH/S-SSB, the followings are proposed.

## SL-U PSSCH/PSCCH MPR

#### Proposal 1: For SL-U PSSCH/PSCCH MPR, consider the inner sub-band configuration and outer sub-band configuration in Table 2-5.

Table 2-5: Outer/Inner sub-band configuration for SL-U wideband operation

| Wideband operation channel bandwidth (MHz) | Contiguous sub-band configuration |  | Non-contiguous sub-band configuration2 |  |
| --- | --- | --- | --- | --- |
|  | Outer | Inner | Outer | Inner |
| 40 | 11, 10, 01 | N/A | N/A | N/A |
| 60 | 111, 110, 011, 100, 001 | 010 | 101 | N/A |
| 80 | 1111, 1110, 0111, 1100, 0011, 1000, 0001 | 0110, 0100, 0010 | 1101, 1011, 1010, 0101, 1001 | N/A |
| 100 | 11111, 11110, 01111, 11100, 00111, 11000, 00011, 10000, 00001 | 01110, 01100, 00110, 01000, 00010, 00100 | 11011, 11010, 01011, 11001, 10011, 10101, 10110, 01101, 10100, 00101, 10010, 01001, 11101, 10111, 10001 | 01010 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. NOTE 2: Both contiguous and non-contiguous sub-band configuration apply to MPR/A-MPR for PSFCH and S-SSB. |  |  |  |  |

#### Proposal 2: For SL-U PSSCH/PSCCH MPR, consider Table 2-4 for SL-U UE power class 5.

Table 2-4 Maximum power reduction (MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full2 (dB) | Partial3 (dB) | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full4 (dB) |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 | ≤ 1.5 | ≤ 1.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 3.0 | ≤ 3.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 2.5 apply. |  |  |  |  |  |  |

## SL-U PSSCH/PSCCH A-MPR

#### Proposal 3: For NS_31 SL-U PSSCH/PSCCH A-MPR, consider the inner sub-band configuration and outer sub-band configuration in Table 2-5.

#### Proposal 4: For NS_31 SL-U PSSCH/PSCCH A-MPR, consider Table 2-8 for SL-U UE power class 5.

Table 2-8. NS_31 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 4.5 | ≤ 6.5 | Follow SL-U MPR table |
|  | 16 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 2-5 apply. |  |  |  |  |  |  |

#### Proposal 5: For NS_53 SL-U PSSCH/PSCCH A-MPR, consider Table 2-11 for SL-U UE power class 5.

Table 2-11. NS_53 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

#### Proposal 6: For NS_58 SL-U PSSCH/PSCCH A-MPR, consider the inner sub-band configuration and outer sub-band configuration in Table 2-5.

#### Proposal 7: For NS_58 SL-U PSSCH/PSCCH A-MPR, consider Table 2-14 for SL-U UE power class 5.

Table 2-14. NS_58 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |
|  |  | Full (dB)2 | Partial (dB)3 | Full (dB) 2 | Partial (dB) 3 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 2-5 apply. |  |  |  |  |  |

#### Proposal 8: For NS_60 SL-U PSSCH/PSCCH A-MPR, consider Table 2-17 for SL-U UE power class 5.

Table 2-17. NS_60 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

#### Proposal 9: For NS_61 SL-U PSSCH/PSCCH A-MPR, consider Table 2-20 for SL-U UE power class 5.

Table 2-20. NS_61 additional maximum power reduction (A-MPR) for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

#### Proposal 9A: For NS_61 SL-U PSSCH/PSCCH A-MPR, consider  the channel bandwidth of 100MHz on top of NR-U NS_61 channel bandwidth of {20MHz, 40MHz, 60MHz, 80MHz}.

## SL-U PSFCH MPR

#### Proposal 10: For SL-U PSFCH MPR and A-MPR, consider simulation scenarios in Table 2-22.

Table 2.22: SL-U PSFCH MPR/A-MPR simulation scenarios

| Sub-band RB sets | Scenario | Bitmap | PSFCH RB location index | SCS(kHz) |
| --- | --- | --- | --- | --- |
| 1 (20MHz) | 1 | 1 | {0 10 20 30 40 50 60 70 80 90 100 104} | 15 |
|  | 2 | 1 | {0 5 10 15 20 25 30 35 40 45 49} | 30 |
| 2 (40MHz) | 3 | 11 | {0 10 20 30 40 50 60 70 80 90 100 104 111 121 131 141 151 161 171 181 191 201 211 215} | 15 |
|  | 4 | 10 | {0 10 20 30 40 50 60 70 80 90 100 104} | 15 |
| 3 (60MHz) | 5 | 111 | {0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 6 | 110 | {0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 7 | 100 | {0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 8 | 010 | {56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 9 | 101 | {0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161} | 30 |
| 4 (80MHz) | 10 | 1111 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 11 | 1110 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 12 | 1100 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 13 | 1000 | { 0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 14 | 0110 | { 56 61 66 71 76 81 86 91 96 101 105 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 15 | 0100 | { 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 16 | 1101 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 17 | 1010 | { 0 5 10 15 20 25 30 35 40 45 49 111 116 121 126 131 136 141 146 151 156 160} | 30 |
|  | 18 | 1001 | { 0 5 10 15 20 25 30 35 40 45 49 167 172 177 182 187 192 197 202 207 212 216} | 30 |
| 5 (100MHz) | 19 | 11111 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 20 | 11110 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 21 | 11100 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 22 | 11000 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 23 | 10000 | { 0 5 10 15 20 25 30 35 40 45 49} | 30 |
|  | 24 | 01110 | { 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 25 | 01100 | { 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 26 | 01000 | { 56 61 66 71 76 81 86 91 96 101 105} | 30 |
|  | 27 | 00100 | { 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 28 | 11011 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 29 | 11010 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 30 | 11001 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 31 | 10101 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 32 | 10110 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 33 | 10100 | { 0 5 10 15 20 25 30 35 40 45 49 112 117 122 127 132 137 142 147 152 157 161} | 30 |
|  | 34 | 10010 | { 0 5 10 15 20 25 30 35 40 45 49 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 35 | 10001 | { 0 5 10 15 20 25 30 35 40 45 49 223 228 233 238 243 248 253 258 263 268 272} | 30 |
|  | 36 | 01010 | { 56 61 66 71 76 81 86 91 96 101 105 167 172 177 182 187 192 197 202 207 212 216} | 30 |
|  | 37 | 11101 | { 0 5 10 15 20 25 30 35 40 45 49 56 61 66 71 76 81 86 91 96 101 105 112 117 122 127 132 137 142 147 152 157 161 223 228 233 238 243 248 253 258 263 268 272} | 30 |

#### Proposal 11: For SL-U PSFCH MPR, consider Table 2-25 or Table 2-26 for SL-U UE power class 5.

Table 2-25 PSFCH MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.5 |
| Non-contiguous sub-band RB sets | ≤ 5.0 | ≤ 4.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |

Table 2-26 PSFCH MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 5.0 | ≤ 4.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |

## SL-U PSFCH A-MPR

#### Proposal 12: For SL-U NS_31 PSFCH A-MPR, consider Table 2-29 or Table 2-30 for SL-U UE power class 5.

Table 2-29 NS_31 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 7.5 | ≤ 7.5 |
| Non-contiguous sub-band RB sets | ≤ 5.5 | ≤ 5.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. |  |  |

Table 2-30 NS_31 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. |  |  |

#### Proposal 13: For SL-U NS_53 PSFCH A-MPR, consider Table 2-33 or Table 2-34 for SL-U UE power class 5.

Table 2-33 NS_53 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤12.5 | ≤9.5 | ≤8.0 | ≤6.5 | ≤5.5 |
| Non-contiguous | N/A | ≤9.5 | ≤8.0 | ≤6.5 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 2-34 NS_53 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤12.5 | ≤9.5 | ≤8.0 | ≤6.5 | ≤5.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### Proposal 14: For SL-U NS_58 PSFCH A-MPR, consider Table 2-37 or Table 2-38 for SL-U UE power class 5.

Table 2-37 NS_58 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous sub-band RB sets | ≤ 4.5 | ≤ 2.5 |
| Non-contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |

Table 2-38 NS_58 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |

#### Proposal 15: For SL-U NS_60 PSFCH A-MPR, consider Table 2-41 or Table 2-42 for SL-U UE power class 5.

Table 2-41 NS_60 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤9.5 | ≤6.5 | ≤6.0 | ≤5.5 | ≤5.5 |
| Non-contiguous | N/A | ≤6.5 | ≤6.0 | ≤5.0 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 2-42 NS_60 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤9.5 | ≤6.5 | ≤6.0 | ≤5.5 | ≤5.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

#### Proposal 16: For SL-U NS_61 PSFCH A-MPR, consider Table 2-45 or Table 2-46 for SL-U UE power class 5.

Table 2-45 NS_61 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous | ≤10.5 | ≤7.5 | ≤6.5 | ≤6.5 | ≤6.0 |
| Non-contiguous | N/A | ≤7.5 | ≤6.5 | ≤6.5 | N/A |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

Table 2-46 NS_61 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤10.5 | ≤7.5 | ≤6.5 | ≤6.5 | ≤6.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

## SL-U S-SSB MPR

#### Proposal 17: For SL-U S-SSB MPR and A-MPR, consider simulation scenarios in Table 2-47.

Table 2-47: SL-U S-SSB MPR/A-MPR simulation scenarios

| Sub-band RB sets | Scenario | Bitmap | S-SSB {11RBs}xN repeated RB location index | N (Repeated#) | SCS(kHz) |
| --- | --- | --- | --- | --- | --- |
| 1 (20MHz) | 1 | 1 | {0 11 22 33 44 55 66 77 88} | 9 | 15 |
|  | 2 | 1 | {0 23 46 69 92} | 5 | 15 |
|  | 3 | 1 | {0 95} | 2 | 15 |
|  | 4 | 1 | {0 11 22 33} | 4 | 30 |
|  | 5 | 1 | {0 39} | 2 | 30 |
| 2 (40MHz) | 6 | 11 | {0 11 22 33 44 55 66 77 88 111 122 133 144 155 166 177 188 199} | 18 | 15 |
|  | 7 | 11 | {0 23 46 69 92 111 134 157 180 203} | 10 | 15 |
|  | 8 | 11 | {0 94 111 205} | 4 | 15 |
|  | 9 | 10 | {0 11 22 33 44 55 66 77 88} | 9 | 15 |
|  | 10 | 10 | {0 23 46 69 92 } | 5 | 15 |
|  | 11 | 10 | {0 94} | 2 | 15 |
| 3 (60MHz) | 12 | 111 | {0 11 22 33 56 67 78 89 112 123 134 145} | 12 | 30 |
|  | 13 | 111 | {0 38 56 94 112 150} | 6 | 30 |
|  | 14 | 110 | {0 11 22 33 56 67 78 89} | 8 | 30 |
|  | 15 | 110 | {0 38 56 94 } | 4 | 30 |
|  | 16 | 100 | {0 11 22 33} | 4 | 30 |
|  | 17 | 100 | {0 38} | 2 | 30 |
|  | 18 | 010 | {56 67 78 89 } | 4 | 30 |
|  | 19 | 010 | {56 94} | 2 | 30 |
|  | 20 | 101 | {0 11 22 33 112 123 134 145} | 8 | 30 |
|  | 21 | 101 | {0 38 112 150} | 4 | 30 |
| 4 (80MHz) | 22 | 1111 | {0 11 22 33 56 67 78 89 111 122 133 144 167 178 189 200} | 16 | 30 |
|  | 23 | 1111 | {0 38 56 94 111 149 167 205} | 8 | 30 |
|  | 24 | 1110 | {0 11 22 33 56 67 78 89 111 122 133 144} | 12 | 30 |
|  | 25 | 1110 | {0 38 56 94 111 149 } | 6 | 30 |
|  | 26 | 1100 | {0 11 22 33 56 67 78 89} | 8 | 30 |
|  | 27 | 1100 | {0 38 56 94} | 4 |  |
|  | 28 | 1000 | {0 11 22 33} | 4 | 30 |
|  | 29 | 1000 | {0 38} | 2 | 30 |
|  | 30 | 0110 | {56 67 78 89 111 122 133 144} | 8 | 30 |
|  | 31 | 0110 | {56 94 111 49} | 4 | 30 |
|  | 32 | 0100 | {56 67 78 89} | 4 | 30 |
|  | 33 | 0100 | {56 94} | 2 | 30 |
|  | 34 | 1101 | {0 11 22 33 56 67 78 89 167 178 189 200} | 12 | 30 |
|  | 35 | 1101 | {0 38 56 94 167 205} | 6 | 30 |
|  | 36 | 1010 | {0 11 22 33 111 122 133 144} | 8 | 30 |
|  | 37 | 1010 | {0 38 111 149} | 4 | 30 |
|  | 38 | 1001 | {0 11 22 33 167 178 189 200} | 8 | 30 |
|  | 39 | 1001 | {0 38 167 205} | 4 | 30 |
| 5 (100MHz) | 40 | 11111 | {0 11 22 33 56 67 78 89 112 123 134 145 167 178 189 200 223 234 245 256} | 20 | 30 |
|  | 41 | 11111 | {0 38 56 94 112 148 167 205 223 261} | 10 | 30 |
|  | 42 | 11110 | {0 11 22 33 56 67 78 89 112 123 134 145 167 178 189 200} | 16 | 30 |
|  | 43 | 11110 | {0 38 56 94 112 148 167 205} | 8 | 30 |
|  | 44 | 11100 | {0 11 22 33 56 67 78 89 112 123 134 145} | 12 | 30 |
|  | 45 | 11100 | {0 38 56 94 112 148} | 6 | 30 |
|  | 46 | 11000 | {0 11 22 33 56 67 78 89} | 8 | 30 |
|  | 47 | 11000 | {0 38 56 94} | 4 | 30 |
|  | 48 | 10000 | {0 11 22 33} | 4 | 30 |
|  | 49 | 10000 | {0 38} | 2 | 30 |
|  | 50 | 01110 | {56 67 78 89 112 123 134 145 167 178 189 200} | 12 | 30 |
|  | 51 | 01110 | {56 94 112 148 167 205} | 6 | 30 |
|  | 52 | 01100 | {56 67 78 89 112 123 134 145} | 8 | 30 |
|  | 53 | 01100 | {56 94 112 148} | 4 | 30 |
|  | 54 | 01000 | {56 67 78 89} | 4 | 30 |
|  | 55 | 01000 | {56 94} | 2 | 30 |
|  | 56 | 00100 | {112 123 134 145} | 4 | 30 |
|  | 57 | 00100 | {112 148} | 2 | 40 |
|  | 58 | 11011 | {0 11 22 33 56 67 78 89 167 178 189 200 223 234 245 256} | 16 | 30 |
|  | 59 | 11011 | {0 38 56 94 167 205 223 261} | 8 | 30 |
|  | 60 | 11010 | {0 11 22 33 56 67 78 89 167 178 189 200} | 12 | 30 |
|  | 61 | 11010 | {0 38 56 94 167 205} | 6 | 30 |
|  | 62 | 11001 | {0 11 22 33 56 67 78 89 223 234 245 256} | 12 | 30 |
|  | 63 | 11001 | {0 38 56 94 223 261} | 6 | 30 |
|  | 64 | 10101 | {0 11 22 33 112 123 134 145 223 234 245 256} | 12 | 30 |
|  | 65 | 10101 | {0 38 112 148 223 261} | 6 | 30 |
|  | 66 | 10110 | {0 11 22 33 112 123 134 145 167 178 189 200} | 12 | 30 |
|  | 67 | 10110 | {0 38 112 148 167 205} | 6 | 30 |
|  | 68 | 10100 | {0 11 22 33 112 123 134 145} | 8 | 30 |
|  | 69 | 10100 | {0 38 112 148} | 4 | 30 |
|  | 70 | 10010 | {0 11 22 33 167 178 189 200} | 8 | 30 |
|  | 71 | 10010 | {0 38 167 205} | 4 | 30 |
|  | 72 | 10001 | {0 11 22 33 223 234 245 256} | 8 | 30 |
|  | 73 | 10001 | {0 38 223 261} | 4 | 30 |
|  | 74 | 01010 | {56 67 78 89 167 178 189 200} | 8 | 30 |
|  | 75 | 01010 | {56 94 167 205} | 4 | 30 |
|  | 76 | 11101 | {0 11 22 33 56 67 78 89 112 123 134 145 223 234 245 256} | 16 | 30 |
|  | 77 | 11101 | {0 38 56 94 112 148 223 261} | 8 | 30 |

#### Proposal 18: For SL-U S-SSB MPR, consider Table 2-50 or Table 2-51 for SL-U UE power class 5.

Table 2-50 : S-SSB MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 8.5 | ≤ 6.5 |
| Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 9.5 | ≤ 7.0 |
| NOTE 1: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |  |  |

Table 2-51 :  S-SSB MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 9.5 | ≤ 7.0 |
| NOTE 1: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. |  |  |  |  |

## SL-U S-SSB A-MPR

#### Proposal 19: For SL-U NS_31 S-SSB A-MPR, consider Table 2-54 or Table 2-55 for SL-U UE power class 5.

Table 2-54: NS_31 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 | ≤ 10.0 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

Table 2-55: NS_31 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

#### Proposal 20: For SL-U NS_53 S-SSB MPR, consider Table 2-58 or Table 2-59 for SL-U UE power class 5.

Table 2-58: NS_53 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤17.5 | ≤13.5 | ≤17.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 | ≤12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 2-59: NS_53 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤17.5 | ≤13.5 | ≤17.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### Proposal 21: For SL-U NS_58 S-SSB A-MPR, consider Table 2-62 or Table 2-63 for SL-U UE power class 5.

Table 2-62: NS_58 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 8.5 | ≤ 6.5 |
| Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 | ≤ 9.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |  |  |

Table 2-63: NS_58 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 9.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |  |  |

#### Proposal 22: For SL-U NS_60 S-SSB A-MPR, consider Table 2-66 or Table 2-67 for SL-U UE power class 5.

Table 2-66: NS_60 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤10.0 | ≤9.0 | ≤11.5 | ≤9.0 | ≤13.0 | ≤10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 2-67: NS_60 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### Proposal 23: For SL-U NS_61 S-SSB MPR, Consider Table 2-70 or Table 2-71 for SL-U UE power class 5.

Table 2-70: NS_61 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation / (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous | ≤13.5 | ≤15.5 | ≤13.5 | ≤15.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| Non-contiguous | N/A | N/A | N/A | N/A | ≤11.5 | ≤11.0 | ≤12.5 | ≤11.0 | ≤13.0 | ≤11.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

Table 2-71: NS_61 S-SSB A-MPR for SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤15.5 | ≤13.5 | ≤15.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

# References

[1] R4-2317720, WF on NR_SL_enh2_UERF_part1, OPPO

[2] R4-2315542, On Tx requirements for NR sidelink evolution, LG Electronics

[3] R4-2314737, WF on NR_SL_enh2_UERF_part1, OPPO

[4] R4-2315963, TR38.786 v1.1.1.0 for SL evolution, OPPO

# Appendix

For MPR and A-MPR simulations for SL-U wideband operation following in-carrier SEM was used:

In the case of non-transmitted 20 MHz channel(s) on the edges of an assigned channel bandwidth the spectrum emission mask for operation with shared spectrum channel access, specified in Table 6.5F.2.2.0-1, is applied by using the total bandwidth of the remaining transmitted channels. The spectrum emission mask for non-transmitted channels is floored at -28dBr inside the assigned channel bandwidth and continues at same level until the channel bandwidth mask is intercepted, i.e. until ΔfOOB, as defined in 38.101-1 Clause 6.5F.2.2.0, equals BWChannel / 2. When ΔfOOB > BWChannel / 2 requirements in clause 6.5F.2.2.0 apply.

The In-Carrier SEM in gap between transmitted channels is defined in Table 5.1 and some examples are shown in Figure 5.1.

The relative power of any UE emission shall not exceed the most stringent levels given by the spectrum emission mask for operation with shared spectrum channel access with full channel bandwidth and the spectrum emission mask for non-transmitted channels with the channel bandwidth of the transmitted channels in the case of non-transmitted channels at the edge of an assigned channel bandwidth or spectrum emission mask defined for gaps between the transmitted channels.

An exception to the spectrum emission mask for non-transmitted channels allows a single [2] MHz bandwidth to extend to [-28] dBc relative to total transmit power, or [-20] dBm, whichever is the greatest.

Table 5-1: In-Carrier SEM in gap between transmitted channels for SL-U wideband operation

| Carrier BW (MHz) | TX bitmap | TX BW (MHz) | Gap (MHz) | ¦Frequency¦ in gap from transmitted channel and mask levels |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 10 | 20 | 30 | MHz |
| 60 | 101 | 40 | 20 | 0 | -20 | -23 |  |  | dBr |
| 80 | 1101 1011 | 60 | 20 | 0 | -20 | -23 |  |  | dBr |
|  | 1001 | 40 | 40 | 0 | -20 | -25 | -25 |  | dBr |
| 100 | 10100 01010 00101 | 40 | 20 | 0 | -20 | -23 |  |  | dBr |
|  | 10110 11010 01101 01011 | 60 | 20 | 0 | -20 | -23 |  |  | dBr |
|  | 10111 11011 11101 | 80 | 20 | 0 | -20 | -23 |  |  | dBr |
|  | 11001 10011 | 60 | 40 (adjacent from ’11’) | 0 | -20 |  | -25 |  | dBr |
|  |  | 60 | 40 (adjacent from’1’) | 0 | -20 | -25 |  |  | dBr |
|  | 10001 | 40 | 60 | 0 | -20 | -25 | -25 | -25 | dBr |

Figure 5-1: In-Carrier SEM in gap between transmitted channels for SL-U wideband operation