3GPP TSG-RAN WG4 Meeting # 109		R4-2319215

Chicago, US, November 13 – 17, 2023

Agenda item:		8.29.2.1

Source:		Samsung

Title:	Discussion on other RRM impacts on Rel-18 MIMO evolution: TDCP

Document for:		Discussion

- Introduction

In previous RAN4 meetings, we discussed the RRM impacts on each objective and had consensus on some objectives for RRM impacts. In last RAN4#108-bis meeting, RAN4 don’t have the final conclusion of feasibility study of TDCP amplitude measurements in last meeting.

- Discussion

TDCP: TRS-based TDCP (time-domain channel properties) reporting

In last RAN4#108-bis meeting, RAN4 discussed the feasibility to define TDCP amplitude measurements.

In the WF [1], it is listed the baselines and the simulation assumption:

Issue 2-1-1: Channel model for TDCP for feasibility study Agreement: Channel model for TDCP for feasibility study - Agree to use TDL as baseline - Other channel model (e.g., simplified/modified TDL, or CDL) can be considered if it finds not feasible with TDL channel. Issue 2-1-3: How to achieve ideal value of TDCP reporting? Agreement: How to achieve ideal value of TDCP reporting for the feasibility study - As starting point, for TDL channel, ideal value can be calculated for each delay value and over different doppler spread (fmax). - Other options (if any) can be considered. Issue 2-1-4: Is it feasible to define TDCP accuracy requirement for TDCP? Agreement: Other essential parameters for the feasibility study of defining TDCP accuracy requirement - Duration between TRS symbols - Doppler spread fmax for TDL - Doppler can be additionally considered if CDL is to be considered. - SNR - Number of averaging samples: one shot as baseline, other UE implementation not precluded for the feasibility study. - Channel bandwidth - Reference channel estimation algorithm - Further study on feasibility study on TDCP accuracy requirements based on existing evaluation results and additional evaluation results in RAN4#109 meeting. Make a conclusion of feasibility study in RAN4#109 meeting. - Draft CR on TDCP is expected to be submitted in the next meeting, and whether it can be agreeable depending on the conclusion of feasibility study. Understanding of other parameters: - Wideband measurement based on RAN1 definition Link level simulation assumptions for evaluating TRS based TDCP measurements in NR Parameter; Value. Delay (between TRS symbols); 1slot. Channel model; TDL-A, delay spread=30ns. Doppler Spread; 10, 30, 75, 100, 200, 300. SNR; 5:5:20. Number of averaging samples:; one shot as baseline, 4 samples. Channel BW; 10MHz. SCS; 30KHz as baseline, 15KHz 2nd priority. Reference Channel estimation; LS CE for TRS as baseline, MMSE CE as 2nd priority. Correlation matrix and antenna configuration; 1x2 Low. Simulation results to be considered: - CDF curves of (including the 90% and 50% and 10% CDF points) for estimated TDCP. TDCP calculation reference: ct,∆t=n=0N-1hnt+∆t∙hn*tnhnt+∆t2∙nhnt2

As mentioned in last meeting, TDCP amplitude is affected by some factors including:

- Doppler spread (UE velocity; Band (CF))

- Duration of two TRS

- SNR

- Channel model and delay spread

By using the simulation assumption (one shot + LS + 30kHzSCS), we achieve part of the simulation results;

| CDF points | 10% | 50% | 90% |
| --- | --- | --- | --- |
| SNR = 20dB; Doppler = 10 | 0.9632 | 0.9857 | 0.9907 |
| SNR = 20dB; Doppler = 300 | 0.6534 | 0.843 | 0.8926 |

According to RAN1’s definition for the 4 bits amplitude reporting:

Table 5.2.1.4.5-1: Mapping of elements of kTDCP: ki to TDCP amplitudes: 1-ai ki; ai. 0; 1256. 1; 11282. 2; 1128. 3; 1642.; ki; ai. 4; 164. 5; 1322. 6; 132. 7; 1162.; ki; ai. 8; 116. 9; 182. 10; 18. 11; 142.; ki; ai. 12; 14. 13; 122. 14; 12. 15; 12...

The 16 levels are:

| 0.9961 | 0.9945 | 0.9922 | 0.9890 |
| --- | --- | --- | --- |
| 0.9844 | 0.9780 | 0.9688 | 0.9558 |
| 0.9375 | 0.9116 | 0.875 | 0.8232 |
| 0.75 | 0.6464 | 0.5 | 0.2929 |

Assuming the mapping table is:

| Estimated TDCP value | Report index |
| --- | --- |
| 0.9945≤Estimated TDCP<0.9961 | 0 |
| 0.9922≤Estimated TDCP<0.9945 | 1 |
| 0.9890≤Estimated TDCP<0.9922 | 2 |
| 0.9844≤Estimated TDCP<0.9890 | 3 |
| 0.9780≤Estimated TDCP<0.9844 | 4 |
| 0.9688≤Estimated TDCP<0.9780 | 5 |
| 0.9558≤Estimated TDCP<0.9688 | 6 |
| 0.9375≤Estimated TDCP<0.9558 | 7 |
| 0.9116≤Estimated TDCP<0.9375 | 8 |
| 0.875≤Estimated TDCP<0.9116 | 9 |
| 0.8232≤Estimated TDCP<0.875 | 10 |
| 0.75≤Estimated TDCP<0.8232 | 11 |
| 0.6464≤Estimated TDCP<0.75 | 12 |
| 0.5≤Estimated TDCP<0.6464 | 13 |
| 0.2929≤Estimated TDCP<0.5 | 14 |
| Estimated TDCP≤0.2929 | 15 |

Observation 1: Estimated TDCP amplitude range is affected by the factors such as SNR, Doppler.

When estimated TDCP can be achieved, how to define TDCP accuracy. In legacy L1-RSRP, the accuracy is defined as 90% probability of estimated RSRP – ideal RSRP are within a certain range.

If not use the ideal TDCP value, which are controversial. For the CDF curve of estimated TDCP, which means in a certain condition, 80% probability estimated reporting can be in a range of [X Y].

Such as first condition, the reported index range is [2 6]. The reported index range is [9 12] for second row.

Proposal 1: It cannot define a common accuracy requirement for TDCP amplitude reporting to cover all conditions. The accuracy is only can be applicable under a certain condition. In RRM requirements, usually the requirements are applicable for all conditions when SNR > side condition. To decide whether can define an accuracy requirement in a certain condition in this meeting.

- Conclusion

In this contribution, we provide our further consideration of TDCP:

Observation 1: Estimated TDCP amplitude range is affected by the factors such as SNR, Doppler.

Proposal 1: It cannot define a common accuracy requirement for TDCP amplitude reporting to cover all conditions. The accuracy is only can be applicable under a certain condition. In RRM requirements, usually the requirements are applicable for all conditions when SNR > side condition. To decide whether can define an accuracy requirement in a certain condition in this meeting.

- Reference

[1] R4-2317371, WF on R18 NR MIMO RRM requirement, Samsung