3GPP TSG-RAN WG4 Meeting #109                                                       R4-2319224

Chicago, US, 13th – 17th Nov. 2023

Agenda Item:	8.26.8.2

Source:	Ericsson

Title:	Simulation results for NR NTN enhancement UE demodulation

Document for:	Information

# 1	Introduction

In the last RAN4#108bis meeting, companies kicked off the discussion on NR NTN UE demodulation requirements. In this contribution, some initial simulation results are delivered.

# 2	Discussion

## 2.1 Simulation assumptions

Table 2.1-1 Simulation parameters for NR NTN PDSCH on Ka band

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Center Frequency |  | GHz | 20 |
| Bandwidth |  | MHz | 100 |
| Subcarrier spacing |  | kHz | 120 |
| Duplex Mode |  |  | FDD |
| TDD Slot Configuration |  |  | N/A |
| Channel model |  |  | NTN-TDLC5-2000 |
| MCS/Rank |  |  | Rank 1 MCS 4/10/17 |
| Antenna configuration |  |  | 1T1R |
| Transform precoding |  |  | CP-OFDM |
| HARQ | Maximum number of HARQ transmissions |  | 4 and disabled |
|  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type |  | 1 |
|  | DM-RS duration |  | single-symbol DM-RS |
|  | Additional DM-RS position |  | pos 1 |
|  | Number of DM-RS CDM group(s) without data |  | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  | DM-RS port |  | {0} |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type |  | A |
|  | Start symbol |  | 2 |
|  | Allocation length |  | 12 |
| Frequency domain resource assignment | RB assignment |  | 100MHz for 120kHz SCS |
| Test metric |  |  | 70% of max throughput at target SNR. |

## 2.2 Simulation results

Table 2.2-1 Simulation results NR NTN PDSCH on Ka band

| Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | k-factor(dB) | Antenna configuration / Rank |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) |  | DM-RS Additional position | HARQ | MCS | SNR (dB) |
| 100MHz/120kHz | NTN-TDLC5- 2000Hz | 8,4 | 1x1, Rank 1 | 70 |  | Pos1 | disabled | 4 | -0.10 |  |
|  |  |  |  |  |  |  | enabled | 4 | -0.92 |  |
|  |  |  |  |  |  |  | disabled | 10 | 5.19 |  |
|  |  |  |  |  |  |  | enabled | 10 | 4.21 |  |
|  |  |  |  |  |  |  | disabled | 17 | 11.07 |  |
|  |  |  |  |  |  |  | enabled | 17 | 9.99 |  |
| 100MHz/120kHz | NTN-TDLC5- 2000Hz(doppler) | 8,4 | 2x2, XPOL, Rank 1 | 70 |  | Pos1 | disabled | 4 | -3.30 |  |
|  |  |  |  |  |  |  | enabled | 4 | -3.91 |  |
|  |  |  |  |  |  |  | disabled | 10 | 1.73 |  |
|  |  |  |  |  |  |  | enabled | 10 | 1.15 |  |
|  |  |  |  |  |  |  | disabled | 17 | 7.55 |  |
|  |  |  |  |  |  |  | enabled | 17 | 6.91 |  |
| 100MHz/120kHz | NTN-TDLC5- 2000Hz(doppler) | 8,4 | 2x2, XPOL low, Rank 2 | 70 |  | Pos1 | disabled | 4 | -0.43 |  |
|  |  |  |  |  |  |  | enabled | 4 | -0.91 |  |
|  |  |  |  |  |  |  | disabled | 10 | 4.91 |  |
|  |  |  |  |  |  |  | enabled | 10 | 4.47 |  |
|  |  |  |  |  |  |  | disabled | 17 | 11.24 |  |
|  |  |  |  |  |  |  | enabled | 17 | 10.72 |  |

Observation 1: The NTN-TDLC with 5ns and 2000Hz Doppler could be feasible for 120kHz SCS with DM-RS 1+1 configuration at least.

Observation 2: If correlation between polarization is very small, performance of 2x2 would be similar as 1x1.

Observation 3: Enabled HARQ outperforms disabled HARQ, but the degradation caused by disable HARQ is small.

# 3	Summary

In this contribution, we shared initial simulation results for NR NTN enhancement PDSCH demodulation.

We summarized our observations as follows:

Observation 1: The NTN-TDLC with 5ns and 2000Hz Doppler could be feasible for 120kHz SCS with DM-RS 1+1 configuration at least.

Observation 2: If correlation between polarization is very small, performance of 2x2 would be similar as 1x1.

Observation 3: Enabled HARQ outperforms disabled HARQ, but the degradation caused by disable HARQ is small.

# References

[1]	R4-2316921: WF on NR_NTN_enh_SAN_UE_demod, Huawei, HiSilicon, RAN4 #108bis, Xiamen, China