3GPP TSG-RAN WG4 Meeting #109                                                       R4-2319232

Chicago, US, 13th – 17th Nov. 2023

Agenda Item:	8.13.8.2

Source:	Ericsson

Title:	On PDSCH requirements for ATG network

Document for:	Discussion

# 1	Introduction

In this contribution, we discuss the possibility of introducing PDSCH requirements with 1024QAM.

# 2	Discussion

1024QAM has been already introduced for TN network, and there are corresponding requirements based on TDLD channel model. Comparing to the existing 1024QAM requirement of TN network, ATG network has similar or even better channel condition so that it gives the worth of consider the possibility. Therefore, we delivered link budget analysis based on the agreed ATG BS and UE assumptions.

The distance between ATG UE and BS could be 10km ~ 300km. The typical propagation channel is single tap channel, so free space propagation loss can be used. Other parameters for ATG BS and ATG UE are captured in table 2-1 which refers to TS38.876 6.2.2 [3]. Subarray antenna model is used for both FDD and TDD ATG BS in following calculation.

Table 2-1 System and antenna parameters for ATG BS and UE

| Centre frequency [GHz] |  | 2 | 4 |
| --- | --- | --- | --- |
| CBW [MHz] |  | 20 | 100 |
| PRB number |  | 106 | 273 |
| ATG BS system | Tx max power [dBm] | 43 | 50 |
|  | noise figure [dB] | 5 | 5 |
| ATG BS antenna | Element gain [dBi] | 6.4 |  |
|  | Array configuration (Row × Column) | 4x8 |  |
|  | Sub-array elements | 3 |  |
| ATG UE system | TRP or EIRP [dBm] | 40 | 43 |
|  | noise figure [dB] | 9 | 9 |
| ATG UE antenna | element gain [dBi] | 0 | 5 |
|  | Array configuration (Row × Column) | \ | (8x2) or (16x1) |

The SNR at receiver side for 2GHz and 4GHz are showed in figure 2-1.

Figure 2-1 Received SNR for FDD and TDD

As illustrated in the figure 2-1, 1024QAM is possible to be considered for distance < 30km for TDD and distance < 50km for FDD.

Observation 1: There is applicable scenarios for downlink 1024QAM according to the link budget analysis.

To assist the discussion, figure 2-2 shows our simulations with 1024QAM to see the operating SNR at different level of max Tput. MCS23 in the MCS table: 1024QAM was used.

Figure 2-2 PDSCH performance of 1024QAM

Based on the simulation results above, with 2x2 and 2x4 antenna configurations. The operating SNR for 2x2 configuration at 70% and 95% max Tput is around 23.5dB and 24.64dB respectively, which verified that it is possible to consider 1024QAM for PDSCH requirements for ATG network.

Observation 2: According to the simulation results, there is enough margin to consider 1024QAM with MCS23 for PDSCH demodulation requirement.

Proposal 1: Introduce 1024QAM requirements with MCS23 for ATG PDSCH demodulation, following test cases can be considered:

| Test number | Duplex mode, SCS and CBW | Propagation condition | Antenna configuration | MCS | SNR @70% maxTput |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz/10MHz | AWGN+220Hz Doppler | 2T2R | 1024QAM Table MCS23 |  |
| 2 |  |  | 2T4R | 1024QAM Table MCS23 |  |
| 3 | TDD 30kHz/40MHz 7DS2U | AWGN+500Hz Doppler | 2T2R | 1024QAM Table MCS23 |  |
| 4 |  |  | 2T4R | 1024QAM Table MCS23 |  |
| 5 | TDD 30kHz/40MHz 30D4S6U | AWGN+500Hz Doppler | 2T2R | 1024QAM Table MCS23 |  |
| 6 |  |  | 2T4R | 1024QAM Table MCS23 |  |

# 3	Summary

In this contribution, we shared our views and simulation results on the possibility of considering 1024QAM in ATG PDSCH requirement. Based on above, we observed and proposed the following:

Observation 1: There is applicable scenarios for downlink 1024QAM according to the link budget analysis.

Observation 2: According to the simulation results, there is enough margin to consider 1024QAM with MCS23 for PDSCH demodulation requirement.

Proposal 1: Introduce 1024QAM requirements for ATG PDSCH demodulation, following test cases can be considered:

| Test number | Duplex mode, SCS and CBW | Propagation condition | Antenna configuration | MCS | SNR @70% maxTput |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz/10MHz | AWGN+220Hz Doppler | 2T2R | 1024QAM Table MCS23 |  |
| 2 |  |  | 2T4R | 1024QAM Table MCS23 |  |
| 3 | TDD 30kHz/40MHz 7DS2U | AWGN+500Hz Doppler | 2T2R | 1024QAM Table MCS23 |  |
| 4 |  |  | 2T4R | 1024QAM Table MCS23 |  |
| 5 | TDD 30kHz/40MHz 30D4S6U | AWGN+500Hz Doppler | 2T2R | 1024QAM Table MCS23 |  |
| 6 |  |  | 2T4R | 1024QAM Table MCS23 |  |

# References

[1]	R4-2311795: Summary of simulation results for ATG UE and BS demodulation requirements, CMCC, RAN4 #108, Toulouse

[2]          R4-2316991: WF for ATG demodulation requirements, CMCC, RAN4 #108bis, Xiamen, China

[3]	TS38.876, v0.2.0