3GPP TSG-RAN4 Meeting #109	R4-2319237

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draft | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on SL-U MPR and A-MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SL-U MPR and A-MPR should be specified for PSSCH/PSCCH, PSFCH and S-SSB based on the endorsed draft big CR (R4-2317793). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce SL-U MPR for PSSCH/PSCCH, PSFCH, and S-SSB. Introduce SL-U A-MPR with NS_51/53/58/60/61 for PSSCH/PSCCH, PSFCH, and S-SSB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for SL-U MPR and A-MPR are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.2F, 6.2E.3F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

### 6.2E.2F	UE maximum output power reduction for Sidelink Unlicensed

#### 6.2E.2F.1	General

When UE is configured for NR sidelink transmissions in the unlicensed operating bands in FR1 defined in Table 5.2E.1F-1, this clause specifies the allowed Maximum Power Reduction (MPR) power for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For wideband operation, only sub-bands which are contiguously transmitted are considered in Table 6.2E.2F-3 for PSCCH/PSSCH.

For wideband operation, sub-bands which are contiguously transmitted and sub-bands which are non-contiguously transmitted in Table 6.2E.2F-3 are considered for PSFCH and S-SSB.

#### 6.2E.2F.2	MPR for NR SL-U UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power is specified in Table 6.2E.2F-1 for power class 5 NR sidelink UE.

Table 6.2E.2F-1 Maximum power reduction (MPR) for NR SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full2 (dB) | Partial3 (dB) | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full4 (dB) |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 | ≤ 1.5 | ≤ 1.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 3.0 | ≤ 3.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2E.2F-2. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2E.2F-2. NOTE 4: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2E.2F-2. NOTE 5: Contiguous sub-band configuration in Table 6.2E.2F-3 applies. |  |  |  |  |  |  |

Table 6.2E.2F-2 Exception MPR mapping for NR SL-U wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | None |
| 80 | 1100, 0011, 0100, 0010 |
| 100 | 00111, 11100, 00011, 11000 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. NOTE 2: Void. |  |

Table 6.2E.2F-3 Outer/Inner sub-band configuration for NR SL-U wideband operation

| Wideband operation channel bandwidth (MHz) | Contiguous sub-band configuration2 |  | Non-contiguous sub-band configuration2 |  |
| --- | --- | --- | --- | --- |
|  | Outer | Inner | Outer | Inner |
| 40 | 11, 10, 01 | N/A | N/A | N/A |
| 60 | 111, 110, 011, 100, 001 | 010 | 101 | N/A |
| 80 | 1111, 1110, 0111, 1100, 0011, 1000, 0001 | 0110, 0100, 0010 | 1101, 1011, 1010, 0101, 1001 | N/A |
| 100 | 11111, 11110, 01111, 11100, 00111, 11000, 00011, 10000, 00001 | 01110, 01100, 00110, 01000, 00010, 00100 | 11011, 11010, 01011, 11001, 10011, 10101, 10110, 01101, 10100, 00101, 10010, 01001, 11101, 10111, 10001 | 01010 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. NOTE 2: Only contiguous sub-band configuration applies to PSCCH/PSSCH. Both contiguous and non-contiguous sub-band configuration apply to PSFCH and S-SSB. |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed MPR for the maximum output power is specified in Table 6.2E.2F-4 for power class 5 NR sidelink UE.

Table 6.2E.2F-4 Maximum power reduction (MPR) for PSFCH transmission for NR SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 5.0 | ≤ 4.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 6.2E.2F-3 apply. |  |  |

For S-SSB transmission, the allowed MPR for the maximum output power is specified in Table 6.2E.2F-5 for power class 5 NR sidelink UE.

Table 6.2E.2F-5 Maximum power reduction (MPR) for S-SSB transmission for NR SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 9.5 | ≤ 9.5 | ≤ 7.0 |
| NOTE 1: Outer sub-band configuration and inner sub-band configuration in Table 6.2E.2F-3 apply. |  |  |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

### 6.2E.3F	UE additional maximum output power reduction for Sidelink Unlicensed

#### 6.2E.3F.1	General

Additional emission requirements can be signalled by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2E.1F-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2E.2F.

Table 6.2E.3F.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2E.3F.1-1A.

Table 6.2E.3F.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (clause) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | n46, n96 | 20, 40, 60, 80 |  | N/A |
| NS_31 | 6.5F.3.3.4 | n46 | 20, 40, 60, 80 |  | 6.2E.3F.2 |
| NS_53 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2E.3F.3 |
| NS_58 | 6.5F.3.3.6 | n102 | 20, 40, 60, 80 |  | 6.2E.3F.4 |
| NS_60 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2E.3F.5 |
| NS_61 | 6.5F.3.3.7 | n96 | 20, 40, 60, 80, 100 |  | 6.2E.3F.6 |
| NOTE 1: The A-MPR shall apply to all active 20 MHz sub-bands contiguously allocated in the channel. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2E.3F.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 |  |  |  | NS_31 |  |  | Reserved |
| n96 | NS_01 | NS_53 |  |  | NS_60 | NS_61 |  | Reserved |
| n102 | NS_01 | NS_58 |  |  |  |  |  | Reserved |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

#### 6.2E.3F.2	A-MPR for NS_31

When NS_31 is indicated by the network or pre-configured radio parameters for NR sidelink UE, this clause specifies the allowed Maximum Power Reduction (MPR) power for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed A-MPR is specified in Table 6.2F.3F.2-1 for power class 5 NR sidelink UE.

Table 6.2E.3F.2-1 A-MPR for NS_31 NR SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 4.5 | ≤ 6.5 | See Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 5: Contiguous sub-band configuration in Table 6.2E.2F-3 applies. |  |  |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed A-MPR is specified in Table 6.2E.3F.2-2 for power class 5 NR sidelink UE.

Table 6.2E.3F.2-2 A-MPR for NS_31 for PSFCH transmission for NR SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. |  |  |

For S-SSB transmission, the allowed A-MPR is specified in Table 6.2E.3F.2-3 for power class 5 NR sidelink UE.

Table 6.2E.3F.2-3 A-MPR for NS_31 for S-SSB transmission for NR SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

#### 6.2E.3F.3	A-MPR for NS_53

When NS_53 is indicated by the network or pre-configured radio parameters for NR sidelink UE, this clause specifies the allowed Maximum Power Reduction (MPR) power for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed A-MPR is specified in Table 6.2F.3F.3-1 for power class 5 NR sidelink UE.

Table 6.2E.3F.3-1 A-MPR for NS_53 NR SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed A-MPR is specified in Table 6.2E.3F.3-2 for power class 5 NR sidelink UE.

Table 6.2E.3F.3-2 A-MPR for NS_53 for PSFCH transmission for NR SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤12.5 | ≤9.5 | ≤8.0 | ≤6.5 | ≤5.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

For S-SSB transmission, the allowed A-MPR is specified in Table 6.2E.3F.3-3 for power class 5 NR sidelink UE.

Table 6.2E.3F.3-3 A-MPR for NS_53 for S-SSB transmission for NR SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤17.5 | ≤13.5 | ≤17.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### 6.2E.3F.4	A-MPR for NS_58

When NS_58 is indicated by the network or pre-configured radio parameters for NR sidelink UE, this clause specifies the allowed Maximum Power Reduction (MPR) power for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed A-MPR is specified in Table 6.2F.3F.4-1 for power class 5 NR sidelink UE.

Table 6.2E.3F.4-1 A-MPR for NS_58 NR SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |
|  |  | Full (dB)2 | Partial (dB)3 | Full (dB) 2 | Partial (dB) 3 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. NOTE 5: Contiguous sub-band configuration in Table 6.2E.2F-3 applies. |  |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed A-MPR is specified in Table 6.2E.3F.4-2 for power class 5 NR sidelink UE.

Table 6.2E.3F.4-2 A-MPR for NS_58 for PSFCH transmission for NR SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 4.5 | ≤ 3.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 2-5 apply. NOTE 3: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |

For S-SSB transmission, the allowed A-MPR is specified in Table 6.2E.3F.4-3 for power class 5 NR sidelink UE.

Table 6.2E.3F.4-3 A-MPR for NS_58 for S-SSB transmission for NR SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 9.5 | ≤ 7.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR applies. |  |  |  |  |

#### 6.2E.3F.5	A-MPR for NS_60

When NS_60 is indicated by the network or pre-configured radio parameters for NR sidelink UE, this clause specifies the allowed Maximum Power Reduction (MPR) power for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed A-MPR is specified in Table 6.2F.3F.5-1 for power class 5 NR sidelink UE.

Table 6.2E.3F.5-1 A-MPR for NS_60 NR SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed A-MPR is specified in Table 6.2E.3F.5-2 for power class 5 NR sidelink UE.

Table 6.2E.3F.5-2 A-MPR for NS_60 for PSFCH transmission for NR SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤9.5 | ≤6.5 | ≤6.0 | ≤5.5 | ≤5.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

For S-SSB transmission, the allowed A-MPR is specified in Table 6.2E.3F.5-3 for power class 5 NR sidelink UE.

Table 6.2E.3F.5-3 A-MPR for NS_60 for S-SSB transmission for NR SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤14.5 | ≤13.5 | ≤14.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

#### 6.2E.3F.6	A-MPR for NS_61

When NS_61 is indicated by the network or pre-configured radio parameters for NR sidelink UE, this clause specifies the allowed Maximum Power Reduction (MPR) power for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed A-MPR is specified in Table 6.2F.3F.6-1 for power class 5 NR sidelink UE.

Table 6.2E.3F.6-1 A-MPR for NS_61 NR SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed A-MPR is specified in Table 6.2E.3F.6-2 for power class 5 NR sidelink UE.

Table 6.2E.3F.6-2 A-MPR for NS_61 for PSFCH transmission for NR SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz |
| Contiguous/Non-contiguous | ≤10.5 | ≤7.5 | ≤6.5 | ≤6.5 | ≤6.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |

For S-SSB transmission, the allowed A-MPR is specified in Table 6.2E.3F.6-3 for power class 5 NR sidelink UE.

Table 6.2E.3F.6-3 A-MPR for NS_61 for S-SSB transmission for NR SL-U UE power class 5

| RB set configuration | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous | ≤13.5 | ≤15.5 | ≤13.5 | ≤15.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 | ≤13.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. |  |  |  |  |  |  |  |  |  |  |

< END OF CHANGE #2 >