3GPP TSG-RAN WG4 Meeting #109	R4-2319247

# Chicago, U.S.A., Nov. 13-17, 2023

Source: 	vivo

Title: 	TP to TR 38.869 on RRM aspects for LP-WUR

Agenda Item:			8.20.4

Document for:	Approval

Introduction

The outcome of RAN4 analysis and conclusions for the RRM part on Study on low-power wake-up signal and receiver for NR had been approved in [3]. This TP provides update to capture RAN4 conclusions on LP-WUR RRM.

Reference

[1] RP-232672, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo, RAN 101.

[2] RP‑231814, TR 38.869 v1.0.0 Study on low-power wake up signal and receiver for NR, RAN#101

[3] R4-2317426, WF on RRM aspects for NR LPWUS, vivo, RAN4 108-bis

Text Proposal to TR 38.869

--------------Start of text proposal 1-------------

- x.x RAN4 RRM Study

Editor’s note: RAN4 related conclusions

x.x.1	General

This clause captures the outcome of RAN4 study on RRM aspects for LP-WUR during SI phase. RAN4’s study on the outcome of “RAN1 studies on serving cell RSRP/RSRQ measurement offloading to LP-WUR for IDLE/INACTIVE mode” are:

RAN4 confirm that the evaluation methodology by RAN1 in TR38.869 for serving cell RSRP/RSRQ measurement offloading to LP-WUR at IDLE/INACTIVE mode is reasonable for SI phase.

- The evaluation methodology refers to the consideration of side condition, number of samples and accuracy.

    - In addition, in RAN4, RF impairment margin is considered.

x.x.2	Noise figure impact

RAN4 understands the determination of SNR target X of LP-WUR should consider at least the NF difference between LP-WUR and MR.

x.x.3	RF calibration margin impact

In RAN4 accuracy requirement, RF impairment margin is considered

x.x.4	SNR target X for serving cell measurement offloading

RAN4 understands the determination of SNR target X of LP-WUR should consider at least the applicable coverage conditions of LP-WUR.

x.x.5	Accuracy

The reference accuracy for serving cell RSRP/RSRQ measurement offloading to LP-WUR for IDLE/INACTIVE mode needs be investigated and further relaxation on the reference accuracy is possible.

--------------End of text proposal 1-------------

--------------Start of text proposal 2-------------

9. Conclusions

…

9.x RRM aspects for serving cell RSRP/RSRQ measurement offloading to LP-WUR for IDLE/INACTIVE mode

RAN4 confirm that the evaluation methodology by RAN1 in TR38.869 for serving cell RSRP/RSRQ measurement offloading to LP-WUR at IDLE/INACTIVE mode is reasonable.

--------------End of text proposal 2-------------