3GPP TSG-RAN WG4 Meeting #109		R4-2319252

# Chicago, US, November 13 – 17, 2023

Source:	LG Electronics

Title:	Discussion on LS of PSFCH power control

Agenda item:	8.30.2.1.1

Document for:	Discussion

# Introduction

In this paper, we provide our view on LS of PSFCH power control which was sent from RAN1.

# Discussion

The following LS was sent from RAN1 [1].

# Overall description Regarding PSFCH power control for “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”, RAN1#114bis made the following working assumption. Working assumption - In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”: - Assume the UE transmits N PSFCH - Denote the final Tx power on one common PRB is P_common - Denote the final Tx power on one dedicated PRB is P_dedicated - P_common <= P_dedicated - (pre-)configure an offset between P_common and P_dedicated - Send an LS to RAN4 asking whether there is any difficulty for supporting the following cases - P_common < P_dedicated - P_common = P_dedicated. # Actions: To RAN4: RAN1 respectfully requests RAN4 to take the above information into account, and provide feedback to RAN1 whether there is any difficulty for supporting the following cases - P_common < P_dedicated - P_common = P_dedicated

RAN4 has discussed SL-U MPR for PSCCH/PSSCH, PSFCH and S-SSB and agreed the following simulation assumption for PSFCH transmission.

  - Table 2-1 SL-U UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - Power per RB is same in PSFCH for all users - Total power is 20dBm - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB-set, RAN4 consider interlacing RBs for PSFCH For multiple RB-sets, RAN4 considers both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend of interlaced transmission should meet at least 80% of channel bandwidth in a single RB-set [, Ngap = RBend – RBstart ]. - RB allocation method as NR SL legacy RB allocation is also considered. |

In this simulation assumption, the different PSFCH transmissions which were agreed in RAN1, i.e., Alt 1-1b, Alt 2-3a, and NR SL legacy RB allocation are considered.

RAN4 should specify same PSFCH MPR requirement for the all PSFCH transmissions

#### Proposal 1: RAN4 applies same SL-U PSFCH MPR requirements for all PSFCH transmissions, i.e, Alt 1-1b, Alt 2-3a, and NR SL legacy RB allocation method.

For following Ran1 working assumption on Alt 1-1b, so far, RAN4 has considered ‘P_common = P_dedicated’ only for PSFCH MPR requirement.

- P_common < P_dedicated

- P_common = P_dedicated

For ‘P_common < P_dedicated’, the impact on the UE RF requirement has not been discussed in RAN4. It requires additional meeting cycles. Therefore, in order to complete this WI in Rel-18 time frame, RAN4 should specify SL-U PSFCH MPR requirements only considering equal power between PSFCH transmission. However, it does not mean that the design of SL-U PSFCH power control is limited to only equal power. The power control design for PSFCH transmission is up to RAN1.

#### Proposal 2: RAN4 specifies SL-U PSFCH MPR requirements only considering equal power between PSFCH transmission in Rel-18.

#### Proposal 3: The SL-U PSFCH MPR requirements do not limit the power control design for PSFCH transmission. It is up to RAN1.

Reply LS based on the proposal 1, proposal 2, and proposal 3 need to be sent to RAN1 for information.

#### Proposal 4: Send reply LS to RAN1 with followings.

- RAN4 applies same SL-U PSFCH MPR requirements for all PSFCH transmissions, i.e, Alt 1-1b, Alt 2-3a, and NR SL legacy RB allocation method.

- RAN4 specifies SL-U PSFCH MPR requirements only considering equal power between PSFCH transmissions in Rel-18.

- The SL-U PSFCH MPR requirements do not limit the power control design for PSFCH transmission. It is up to RAN1.

# Conclusion

In this contribution, we provided our view on LS of PSFCH power control [1] and proposals.

#### Proposal 1: RAN4 applies same SL-U PSFCH MPR requirements for all PSFCH transmissions, i.e, Alt 1-1b, Alt 2-3a, and NR SL legacy RB allocation method.

#### Proposal 2: RAN4 specifies SL-U PSFCH MPR requirements only considering equal power between PSFCH transmission in Rel-18.

#### Proposal 3: The SL-U PSFCH MPR requirements do not limit the power control design for PSFCH transmission. It is up to RAN1.

#### Proposal 4: Send reply LS to RAN1 with followings.

- RAN4 applies same SL-U PSFCH MPR requirements for all PSFCH transmissions, i.e, Alt 1-1b, Alt 2-3a, and NR SL legacy RB allocation method.

- RAN4 specifies SL-U PSFCH MPR requirements only considering equal power between PSFCH transmissions in Rel-18.

- The SL-U PSFCH MPR requirements do not limit the power control design for PSFCH transmission. It is up to RAN1.

# References

[1] R4-2318011, “LS on PSFCH power control”, Huawei