3GPP TSG-RAN4 Meeting #109	R4-2319259

# Chicago, US, 13th Nov 2023 - 17th Nov 2023

Source:	LG Electronics, KDDI

Title:	DC_18-n77 and CA_n18-n77 MSD analysis

Agenda item:	7.1.1.2

Document for:	Discussion

Introduction

In the last RAN4#108bis, the issue regarding DC_18-n77 and CA_n18-n77 was raised [1]. We would like to share our point of view for this issue and provide our analysis results.

Discussion

MSD values on DC_18A_n77A and CA_n18-n77 PC2/PC5

In the last meeting (RAN4#108bis), there are proposal that CA_n18-n77 and DC_18_n77 MSD value could copy from CA_n14-n77 and CA_n5-n77. Although it is one of the common practices in RAN4, we believe that analyzing the MSD values specifically for CA_n18-n77 and DC_18-n77 would provide more accurate results compared to reusing values from similar bands.

Table 1 DC_18-n77 and CA_n18-n77 MSD value about IMD 4 and IMD 5 [1].

Proposal for CA_n18-n77 and DC_18_n77 PC3 MSD test points: The relevant parts of the table below are used for the specification in 38.101-1 and 38.101-3 MSD due to IMDs of 2UL band tables. The IMD4 values are copied from similar band combination CA_n5-n77 and IMD5 values from CA_n14-n77. NR or E-UTRA Band / Channel bandwidth / NRB / MSD. NR-CA or ENDC Configuration; EUTRA or NR band; UL Fc (MHz); UL/DL BW (MHz); UL LCRB; DL Fc (MHz); MSD (dB); IMD order. DC_18A_n77A; 18; 827.5; 5; 25; 872.5; 8.3; IMD4 X. n77; 3355; 10; 50; 3355; N/A; N/A. 18; 817.5; 5; 25; 862.5; 5.5; IMD5 X. n77; 4130; 10; 50; 4130; N/A; N/A. CA_n18-n77; n18; 827.5; 5; 25; 872.5; 8.3; IMD4 X. n77; 3355; 10; 50; 3355; N/A; N/A. n18; 817.5; 5; 25; 862.5; 5.5; IMD5 X. n77; 4130; 10; 50; 4130; N/A; N/A. NOTE X: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range.. Proposal for CA_n18-n77 and DC_18_n77 PC2 MSD test points: The relevant parts of the table below are used for the specification in 38.101-1 and 38.101-3 MSD due to IMDs of 2UL band tables. The IMD4 and IMD5 values are copied from similar band combination CA_n5-n77. NR or E-UTRA Band / Channel bandwidth / NRB / MSD. NR-CA or ENDC Configuration; EUTRA or NR band; UL Fc (MHz); UL/DL BW (MHz); UL LCRB; DL Fc (MHz); MSD (dB); IMD order. DC_18A_n77A; 18; 827.5; 5; 25; 872.5; 18.4; IMD4 X. n77; 3355; 10; 50; 3355; N/A; N/A. 18; 817.5; 5; 25; 862.5; 11.7; IMD5 X. n77; 4130; 10; 50; 4130; N/A; N/A. CA_n18-n77; n18; 827.5; 5; 25; 872.5; 18.4; IMD4 X. n77; 3355; 10; 50; 3355; N/A; N/A. n18; 817.5; 5; 25; 862.5; 11.7; IMD5 X. n77; 4130; 10; 50; 4130; N/A; N/A. NOTE X: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range..

Observation: Analyzing the MSD values specifically for CA_n18-n77 and DC_18-n77 would have more accurate results compared to reusing values from similar bands.

MSD analysis for DC_18-n77 and CA_n18-n77 PC2/PC3

Fig 1. The architecture for DC_18-n77 and CA_n18-n77

Table 1 RF component assumption

| Antenna isolation, dB | 10 |
| --- | --- |
| Front end loss, dB (FDD) | 4 |
| Front end loss, dB (TDD) | 5 |
| duplexer up/down isolation | 50 |
| duplexer high frequency band isolation | 15 |
| Diplexer H/L band isolation | 15 |
| Filter isolation | 30 |
| PCB isolation | 60 |
| PA forward IP4 | 30 |
| PA reverse IP4 | 33 |
| PA forward IP5 | 30 |
| PA reverse IP5 | 33 |

Table 2 Main/Div total IMD 4 power and MSD value for PC3

| Main total IMD 4 | -81.5 dBm |
| --- | --- |
| Div total IMD 4 | -91.5 dBm |
| MSD after MRC at SNR = -1 (with 1.5 dB margin) | 8.6 dB |

Table 3 Main/Div total IMD 5 power and MSD value for PC3

| Main total IMD 5 | -89.0 dBm |
| --- | --- |
| Div total IMD 5 | -99.0 dBm |
| MSD after MRC at SNR = -1 (with 1.5 dB margin) | 1.1 dB |

Table 4 Main/Div total IMD 4 power and MSD value for PC2

| Main total IMD 4 | -74.0 dBm |
| --- | --- |
| Div total IMD 4 | -84.0 dBm |
| MSD after MRC at SNR = -1 (with 1.5 dB margin) | 16.1 dB |

Table 5 Main/Div total IMD 5 power and MSD value for PC2

| Main total IMD 5 | -80.0 dBm |
| --- | --- |
| Div total IMD 5 | -90.0 dBm |
| MSD after MRC at SNR = -1 (with 1.5 dB margin) | 10.1 dB |

Proposal : Use the analyzed MSD value for DC_18_n77A and CA_n18-n77 PC2/PC3

For PC3

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR-CA or ENDC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_18A_n77A | 18 | 827.5 | 5 | 25 | 872.5 | 8.6 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | 18 | 817.5 | 5 | 25 | 862.5 | 1.1 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 8.6 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 1.1 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| NOTE X: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range. |  |  |  |  |  |  |  |

For PC2

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR-CA or ENDC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_18A_n77A | 18 | 827.5 | 5 | 25 | 872.5 | 16.1 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | 18 | 817.5 | 5 | 25 | 862.5 | 10.1 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 16.1 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 10.1 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| NOTE X: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range. |  |  |  |  |  |  |  |

Conclusion

Observation: Analyzing the MSD values specifically for CA_n18-n77 and DC_18-n77 would have more accurate results compared to reusing values from similar bands.

Proposal : Use the analyzed MSD value for DC_18_n77A and CA_n18-n77 PC2/PC3

For PC3

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR-CA or ENDC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_18A_n77A | 18 | 827.5 | 5 | 25 | 872.5 | 8.6 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | 18 | 817.5 | 5 | 25 | 862.5 | 1.1 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 8.6 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 1.1 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| NOTE X: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range. |  |  |  |  |  |  |  |

For PC2

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR-CA or ENDC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_18A_n77A | 18 | 827.5 | 5 | 25 | 872.5 | 16.1 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | 18 | 817.5 | 5 | 25 | 862.5 | 10.1 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 16.1 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 10.1 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| NOTE X: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range. |  |  |  |  |  |  |  |

- References

[1]R4-2316397, On DC_18-n77 and CA_n18-n77 operator specific frequency range (Skyworks Solutions Inc.)