3GPP TSG-RAN WG4 Meeting # 109										R4-2319260

Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.26.2

Source: 	CATT, Ericsson, Huawei, Qualcomm, Samsung, Thales, ZTE

Title: 	Joint proposals on NTN co-existence study

Document for:	Approval

Introduction

This document captured joint proposals on NTN co-existence study in above 10GHz bands

Discussion

With ongoing calibration on NTN co-existence study, companies have the following joint proposals on NTN co-existence study.

Proposal 1: Companies are encouraged to consider following settings for Issues in 25 degree

- For UL: decrease RB numbers

- For DL

- Drop users below -10dB SNR with current Tx power settings OR

- Increase satellite Tx power.

- For Case 5, use Uma between TN BS and NTN UE rather than Free Space model

- For Fixed VSAT (22.5m) use Uma while using the VSAT height as UE height

- For L-ESIM (1.5m) use Uma.

Proposal 2: Change NTN UE antenna model parameter as below

1                for θ=0

4J1kasinθπ180kasinθπ1802        for 0<θ≤90°

For UE, 4J1kasinπ2kasinπ22          for θ>90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument ‘x’;

-	a is the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency. And the sin () function is in radian.

Proposal 3: RAN4 to approve the updates captured in Attachment 1 in track changes

Proposal 4: Further discuss the work to incorporate co-existence study into TR38.863

Conclusion

Companies are encouraged to further discuss NTN co-existence study with the above proposals.

- Attachment 1 Updated Simulation assumptions for NTN co-existence study in bands above 10GHz