3GPP TSG-RAN WG4 Meeting #109	R4-2319282

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on LTM candidate cell TCI state activation delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Requirements are missing for TCI state activation delay before LTM cell switch |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text proposals for TCI state activation delay and active TCI state list update delay before LTM cell switch command are given using the postponed draft CR from vivo in R4-2317325 as baseline. Main proposed changes compared to the postponed draft CR: - Extend the delay requirement to also cover UL TCI state and unknown TCI state - Addition of active TCI state list update delay - Tok=0 if TCI state is on LTM or serving cell active TCI state list |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification for LTM is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

## 8.X	TCI state activation delay for unified TCI before LTM cell switch command

### 8.X.1	Introduction

The requirements in this clause apply for a UE configured with DLorJoint-TCIState configurations on a candidate cell. UE shall complete the activation of downlink and uplink TCI state within the delay defined in this clause.

When the target DL/UL TCI state refers to a candidate cell and the activation of TCI is performed before cell switch through Candidate Cell TCI States Activation/Deactivation MAC CE, the requirements in this clause are applicable provided that following conditions are met:

-	The candidate cell is known to the UE.

-	At least one SSB is available for candidate cell L1-RSRP measurement every 160ms.

-	Editor’s Note: TBD other conditions

Editor’s Note: TBD definition of known candidate cell

### 8.X.2	Known TCI state conditions

The joint DL/UL TCI state or separate DL and UL TCI states in Candidate Cell TCI States Activation/Deactivation MAC CE are known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink/uplink TCI state to the completion of active downlink/uplink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target downlink/uplink TCI state or QCLed to the target downlink/uplink TCI state

-	Downlink/uplink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target downlink/uplink TCI state before the TCI state switch command

-	The target downlink/uplink TCI state remains detectable during the TCI state switching period

-	The SSB associated with the downlink/uplink TCI state remain detectable during the downlink TCI switching period

-	SNR of the downlink TCI state ≥ -3dB

Otherwise, the joint DL/UL TCI state or separate DL and UL TCI states are unknown.

### 8.X.3	 MAC-CE based TCI state activation delay before LTM cell switch

Editor’s note: The title of this section may be revised.

The requirements in this clause shall apply for DL/UL TCI state switch using separate DL and UL TCI states or joint TCI state of unified TCI state switch framework.

The requirements in this clause shall apply if the candidate cell satisfies the known cell condition defined in 8.X.1. If the known cell condition is not met, longer delay may be expected.

Upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to

- receive the cell switch command with the activated joint DL/UL or pair of separate DL and UL TCI states as the target DL/UL TCI state, and perform cell switch with Tfirst-RS = 0 and TRS-proc = 0, or

- receive the PDCCH order to target cell indicating RACH-based early TA acquisition with the SSB associated to the activated TCI state, and complete the PDCCH ordered RACH with Tfirst-SSB = 0

at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk* (Tfirst-SSB+ TSSB-proc)/ NR slot length, when the target DL/UL TCI state is known, or

at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TL1-RSRP TOuk* (Tfirst-SSB+ TSSB-proc)/ NR slot length, when the target DL/UL TCI state is unknown,

Where

THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tfirst-SSB is the first SSB after MAC CE command is decoded by the UE; The SSB shall be the associated to the activated DL TCI state.

TSSB-proc = 2 ms;

TOk = 1 if target TCI state is not in the LTM candidate cell active TCI state list or in the serving cell active TCI state list , 0 otherwise.

TOuk = 1 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD. TOuk = 1 when TCI state switching involves other QCL types only

T L1-RSRP = 0 in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

### 8.X.4	Active TCI state list update delay

The requirements in this clause shall apply for DL/UL TCI state switch using separate DL and UL TCI states or joint TCI state of unified TCI state switch framework, when a MAC CE activates more than one target joint TCI state or more than one pair of separate UL and DL TCI states.

The requirements in this clause shall apply if the candidate cell satisfies the known cell condition defined in 8.X.1. If the known cell condition is not met, longer delay may be expected.

If the target UL/DL TCI states are known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to

- receive the cell switch command with one of the activated joint DL/UL or pair of separate DL and UL TCI states as the target DL/UL TCI state, and perform cell switch with Tfirst-RS = 0 and TRS-proc = 0, or

- receive the PDCCH order indicating RACH-based early TA acquisition to target cell with the SSB associated to one of the activated TCI states in the MAC-CE, and complete the PDCCH ordered RACH with Tfirst-SSB = 0

at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk* (Tfirst-SSB-List+ TSSB-proc)/ NR slot length, where

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

-	Tfirst-SSB-List is the is the Tfirst-SSB for which the first SSB associated to the activated TCI state is the latest in time after decoding the MAC-CE; The SSB shall be associated to the activated TCI state.

-	TSSB-proc = 2 ms;

-	TOk = 1 if target TCI state is not in the LTM candidate cell active TCI state list or the serving cell active TCI state list, 0 otherwise.

<< End of change 1>>