3GPP TSG-RAN WG4 Meeting #109	R4-2319284

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on LTM cell switch delay requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 agreements on LTM cell switch delay are incomplete and have lots of FFS points open. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capturing the agreement reached in RAN4#108bis meeting: - When the target cell is a current serving cell (role switch) and the target TCI state in LTM cell switch command or SSB index indicated in PDCCH order is already on the active TCI state list for that serving cell or on the LTM candidate cell active TCI state list, consider the target TCI state activated. Capturing further proposals for the FFS points: - The UE supporting capability [earlyDecodingAndValidityCheck], shall have performed early ASN.1 decoding and validity check before the LTM cell switch command if the target TCI state in the LTM cell switch command is on the LTM candidate cell active TCI state list or the UE has received PDCCH order for the target cell from the network before the LTM cell switch command. - TLTM-processing is [TBD] when the target cell is a current serving cell - TLTM-interrupt = TLTM-processing + TLTM-IU ms - Known cell condition - Known TCI state condition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification for LTM is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 6.x and 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

## 6.X	L1/L2-Triggered Mobility

### 6.X.1	PCell Cell Switch

#### 6.X.1.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to another cell. The requirements in this section are applicable to LTM PCell switch.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighboring cell.

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

PCell switch to a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

The requirements in this clause are applicable to NR-DC for the following scenarios:

PCell switch to a neighboring cell

- FR1 cell to FR1 cell

PCell switch to a serving SCell in MCG

- FR1 cell to FR1 cell

#### 6.X.1.2	LTM Cell Switch delay

LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

When the target cell and the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, the LTM cell switch delay is defined as:

DLTM = Tcmd + TLTM-RRC-processing + Tfirst-RS + TRS-proc + TLTM-interrupt

Where:

Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement as specified in TS 38.213.

TLTM-RRC-processing is the time for early ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

TLTM-RRC-processing  = 0, if the the UE supporting capability [earlyDecodingAndValidityCheck], has performed early ASN.1 decoding and validity/compliance check of the complete LTM candidate cell configuration prior to the cell switch command for the LTM target cell indicated in the cell switch command. The UE supporting capability [earlyDecodingAndValidityCheck], shall have performed early ASN.1 decoding and validity check before the LTM cell switch command if the target TCI state in the LTM cell switch command is on the LTM candidate cell active TCI state list or the UE has received PDCCH order for the target cell from the network before the LTM cell switch command.

Otherwise TLTM-RRC-processing = 10 ms.

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

TRS-proc is the time for SSB processing.

Tfirst-RS = 0 and TRS-proc= 0 under the following conditions:

- The target TCI state indicated in the LTM cell switch command is in the LTM candidate cell active TCI state list or the serving cell active TCI state list, and

- L1-RSRP measurement period is not larger than 160 ms.

- Editor’s note: FFS other conditions.

Otherwise,

Tfirst-RS is the time to the first SSB transmission on the target cell [after Tcmd and TLTM-RRC-processing].

Editor’s note: FFS whether TRS transmission is also considered.

TRS-proc = 2 ms.

TLTM-interrupt is as stated in section 6.X.1.3.

The target cell in the LTM cell switch command is known if the following conditions are met:

  - The UE has sent a valid L1 [or L3] measurement report for the target cell and

  - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

Otherwise, the cell is unknown.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

- During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target DL/UL TCI state to the completion of LTM cell switch, where the RS resource for L1-RSRP measurement is the RS in target DL/UL TCI state or QCLed to the target DL/UL TCI state

  - LTM cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

  - The UE has sent at least 1 L1-RSRP report for the target DL/UL TCI state before the LTM cell switch command

  - The target DL/UL TCI state remains detectable during the LTM cell switching period

  - The SSB associated with the target DL/UL TCI state remain detectable during the cell switching period

    - SNR of the TCI state ≥ -3dB

- Otherwise, the target joint DL/UL TCI state or separate DL and UL TCI state is unknown.

#### 6.X.1.3 Interruption time

The interruption time TLTM-interrupt is the time between end of the last TTI containing the MAC-CE command for LTM cell switch until the time the UE transmits the first UL message on the target cell, excluding Tcmd[, Tfirst-RS and TRS-proc and TLTM-RRC-processing] stated in section 6.X.1.2.

TLTM-interrupt  = TLTM-processing + TLTM-IU ms,

Where:

TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

When the LTM target cell is a neighboring cell:

The value of TLTM-processing is 20 ms for FR1 to FR1 and FR2 to FR2 LTM cell switch.

The value of TLTM-processing is 40 ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

When the LTM target cell is a serving cell:

The value of TLTM-processing is [TBD] ms for FR1 to FR1 and FR2 to FR2 LTM cell switch.

The value of TLTM-processing is [TBD] ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

TLTM-IU is the interruption uncertainty during LTM cell switch.

For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

<< End of change 1>>

<< Start of change 2>>

## 8.X	L1/L2-Triggered Mobility

### 8.X.1	PSCell Cell Switch

#### 8.X.1.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to another cell. The requirements in this section are applicable to LTM PSCell switch.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements in this clause are applicable to NR-DC for the following scenarios:

PSCell switch to a neighboring cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

PSCell switch to a serving SCell in SCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

#### 8.X.1.2	LTM Cell Switch delay

LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

When the target cell and the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, the LTM cell switch delay is defined as:

DLTM = Tcmd + TLTM-RRC-processing + Tfirst-RS + TRS-proc + TLTM-interrupt

Where:

Tcmd, TLTM-RRC-processing, Tfirst-RS and TRS-proc are as stated in section 6.X.1.2.

TLTM-interrupt is as stated in section 8.X.1.3.

The definition of known LTM target cell and known target joint DL/UL TCI state or separate DL and UL TCI states are as stated in section 6.X.1.2.

#### 8.X.1.3 Interruption time

The interruption time TLTM-interrupt is the time between end of the last TTI containing the MAC-CE command for LTM cell switch until the time the UE transmits the first UL message on the target cell, excluding Tcmd[, Tfirst-RS and TRS-proc and TLTM-RRC-processing] stated in section 8.X.1.2.

TLTM-interrupt  = TLTM-processing + TLTM-IU ms,

Where:

TLTM-processing and TLTM-IU are as stated in section 6.X.1.3.

<< End of change 2>>