3GPP TSG-RAN WG4 Meeting # 109			                         	R4-2319292

Chicago, US, November 13 – 17, 2023

Agenda item:	5.2.3

Source: 	ZTE Corporation

Title: 	[NR_redcap-Core] Disscussion on R17 RedCap RRM maintenance

Document for:	Approval

Introduction

In this contribution, we discussed the remaining issue from the previous meeting [1] and expressed our views.

Discussion

About the definition of intra-frequency and inter-frequency handover. Due to the fact that the serving cell SSB and target cell SSB can be any combination of CD-SSB and NCD-SSB in RedCap UE, the definition of intra-frequency and inter-frequency handover in the specification is unclear. Therefore, we revised the definition of intra/inter-frenquency handover that the reference SSB of the serving cell is the SSB configured in the active DL BWP of the serving cell and the reference SSB of the target cell is the SSB configured in the DL firstActiveBWP of the target cell.

The revised definition is more clear, but when there is no SSB in the active DL BWP of the serving/target cell, whether and how to update the definition of frequency switching is a problem that needs to be considered.

Revision of the intra/inter-frequency definition in handover FFS whether to introduce requirements for scenarios when the active DL BWP of serving cell doesn’t contain an SSB

In our understanding, whether to consider the scenario when the active DL BWP of serving cell doesn’t contain an SSB is depend on UE capability. If bwp-withoutRestriction is supported, it’s ok for us to define the intra-frequency handover based on a reference SSB. And the reference SSB for both the serving cell and the target cell could be the corresponding CD-SSB.

Proposal 1: When the active DL BWP of serving cell cell doesn’t contain an SSB, then CD-SSB is used as the reference SSB for the serving cell to define the intra-frequency handover.

Conclusions

In this contribution, we discussed the remaining issue from the previous meeting and expressed our view. Based on the discussion, we have made following proposal:

Proposal 1: When the active DL BWP of serving cell cell doesn’t contain an SSB, then CD-SSB is used as the reference SSB for the serving cell to define the intra-frequency handover.

References

- R4-2314417, WF on R17 RedCap RRM maintenance, Ericsson