3GPP TSG-RAN WG4 Meeting #109 	R4-2319311

# Chicago, USA,  13st  - 17th November, 2023

Source:	Ericsson

Title:	Simulation results for Rel-18 NR PRACH repetitions demodulation requirements

Agenda item:	8.27.2

Document for:	Information

# Introduction

In the last RAN4#108bis meeting, companies kicked off the discussion on NR FR2 PRACH enhancement for BS demodulation requirements. In this contribution, some initial simulation results are delivered.

# Simulation result

## 2.1	Simulation assumptions

Table 2.1-1: Test parameters for PRACH repetition demodulation requirement

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLA30-300 |
| B4 | 60 | 0.13 us | 0.28 us |
|  | 120 | 0.07 us | 0.22 us |

## 2.2	Simulation results

Table 2.2-1: PRACH repetition missed detection requirements for Normal Mode, 120 kHz SCS in FR2-1

| Number | Number of | Propagation | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (Annex G) | offset | Repetition | SNR |
| 1 | 2 | AWGN | 0 | 0 | -19.5 |
|  |  | TDLA30-300 Low | 4000 Hz | 0 | -10.2 |
|  |  | AWGN | 0 | 2 | -22.2 |
|  |  | TDLA30-300 Low | 4000 Hz | 2 | -12.8 |

# Conclusions

Trial simulation results for Rel-18 NR PRACH enhancement demodulation requirements are delivered.