3GPP TSG-RAN WG4 Meeting #109 	R4-2319314

# Chicago, USA,  13st  - 17th November, 2023

Source:	Ericsson

Title:	Simulation results for NR NTN enhancement SAN demodulation requirements

Agenda item:	8.26.8.2

Document for:	Information

# Introduction

In the last RAN4#108bis meeting, companies kicked off the discussion on NR NTN SAN demodulation requirements. In this contribution, some initial simulation results are delivered.

# Simulation result

## 2.1	Simulation assumptions

Table 2.1-1 Simulation parameters for NR NTN PUSCH on Ka band

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Transform precoding |  |  | Disabled / enabled |
| Default TDD UL-DL pattern |  |  | 120 kHz SCS: FDD |
| HARQ |  | Maximum number of HARQ transmissions | 4 |
|  |  | RV sequence | 0, 2, 3, 1 |
| DM-RS |  | DM-RS configuration type | 1 |
|  |  | DM-RS duration | single-symbol DM-RS |
|  |  | Additional DM-RS position | pos1 |
|  |  | Number of DM-RS CDM group(s) without data | 2 |
|  |  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  |  | DM-RS port | {0} |
|  |  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain |  | PUSCH mapping type | B |
| resource |  | Start symbol | 0 |
| assignment |  | Allocation length | 10 |
| Frequency domain resource |  | RB assignment | Full applicable test bandwidth 120kHz SCS: 50MHz |
| assignment |  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  |  | Disabled |
| PTRS | Frequency density (KPT-RS) |  | 2, Disabled |
| configuration | Time density (LPT-RS) |  | 1, Disabled |

## 2.2	Simulation results

Table 2-2 Simulation results for 120kHz SCS, 50 MHz

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and corrélation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 | 70 % | 2 | pos1 | -3.2 |
|  |  | Normal | NTN-TDLC5-3000 | 70 % | 16 | pos1 | 10.5 |
|  |  | Normal | NTN-TDLC5-3000 | 70 % | 20 | pos1 | 14.2 |
| 2 | 2 | Normal | NTN-TDLC5-3000 | 70 % | 2 | pos1 | -3.4 |
|  |  | Normal | NTN-TDLC5-3000 | 70 % | 16 | pos1 | 9.7 |
|  |  | Normal | NTN-TDLC5-3000 | 70 % | 20 | pos1 | 13.1 |

Observation 1	The NTN-TDLC with 5ns and 3000Hz Doppler could be feasible for 120kHz SCS with DM-RS 1+1 configuration at least.

Observation 2	If correlation between polarization is very small, performance of 2x2 would be similar as 1x1.

# Conclusions

Trial simulation results for NR NTN enhancement PUSCH demodulation requirements are delivered.

Observation 1	The NTN-TDLC with 5ns and 3000Hz Doppler could be feasible for 120kHz SCS with DM-RS 1+1 configuration at least.

Observation 2	If correlation between polarization is very small, performance of 2x2 would be similar as 1x1.

# References

[1]	R4-2316921, WF for NR NTN enhancement demodulation, Huawei, HiSilicon