3GPP TSG-RAN WG4 Meeting #109	R4-2319316

# Chicago, USA, 13th – 17th November 2023

Source:	Ericsson

Title:	Simulation results for NR less than 5MHz demodulation requirements

Agenda item:	8.14.6.2

Document for:	Information

# Introduction

This contribution delivers some trial simulation results to compare performance difference between 5MHz and 3MHz on PUSCH and PUCCH.

# Simulation results

## 2.1 	PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | 3MHz: 12PRB |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) 5MHz 3MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | 2 | pos1 | -5.03 | -4.50 |
|  |  | Normal | TDLC300-100 Low | 70 % | 16 | pos1 | 7.89 | 7.71 |
|  |  | Normal | TDLA30-10 Low | 70 % | 20 | pos1 | 9.36 | 9.18 |
|  |  | Normal | TDLA30-10 Low | 70 % | 20 in Table 2 | pos1 | 15.95 | 15.93 |
|  |  | Normal | TDLC300-100 Low | 30 % | 16 | pos1 | 0.65 | 0.76 |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | 2 | pos1 | -1.67 | -1.44 |
|  |  | Normal | TDLC300-100 Low | 70 % | 16 | pos1 | 15.57 | 15.80 |

## 2.2	PUCCH

### Format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test Metric | DTX to ACK < 1% ACK miss <1% |

| Number of | Number of RX | Propagation conditions and | Number of | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
| TX antennas | antennas | correlation matrix (Annex G) | OFDM symbols | 5MHz | 3MHz |
| 1 | 2 | TDLC300-100 Low | 1 | 6.88 | 6.93 |
|  |  |  | 2 | 0.92 | 1.38 |

### Format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test Metric | DTX to ACK < 1% NACK to ACK <0.1% ACK miss <1% |

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Metric | Propagation conditions and correlation matrix (Annex G) | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5MHz | 3MHz |
| 1 | 2 | Normal | NACK to ACK <0.1% | TDLC-300-100 Low | -8.11 | -7.73 |
|  |  |  | ACK miss < 1% | TDLC-300-100 Low | -6.34 | -6.23 |

### Format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK <1% ACK miss <1% |

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Metric | Propagation conditions and correlation matrix (Annex G) | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5MHz | 3MHz |
| 1 | 2 | Normal | ACK miss < 1% | TDLC-300-100 Low | 2.98 | 3.05 |

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | UCI BLER <1% |

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Metric | Propagation conditions and correlation matrix (Annex G) | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5MHz | 3MHz |
| 1 | 2 | Normal | UCI BLER <1% | TDLC-300-100 Low | -1.92 | 0.41 |

### Format 3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Test metric | UCI BLER <1% |

| Test | Number of | Number of | Cyclic | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | TX antennas | RX antennas | Prefix |  |  |  |  |
|  |  |  |  |  |  | 5MHz | 3MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | -2.31 | -2.03 |
|  |  |  |  |  | Additional DM-RS | -3.02 | -2.68 |

### Format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | UCI BLER <1% |

| Number of | Number of | Cyclic | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | RX antennas | Prefix |  |  | SNR [dB] |  |
|  |  |  |  |  | 5MHz | 3MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | -0.49 | -0.18 |
|  |  |  |  | Additional DM-RS | -0.92 | -0.58 |

From among the evaluated PUCCH formats, only PUCCH format 2 with UCI BLER test case has an obvious performance difference.

# Conclusions

In the previous sections we made the following observations:

Observation 1	From among the evaluated PUCCH formats, only PUCCH format 2 with UCI BLER test case has an obvious performance difference.

Based on the discussion in the previous sections we propose the following:

No table of figures entries found.

# References

[1]	R4-2316924, Way Forward for NR less than 5MHz, Nokia, Nokia Shanghai Bell