3GPP TSG-RAN WG4 Meeting #109	R4-2319318

# Chicago, USA, 13th – 17th November 2023

Source:	Ericsson

Title:	Discussion on FR2 PUSCH 256QAM demodulation requirements

Agenda item:	8.6.4.1

Document for:	Discussion

# Introduction

In previous RAN4#108bis meeting, good progress was achieved and following agreements are captured in WF [1].

Issue 1-1: Tx EVM

Agreement: Impact for TxEVM shall be included in impairment result.

Issue 1-2: SNR Limit

Agreement: If impairment result for MCS 20 is over 20dB then check with TE vendors if this is testable. Request companies to include results with impairment for next meeting.

Issue 1-3a: Tx Phase Noise Model

Agreement: Do not consider Tx phase noise model or impact for requirement derivation.

Issue 1-3b: Rx Phase Noise Model

Agreement: Do not specify Rx phase noise model but consider Rx phase noise impact for impaired results.

Issue 1-4: Channel

Agreement: TDLD 30-35

Note: Review channel if impaired results create testability issues.

Issue 1-5: Antenna Configuration

Agreement: 1T2R

Issue 1-8: DMRS

Agreement: Both 1+1 and 1+0, cases selected for tests to be based on BS Manufacture Declaration and applicability rule

Issue 1-9: PTRS

Agreement: Define with both PTRS enabled and disabled, cases selected for tests to be based on BS Manufacturer Declaration and applicability rule to test requirements where a testable case is available.

For information below is a table of expected testable cases (with impairment, and testable being less than 20 dB SNR).

Table 1 : Expected Testable Cases with PTRS and DMRS for UL 256 BS Demod,

based on simulations for RAN4#108bis

|  |  | PTRS |  |
| --- | --- | --- | --- |
|  |  | Disabled | Enabled |
| DMRS | 1+0 | Likely not testable (based on simulations) | Likely testable (based on simulations) |
|  | 1+1 | Likely testable (based on simulations) | Likely testable (based on simulations) |

Issue 1-10: MCS

Agreement: MCS 20

In this contribution, open issues on CBW and applicability rules are furtherly analyzed.

# Discussion

Issue 1-6: 60 kHz SCS and corresponding carrier BW

Agreement: Companies to use 50 MHz for 60 kHz, however decision whether to include requirements for SCS of 60 kHz to be deferred to RAN4#109.

Issue 1-7: 120 kHz SCS and corresponding carrier BW

Agreement:

Companies to use 50 MHz initially for requirements derivation, with FFS on 100 MHz and 200 MHz.

Companies to provide simulations for 50, 100, 200 with a priority in order of 50, 200, 100, with a view on testable SNR.

Note: Could use existing applicability rule to extend 50 MHz to other CBW.

As for 60kHz SCS, it is true that no deployment in current FR2 real network, but it is no harm to define the requirements to avoid revisit in the future. It won’t add too much effort on specification modification and simulation if only one CBW is considered.

No much effort is needed to consider 60kHz SCS requirement.

Proposal 1	Introduce 60kHz SCS 50MHz CBW requirements for FR2-1 PUSCH 256QAM.

As for 120kHz SCS, it is agreed that minimum CBW 50MHz should be considered, but FFS on typical CBW 100MHz and the maximum CBW 200MHz. Based on the simulation results, no much performance difference is seen between CBW, then it will not lead difference to consider 100MHz requirement or 200MHz requirement from test coverage point of view. The key point here might be the testability. The larger CBW would acquire higher TE output power which would impact the link budget. Regarding the target SNR for 256QAM is close to 20dB limit and 200MHz would need 6dB more link budget margin compared to 50MHz, it might be more suitable to consider 100MHz CBW here which need 3dB more margin only.

200MHz CBW need 6dB higher link budget margin for OTA test setup than 50MHz CBW.

Proposal 2	Consider 120kHz SCS 100MHz CBW additional to 50MHz CBW requirements if necessary.

# Conclusions

In the previous sections we made the following observations:

Observation 1	No much effort is needed to consider 60kHz SCS requirement.

Observation 2	200MHz CBW need 6dB higher link budget margin for OTA test setup than 50MHz CBW.

Based on the discussion in the previous sections we propose the following:

_Toc149941200

Proposal 1	Introduce 60kHz SCS 50MHz CBW requirements for FR2-1 PUSCH 256QAM.

Proposal 2	Consider 120kHz SCS 100MHz CBW additional to 50MHz CBW requirements if necessary.

# References

[1]	R4-2316903, Way Forward 256 QAM BS Demod, Nokia, Nokia Shanghai Bell

[2] 	R4-2319319, Simulation results for FR2 PUSCH 256QAM, Ericsson