3GPP TSG-RAN WG4 Meeting #109 	R4-2319322

# Chicago, USA,  13st  - 17th November, 2023

Source:	Ericsson

Title:	Simulation results for ATG demodulation requirements

Agenda item:	8.13.8.3

Document for:	Information

# Introduction

In the last RAN4#108bis meeting, companies have agreed on all parameters for the ATG PUSCH demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered.

# Simulation result

## 2.1	Simulation assumptions

Table 2.1-1 Simulation parameters for normal PUSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 5 | 10 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
| Channel model |  |  | AWGN+200Hz | AWGN+500Hz |
| MCS/Rank |  |  | Rank 1 64QAM MCS28 in Table 1 256QAM MCS22 in Table 2 | Rank 1 64QAM MCS28 in Table 1 256QAM MCS22 in Table 2 |
| Antenna configuration |  |  | 1T2R | 1T2R |
| Transform precoding |  |  | CP-OFDM |  |
| HARQ | Maximum number of HARQ transmissions |  | 4 |  |
|  | RV sequence |  | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type |  | 1 |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |
|  | Additional DM-RS position |  | pos 1 |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |
|  | DM-RS port |  | {0} |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type |  | A |  |
|  | Start symbol |  | 0 |  |
|  | Allocation length |  | 14 |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |
|  | Frequency hopping |  | Disabled |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

## 2.2	Simulation results

Table 2.2-1 Simulation results with 70% of maximum throughput for 15kHz SCS, 5MHz CBW

| Tx | Rx | DM-RS | Channel model | MCS | Mapping type | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| 1 | 2 | 1+1 | AWGN +200Hz | 28 | A | 16.45 | 18.75 |
|  |  |  |  | 22 (256QAM) | A | 16.95 | 19.25 |

Table 2.2-2 Simulation results with 70% of maximum throughput for 30kHz SCS, 10MHz CBW

| Tx | Rx | DM-RS | Channel model | MCS | Mapping type | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| 1 | 2 | 1+1 | AWGN +500Hz | 28 | A | 16.15 | 18.45 |
|  |  |  |  | 22 (256QAM) | A | 17.2 | 19.5 |

# Conclusions

Simulation results for ATG PUSCH demodulation requirements are delivered.

# References

[1]	R4-2309796, WF for ATG demodulation, CMCC