3GPP TSG-RAN4 Meeting # 109	R4-2319333

Chicago, US, November 13 – 17, 2023

Source:	Samsung

Title:	discussion and simulation results on 8Rx PDSCH requirements

Agenda item:	8.3.3.1.2

Document for:	Discussion

- Introduction

In last RAN4 #108bis meeting, general test scope, simulation assumptions for FR1 8Rx CA PDSCH requirements have some agreements in WF [1]. In this contribution, we share our simulation results on below agreements of FR1 8Rx CA PDSCH requirements.

Issue 2-2-2: CBW for PDSCH CA requirements - Agreement - Define the performance requirements for the following channel bandwidths for each SCS: - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz Issue 2-2-3: CA duplex model and SCS combination - Agreement - Consider the following duplex model and SCS combination for 8Rx CA - FDD 15kHz + TDD 30kHz - FDD 15kHz + FDD 15kHz - TDD 30kHz + TDD 30kHz Issue 1-1-2: Number of RX antenna ports for CA demodulation requirements - Agreement - Consider Hybrid Rx for 8Rx CA perf requirements, such as 2Rx+8Rx, 4Rx+8Rx Issue 2-2-4: Rank - Agreement - Rank 8 for 8Rx+8Rx, Rank 2 for 2Rx+8Rx and 4Rx+8Rx - Rank 8 + Rank 8 for 8Rx+8Rx for UE supporting Rank 8 for 8Rx - Rank 2 + Rank 2 for 4Rx+8Rx - Rank 2 + Rank 2 for 2Rx+8Rx Issue 2-2-5: HARQ-ACK configuration per PUCCH group - Agreement - Spatial bundling of PUCCH HARQ ACKs is disabled - Companies are encouraged to double check this simulation assumptions in the simulation configuration, if needed, updated simulation results are welcome to be submitted for next meeting. SDR CA requirements - Agreement - Reuse the existing methodology of Re-15 SDR CA test Issue 4-1-1: CA test for CQI reporting - Agreement - Not consider CQI reporting for CA.

- Test assumptions and simulation results

  - Test assumptions

According the simulation assumptions in the WF [1] as below. Other common parameters, refer to TS 38.101-4: Table 5.2-1: Common test parameters for PDSCH.

Table 1: Simulation assumptions for PDSCH performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD, FDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | TDD: 30/40 FDD: 15/10 |
| MIMO layer and Antenna configuration |  |  | Rank 2: 2x8 Rank 8: 8x8 |
| Propagation condition and antenna correlation |  |  | Rank 2: TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) Rank 8: TDLA30-10 Low |
| MCS |  |  | Rank2: MCS19 Rank 8: MCS17 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12 TDD: specific to each Reference channel |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Rank2: Single-symbol Rank 8: Double-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| NZP-CSI-RS configuration |  |  | Rank 8: row 6 |
| Overhead |  |  | 0 |
| N1 and N2 configurations for 8Tx cases |  |  | Use (N1,N2) = (4,1), (O1, O2) = (4,1) |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 8 layers cases |
| Number of HARQ Processes |  |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |
| Test metric |  |  | SNR@70% max TP |

  - Simulation results

Below tables list our simulation results with impairments.

Table 2: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-X.Y1 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.63 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.63 |
| 15 | R.PDSCH.1-X.Y2 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.75 |
| 20 | R.PDSCH.1-X.Y3 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.86 |
| 25 | R.PDSCH.1-X.Y4 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.07 |
| 30 | R.PDSCH.1-X.Y5 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.09 |
| 35 | R.PDSCH.1-X.Y6 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.11 |
| 40 | R.PDSCH.1-X.Y7 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.39 |
| 45 | R.PDSCH.1-X.Y8 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.56 |
| 50 | R.PDSCH.1-X.Y9 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.56 |

Table 3: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-X.Y1 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.31 |
| 10 | R.PDSCH.2-X.Y2 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.31 |
| 15 | R.PDSCH.2-X.Y3 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.44 |
| 20 | R.PDSCH.2-X.Y4 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.45 |
| 25 | R.PDSCH.2-X.Y5 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.51 |
| 30 | R.PDSCH.2-X.Y6 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.62 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.67 |
| 50 | R.PDSCH.2-X.Y7 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.82 |
| 60 | R.PDSCH.2-X.Y8 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.93 |
| 80 | R.PDSCH.2-X.Y9 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.10 |
| 90 | R.PDSCH.2-X.Y10 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.21 |
| 100 | R.PDSCH.2-X.Y11 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.24 |

Table 4: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-Z.Y1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.21 |
| 10 | R.PDSCH.1-3.7 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.24 |
| 15 | R.PDSCH.1-Z.Y2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.37 |
| 20 | R.PDSCH.1-Z.Y3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.42 |
| 25 | R.PDSCH.1-Z.Y4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.59 |
| 30 | R.PDSCH.1-Z.Y5 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.61 |
| 35 | R.PDSCH.1-Z.Y6 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.78 |
| 40 | R.PDSCH.1-Z.Y7 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.92 |
| 45 | R.PDSCH.1-Z.Y8 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.98 |
| 50 | R.PDSCH.1-Z.Y9 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.06 |

Table 5: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-Z.Y1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.69 |
| 10 | R.PDSCH.2-Z.Y2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.69 |
| 15 | R.PDSCH.2-Z.Y3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.72 |
| 20 | R.PDSCH.2-Z.Y4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.77 |
| 25 | R.PDSCH.2-Z.Y5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.85 |
| 30 | R.PDSCH.2-Z.Y6 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.83 |
| 40 | R.PDSCH.2-3.7 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.85 |
| 50 | R.PDSCH.2-Z.Y7 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.13 |
| 60 | R.PDSCH.2-Z.Y8 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.36 |
| 80 | R.PDSCH.2-Z.Y9 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.57 |
| 90 | R.PDSCH.2-Z.Y10 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.61 |
| 100 | R.PDSCH.2-Z.Y11 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.62 |

Observation 1: The SNR differences between the minimum and maximum bandwidths are no more than 1dB for both rank 2 and rank 8 scenarios.

- Conclusion

In this contribution, we provide our simulation results for rank 2 and rank 8 cases for 8Rx CA scenarios, both FDD and TDD. And from the simulation results, we have below observation:

Observation 1: The SNR differences between the minimum and maximum bandwidths are no more than 1dB for both rank 2 and rank 8 scenarios.

- Reference

[1] R4-2316914, “WF for 8Rx and 4Tx performance requirements”, Huawei, HiSilicon, 3GPP RAN4 #108bis meeting