3GPP TSG-RAN WG4 Meeting # 109	R4-2319367

Chicago, US, November 13 – 17, 2023

Agenda item:	8.33.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for R18 mobile IAB

Document for:	Discussion

# Introduction

For core requirements for Rel-18 Mobile IAB, most issues are solved. The performance part is scheduled to start at this meeting. In this meeting, we provide our analysis on performance impacts for mobile IAB.

# Discussion

According to the progress achieved in core requirements, following RRM requirements are defied for Rel-18 Mobile IAB.

- Intra-frequency HO

- Intra-frequency RRC re-establishment

- Active TCI state switching

- RLM

- Link recovery procedure

- L1-RSRP

- Intra-frequency measurement requirements

Thus, for Rel-18 mobile, it is proposed to define test cases to verify above requirements.

One thing which is different from normal UE is the test configuration used in the test. For IAB-MT, the supporting of UE capabilities and configurations are different from normal UE. The test configurations may not be supported by all IAB-MT, which is declared by manufacturer. This issue was also discussed in Rel-16/17 and the conclusion is that the configuration parameters are left to implementation and manufacturer declaration, and the corresponding test requirements shall be based on the actual configuration parameters used in the test.

Observation 1: For IAB-MT, most of the configuration parameters used in the test cases are declared by manufacturer, which means we cannot have a unified test configuration for all IAB-MT.

Proposal 1: RAN4 to define test cases for Rel-18 Mobile IAB to verify following requirements. To keep the same principle as Rel-16/17 that the configuration parameters are left to implementation and manufacturer declaration, and the corresponding test requirements shall be based on the actual configuration parameters used in the test.

- Intra-frequency HO

- Intra-frequency RRC re-establishment

- Active TCI state switching

- RLM

- Link recovery procedure

- L1-RSRP

- Intra-frequency measurement requirements

For measurement accuracy requirements, it is straightforward that legacy accuracy requirements for UE can be reused for mobile IAB-MT. In details, the related accuracy requirements are as follows:

Table I. Related accuracy requirements for legacy UE.

10.1.2 Intra-frequency RSRP accuracy requirements for FR1

10.1.3 Intra-frequency RSRP accuracy requirements for FR2

10.1.7 Intra-frequency RSRQ accuracy requirements for FR1

10.1.10 Inter-frequency RSRQ accuracy requirements for FR2

10.1.19 L1-RSRP accuracy requirements for FR1

10.1.20 L1-RSRP accuracy requirements for FR2

Proposal 2: Accuracy requirements in table I in TS 38.133 can be reused for Rel-18 Mobile IAB.

# Conclusions

Proposal 1: RAN4 to define test cases for Rel-18 Mobile IAB to verify following requirements, and keep the same principle as Rel-16/17 that the configuration parameters are left to implementation and manufacturer declaration, and the corresponding test requirements shall be based on the actual configuration parameters used in the test.

- Intra-frequency HO

- Intra-frequency RRC re-establishment

- Active TCI state switching

- RLM

- Link recovery procedure

- L1-RSRP

- Intra-frequency measurement requirements

Proposal 2: Accuracy requirements in table I in TS 38.133 can be reused for Rel-18 Mobile IAB.

# References