3GPP TSG-RAN WG4 Meeting # 109	R4-2319375

Chicago, US, November 13 – 17, 2023

Title: 	Discussion on performance requirements for mobility enhancements

Source: 	Huawei, HiSilicon

Agenda item:	8.24.3

Document for:	Discussion

Introduction

This contribution provides analysis on performance requirements for mobility enhancement.

Discussion

LTM

- Accuracy

The R17 existing accuracy for L1-RSRP measurements are defined for the intra-frequency L1-RSRP measurement. Herein for FR1 the relative L1-RSRP measurement accuracy is less than absolute measurement accuracy as the UE implementation margin can be eliminated when two cells on the same frequency are compared. If inter-frequency L1-RSRP measurement is supported, the inter-frequency measurement accuracy has to be analyzed. In theory for FR1 inter-frequency measurement the relative accuracy is supposed be the same as absolute accuracy because the UE RF margin is different on different frequency.

In R18 LTM, the L1-RSRP based event trigger is not supported. Network indicates L1-RSRP resource in resource set, and then UE performs corresponding L1-RSRP measurement. Herein it is possible that both candidate LTM intra-frequency cells and inter-frequency candidate LTM cells are included in the same resource set. For the beam selection for SSB based L1-RSRP measurement report, it is up to UE to report M*L beams.

Agreement - For the beam selection for SSB based L1-RSRP measurement report, - For the value of M, L - the RRC configured candidate values are: - M = 1, 2, 3, 4 - L = 1, 2, 3, 4 - Note: the maximum value of M*L and combination of M and L is up to UE capability - Note: the common understanding is that L=1 with configuration of inclusion of serving cell is not a typical case. - No need to confirm the corresponding working assumption (made in RAN1#113).

The LTM L1-RSRP reporting scheme reuses the differential scheme which is similar as legacy L1-RSRP reporting. So the absolute and relative accuracy for intra-frequency and inter-frequency are needed

Agreement - Absolute value and differential values are used for L1-RSRP reporting: - For absolute L1-RSRP, the L1-RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size - For differential L1-RSRP, the L1-RSRP value is quantized to a 4-bit value where the differential L1-RSRP value is computed with 2 dB step size from reference L1-RSRP value

Proposal 1: Absolute and relative accuracy for intra-frequency and inter-frequency LTM L1-RSRP measurement are to be specified.

The core requirements of LTM are under discussion. As per current progress, the following test cases can be specified:

- Test cases for cell switching delay

- Test cases for intra-frequency L1-RSRP measurement with UE capability within CP

- Test cases for inter-frequency L1-RSRP measurement

Other tests are not precluded based on further conclusion of core requirements.

Proposal 2: For LTM the following test cases can be specified:

- Test case for cell switching delay

- Test case for intra-frequency L1-RSRP measurement with UE capability within CP

- Test case for inter-frequency L1-RSRP measurement

Other tests are not precluded based on further conclusion of core requirements.

NR-DC with selective activation of cell groups via L3 enhancements

The core requirements are settled down. The test case for Subsequent conditional PSCell change is to be specified.

Proposal 3: The test case for Subsequent conditional PSCell change is to be specified.

Improvement on SCell/SCG setup delay

As so far there is no concrete solution for this objective, the test cases related discussion would wait for more progress.

Proposal 4: The performance part of improvement on SCell/SCG setup delay wait for more progress on core part.

Enhanced CHO configurations

The core requirements for CHO including target MCG and target SCG (obj#3) and CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj#4). The following test cases can be defined:

-Conditional handover including target MCG and target SCG in NR-DC

- FR1-FR1 NR-DC to FR1-FR1 NR-DC,

- FR1-FR2 NR-DC to FR1-FR1 NR-DC,

- FR1-FR1 NR-DC to FR1-FR2 NR-DC,

- FR1-FR2 NR-DC to FR1-FR2 NR-DC.

- Conditional Handover including target MCG and candidate SCG in NR-DC

- FR1-FR1 NR-DC to FR1-FR1 NR-DC,

- FR1-FR2 NR-DC to FR1-FR1 NR-DC,

- FR1-FR1 NR-DC to FR1-FR2 NR-DC,

- FR1-FR2 NR-DC to FR1-FR2 NR-DC.

Although some FR2 tests have testability issues, they can be specified in test case with a clarification that UE does not have to pass the test case.

Proposal 5: The following test cases can be defined:

Conditional handover including target MCG and target SCG in NR-DC

- FR1-FR1 NR-DC to FR1-FR1 NR-DC,

- FR1-FR2 NR-DC to FR1-FR1 NR-DC,

- FR1-FR1 NR-DC to FR1-FR2 NR-DC,

- FR1-FR2 NR-DC to FR1-FR2 NR-DC.

- Conditional Handover including target MCG and candidate SCG in NR-DC

- FR1-FR1 NR-DC to FR1-FR1 NR-DC,

- FR1-FR2 NR-DC to FR1-FR1 NR-DC,

- FR1-FR1 NR-DC to FR1-FR2 NR-DC,

- FR1-FR2 NR-DC to FR1-FR2 NR-DC.

Conclusions

This contribution provides analysis on performance requirements for mobility enhancement. The following proposals are provided:

Proposal 1: Absolute and relative accuracy for intra-frequency and inter-frequency LTM L1-RSRP measurement are to be specified.

Proposal 2: For LTM the following test cases can be specified:

- Test case for cell switching delay

- Test case for intra-frequency L1-RSRP measurement with UE capability within CP

- Test case for inter-frequency L1-RSRP measurement

Other tests are not precluded based on further conclusion of core requirements.

Proposal 3: The test case for Subsequent conditional PSCell change is to be specified.

Proposal 4: The performance part of improvement on SCell/SCG setup delay wait for more progress on core part.

Proposal 5: The following test cases can be defined:

Conditional handover including target MCG and target SCG in NR-DC

- FR1-FR1 NR-DC to FR1-FR1 NR-DC,

- FR1-FR2 NR-DC to FR1-FR1 NR-DC,

- FR1-FR1 NR-DC to FR1-FR2 NR-DC,

- FR1-FR2 NR-DC to FR1-FR2 NR-DC.

- Conditional Handover including target MCG and candidate SCG in NR-DC

- FR1-FR1 NR-DC to FR1-FR1 NR-DC,

- FR1-FR2 NR-DC to FR1-FR1 NR-DC,

- FR1-FR1 NR-DC to FR1-FR2 NR-DC,

- FR1-FR2 NR-DC to FR1-FR2 NR-DC.

Reference

[1] R4-2317329, WF on RRM performance requirements of R18 Further NR mobility enhancement, Apple