3GPP TSG-RAN4 Meeting #109	R4-2319389

Chicago, US, November 13 – 17, 2023

Source:	China Telecom

Title:	Discussion on PDSCH CA requirements for UE with 8Rx: Simulation results

Agenda Item:	8.3.3.1.2

Document for:	Discussion

- Introduction

This contribution provides the simulation results about 8Rx demodulation performance for PDSCH CA, and the assumption is based on the approved WF in [1].

- Simulation Results

| Duplex CBW/SCS | Antenna configuration | Rank, MCS (Table) | Channel model | % of TP | Channel bandwidth for single CC (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| TDD 40 MHz/30 kHz | 2T8R ULA Medium B | Rank 2, MCS19 (Table 1) | TDLC300-100 | 70 | 9.89 | 9.86 | 10.11 | 10.30 | 10.13 | 10.14 | 10.13 | 10.51 | 10.41 | 10.36 |
|  | 8T8R Low | Rank 8, MCS17 (Table 1) | TDLA30-10 |  |  |  |  |  |  |  |  |  |  |  |

Table 1-1.  Ideal simulation results for 8Rx CA PDSCH under FDD 15kHz SCS

Table 1-2.  Impairment simulation results for 8Rx CA PDSCH under FDD 15kHz SCS

| Duplex CBW/SCS | Antenna configuration | Rank, MCS (Table) | Channel model | % of TP | Channel bandwidth for single CC (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| TDD 40 MHz/30 kHz | 2T8R ULA Medium B | Rank 2, MCS19 (Table 1) | TDLC300-100 | 70 | 11.89 | 11.86 | 12.11 | 12.30 | 12.13 | 12.14 | 12.13 | 12.51 | 12.41 | 12.36 |
|  | 8T8R Low | Rank 8, MCS17 (Table 1) | TDLA30-10 |  |  |  |  |  |  |  |  |  |  |  |

Table 2-1.  Ideal simulation results for 8Rx CA PDSCH under TDD 30kHz SCS

| Duplex CBW/SCS | Antenna configuration | Rank, MCS (Table) | Channel model | % of TP | Channel bandwidth for single CC (MHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| TDD 40 MHz/30 kHz | 2T8R ULA Medium B | Rank 2, MCS19 (Table 1) | TDLC300-100 | 70 | 9.97 | 10.52 | 10.70 | 10.65 | 10.71 | 10.61 | 10.92 | 10.73 | 11.05 | 11.03 | 10.97 | 11.25 |
|  | 8T8R Low | Rank 8, MCS17 (Table 1) | TDLA30-10 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2-2.  Impairment simulation results for 8Rx CA PDSCH under TDD 30kHz SCS

| Duplex CBW/SCS | Antenna configuration | Rank, MCS (Table) | Channel model | % of TP | Channel bandwidth for single CC (MHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| TDD 40 MHz/30 kHz | 2T8R ULA Medium B | Rank 2, MCS19 (Table 1) | TDLC300-100 | 70 | 11.97 | 12.52 | 12.70 | 12.65 | 12.71 | 12.61 | 12.92 | 12.73 | 13.05 | 13.03 | 12.97 | 13.25 |
|  | 8T8R Low | Rank 8, MCS17 (Table 1) | TDLA30-10 |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Reference

[1] R4-2316914, WF for 8Rx UE performance requirements, Huawei, HiSilicon, RAN4 #108-bis, Oct, 2023