TSG-RAN Working Group 4 (Radio) meeting #109	R4-2319395

Chicago, US, 13th to 17th November 2023

Source:	Ericsson

Title:	TP to TR 38.858: Additions and corrections relevant for adjacent channel co-existence evaluation results in clause 11

Agenda item:	8.19.2.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#108-bis in Xiamen) all RAN4 input to TR 38.585 was collected in a draft text proposal to TR 38.858 [1]. Technical background information relevant for feasibility, coexistence evaluation and regulatory aspects was agreed and included.

In this contribution we provide some additional information and corrections with the intent to improve and finalize the adjacent channel coexistence evaluation, part of the SBFD SI (FS_NR_duplex_evo). At the end of this contribution a text proposal for TR 38.858, clause 11 is attached for approval.

# Discussion

At last meeting all simulation results were collected in Excel format for all considered Scenarios and Cases in a separate contribution [2]. The size of the information is extensive and needs to be condensed to capture general trends for a given Scenario/Case combination. Based on an analysis of the simulation results conclusion was made. An initial version of conclusion was captured in TR 38.858, clause 11.

When clause 11 was reviewed we discovered some errors and well as potential improvements of the text to increase the readability. Also, we discovered some misalignments between the Excel data and the conclusions when the results were cross checked. To improve the technical information in clause 11 a text proposal with updates was created.

Summary of changes:

- In subclause 11.1, the editor’s note is removed, references to tables corrected and minor editorial corrections.

- In subclause 11.3, the editor’s note is replaced with introduction to section.

- In subclause 11.3.1, table heading is added.

- In subclause 11.3.2, table heading is added.

- In subclause 11.3.3, table headings are added.

- In subclause 11.3.4, table headings are added.

- In subclause 11.3.1 to subclause 11.3.4, reference to considered scenario is added.

- In subclause 11.3.2, align Scenario 4/Case 2 UL conclusion with other Scenario conclusions, as described in section 2.2.

- In subclause 11.3.1 to subclause 11.3.4, harmonization of terminology (BS, antenna configuration, grid-shift, etc.)

- In subclause 11.3.3, align Scenario 1/Case 3 with simulation results in Excel-sheet, as described in section 2.1.

  - Scenario 1/Case 3

Using data from the Excel-sheet [2] we can conclude that Scenario 1/Case 3 suffer from throughput degradation both for average and cell-edge throughput. The current text does not reflect collected simulation results, as shown Table 2.1-1.

# Table 2.1-1: Simulation results for Scenario 1/Case 3 UL

| Case | Victim | Aggressor | Company | Antenna cofiguration | grid shift (%) | BS Tx power (dBm) | Enhanced NF | TP degradation @5% in % | TP degradation @Mean in % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | SBFD (UL) | NR TDD DL | Ericsson | 2 | 100 | 49 | no | 60.48 | 15.74 |
| 3 | SBFD (UL) | NR TDD DL | Nokia | 1 | 100 | 50 | no | NaN | 17.92 |
| 3 | SBFD (UL) | NR TDD DL | CableLabs | 1 | 100 | 50 | no | NaN | 18.78 |
| 3 | SBFD (UL) | NR TDD DL | CableLabs | 1 | 100 | 50 | yes | 100 | 18.20 |
| 3 | SBFD (UL) | NR TDD DL | CMCC | 2 | 100 | 49 | no | 21.66 | 3.74 |
| 3 | SBFD (UL) | NR TDD DL | Samsung SBFD Ant 2 | 2 | 100 | 49 | no | 38.77 | 6.77 |
| 3 | SBFD (UL) | NR TDD DL | Huawei | 2 | 100 | 49 | no | N/A | N/A |
| 3 | SBFD (UL) | NR TDD DL | CableLabs | 2 | 100 | 49 | no | 29.05 | 6.29 |
| 3 | SBFD (UL) | NR TDD DL | ZTE | 2 | 100 | 49 | no | 76.99 | 13.00 |
| 3 | SBFD (UL) | NR TDD DL | Qualcomm | 2 | 100 | 49 | no | 41.95 | 7.7 |
| 3 | SBFD (UL) | NR TDD DL | Samsung SBFD Ant 2 with improved NF | 2 | 100 | 49 | yes | 38.03 | 6.56 |
| 3 | SBFD (UL) | NR TDD DL | CableLabs | 2 | 100 | 49 | yes | 28.57 | 6.33 |
| 3 | SBFD (UL) | NR TDD DL | Nokia | 2 | 100 | 53 | no | NaN | 16.79 |
| 3 | SBFD (UL) | NR TDD DL | CableLabs | 2 | 100 | 53 | no | 100 | 9.97 |
| 3 | SBFD (UL) | NR TDD DL | CableLabs | 2 | 100 | 53 | yes | 100 | 10.05 |
| 3 | SBFD (UL) | NR TDD DL | CATT | 2 | 100 | 49 | no | 44.43 | 8.37 |

Proposal 1: For Scenario 1/Case 3, update the conclusion for UL to capture that throughput degradation is observed at cell edge and at average.

  - Scenario 4/Case 2

For the case where a scenario based on different types of cell grids (like UMa and UMi), the meaning of grid-shift in general terms needs careful consideration.

# Figure 2.2-1: Overview of cell grids for Scenario 4 for grid-shift (a) 10%; (b) 25%; (c) 100%

Sweeping the grid-shift for a scenario with equal cell size would show that BS-to-BS interference increases when the grid-shift reduces. However, for Scenario 4 as shown in Figure 2.2-1, since UMa and UMi scenarios have different antenna heights, inter-site distance and the coverage area of UMi scenario is smaller than that of UMa scenario, the grid-shift increase/decrease does not linearly correspond to a decrease/increase of the distance between aggressor and victim base stations for all the base stations, as in turn happens in scenarios where the two operators have the same coverage area/inter-site distance. For example, while grid-shift increases, the central victim (red) base station gets further from the aggressor central base station (blue), but other red base stations get closer to other blue base stations, which with smaller grid-shift did not have other closer base stations, and so were receiving less inter-BS ACI. The sentence in current conclusion which mentions an “exception” could be interpretated as if the interference may reduce by reducing the grid-shift. This would lead to the wrong conclusions, while in reality when the distance between base stations is reduced the BS-to-BS interference will be stronger, which will increase the throughput loss.

For the given scenario the conclusion should not have an “exception” for the Scenario 4/Case 2. The general trend is similar as for other UL results in Case 2.

Proposal 2: Change the conclusion for Case 2 to capture: The throughput degradation is due to the inter-BS ACI introduced by SBFD, which increases as grid shift (BS-to-BS distance) decreases in scenarios 1, 2 and 5. In scenario 4 (UMa-to-UMi scenario), the degradation increases and then decreases due to the relative distance and elevation angle between UMa and UMi base stations. This is a result of the grid shift reduction and consequent changes in antenna discrimination.

# Conclusion

A text proposal has been created to improve the conclusions from the adjacent channel coexistence evaluation part of the SBFD study. The majority of updates relates to editorial corrections to improve readability. Also, two updates of conclusion have been proposed to reflect the collection of simulation results.

Based on analysis of simulation results the following updates to conclusion are presented for approval:

Proposal 1: For Scenario 1/Case 3, update the conclusion for UL to capture that throughput degradation is observed at cell edge and at average.

Proposal 2: Change the conclusion for Case 2 to capture: The throughput degradation is due to the inter-BS ACI introduced by SBFD, which increases as grid shift (BS-to-BS distance) decreases in scenarios 1, 2 and 5. In scenario 4 (UMa-to-UMi scenario), the degradation increases and then decreases due to the relative distance and elevation angle between UMa and UMi base stations. This is a result of the grid shift reduction and consequent changes in antenna discrimination.

At the end of this contribution a text proposal to TR 38.858, clause 11 is attached for approval.

# References

[1]	R4-2317010, “Draft TR 38.858”, CMCC

[2] 	R4-2316989, “Simulation results for SBFD coexistence”, CMCC

TEXT PROPOSAL:

# 11	Adjacent channel co-existence evaluation results.

## 11.1	Introduction

The adjacent channel co-existence studies were performed to the deployment scenarios described in Table 11.1-1 below. The co-existence cases are described in the Table 11.1-2 below, and they were performed for each scenario listed in Table 11.1-1. The detailed assumptions associated with these scenarios and cases can be found in Annex E.

Table 11.1-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | SBFD Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR2-1 | Urban Macro | Urban Macro |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 8 | FR2-1 | Urban Dense | Urban Dense |
| 9 | FR2-1 | Indoor | Indoor |
| Note 1: This scenario has been down-selected. |  |  |  |

Table 11.1-2: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  |
| 2 | SBFD | TDD UL |  |
| 3 | TDD DL | SBFD |  |
| 4 | TDD UL | SBFD |  |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |

The Urban Hotspot reuses most parameter assumption as Urban Macro, except that Urban Macro adopts random dropping method for UE while Urban Hotspot adopts cluster-based dropping method for UE. Other differences are described in Annex E in Table E.2.1-1, Table E.2.1-2, and Table E.2.1-3.

The coexistence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in an adjacent channel as well as to understand the impact of legacy TDD network on SBFD network, as described in Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable to the {D, U, D} SBFD configuration in terms of performance (based on the agreed RAN4 models and parameters related to leakage).

Additionally, it should be noted that the RAN4 co-existence studies have special assumptions due to the adopted simulation methodologies such as:

- Power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target SINR value according to the power control scheme in TS 38.213.

- It is assumed that all the slots configurations are the same with the time-invariant ACLR modelling assumption. Compared with the average throughput over all time slots with different configuration, this is the worst case with largest degradation value.

Moreover, in the following sections, all the throughput degradation data were given in a range, where these data are defined as follows:

- The {positive number} means the co-existence evaluation shows throughput loss;

- The {negative number} means the co-existence evaluation shows throughput gain;

- The {n/a} means the co-existence evaluation finds the performance basis have no throughput, thus throughput degradation percentage cannot be mathematically calculated from such basis.

## 11.2	Summary of all simulation results

Editor's note: This section will capture the excel files and word file that moderator summarized based on all companies’ input with some description of these files.

[

One example:

We list three files.

First excel file is the collection of all simulation results using the template format that companies use to provide simulation results.

Word file is the summary of all simulation results categorized in terms of scenario, case, victim and parameters values of four simulation assumptions, i.e. antenna configuration 1 or 2, gNB Tx power value, grid shift value and enhanced NF or not. In addition to, we list the min value, max value and median for relative ACIR, ACIR enhanced by 2, 4, 6, and 8dB. We also list the numbers of companies that show simulation results for each scenario, each case and each victim band.

Second excel file is the excel file of word file which is much straightforward compared with the word.

[placeholder for clarifying text on ACIR enhancement definition for simulation and results interpretation]

]

## 11.3	Conclusion

This section captures conclusions per case based on adjacent channel co-existence simulation results.

### 11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL

Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.1-1.

Table 11.3.1-1: Case 1

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different BS Tx powers (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid shifts (5% to 100%), and different SBFD antenna configurations. |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid-shifts (5% to 100%) and BS Tx powers (49 dBm to 53 dBm). |
| Urban Macro -> Urban Micro Scenario 4 | FR1 | No DL throughput degradation for both average throughput and cell edge throughput is observed. |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 | FR1 and FR2-1 |  |

### 11.3.2 Case 2: aggressor SBFD DU victim NR TDD UL

Case 2 considers legacy TDD in UL slot as a victim while SBFD is operating in the UL slot in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.2-1.

Table 11.3.2-1: Case 2

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 | FR1 | The TDD UL has significant throughput degradation for different SBFD BS antenna configuration and different BS Tx powers. - The cell edge throughput degradation is worse than the average throughput degradation. - The throughput degradation is due to the inter-BS ACI introduced by SBFD, which increases as grid shift (BS-to-BS distance) decreases in scenarios 1, 2 and 5. In scenario 4 (UMa-to-UMi scenario), the degradation increases and then decreases due to the relative distance and elevation angle between UMa and UMi base stations. This is a result of the grid-shift reduction and consequent changes in antenna discrimination. - The throughput degradation increases with SBFD BS Tx power. - SBFD BS antenna configuration slightly impacts the throughput degradation. |
| Urban Hotspot -> Urban Hotspot Scenario 2 |  |  |
| Urban Macro ->Urban Micro Scenario 4 |  |  |
| Urban Micro -> Urban Micro Scenario 5 |  |  |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 | No TDD UL throughput degradation is observed. |
| Urban Macro -> Urban Macro Scenario 6 | FR2-1 | TDD UL throughput degradation is observed at cell edge, no strong degradation is observed for the average throughput. |
| Urban Dense -> Urban Dense Scenario 8 |  |  |

### 11.3.3 Case 3: aggressor NR TDD DL victim SBFD DU

Case 3 considers SBFD as a victim while NR TDD is operating DL in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in for SBFD DL in Table 11.3.3-1 and for SBFD UL in Table 11.3.3-2.

Table 11.3.3-1: Case 3, SBFD DL

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput for different BS Tx powers, ranging (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid-shifts (5% to 100%), and different SBFD antenna configurations. |
| Urban Hotspot -> Urban Hotspot Scenario 2 |  |  |
| Indoor -> Indoor Scenario 3 and Scenario 9 |  |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 |  |  |

Table 11.3.3-2: Case 3, SBFD UL

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 | For SBFD UL, observed throughput degradation is observed at cell edge throughput and average throughput. With higher BS Tx power and lower grid- shifts, the degradation is increased. |
|  | FR2-1 | No degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher BS Tx power and lower grid-shifts. |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | For SBFD UL, observed throughput degradation is observed at cell edge throughput and average throughput. With higher BS Tx power and lower grid-shifts, the degradation is increased. |
| Indoor -> Indoor Scenario 3 and Scenario 9 | FR1 and FR2-1 | No SBFD UL throughput degradation for both average throughput and cell edge throughput is observed. |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 | FR1 | Under FR1 Urban micro 38 dBm Tx power assumption, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher BS Tx power (46 dBm) and lower grid-shifts. |
|  | FR2-1 | For SBFD UL throughput degradation is observed only for cell edge throughput and no degradation is observed for average throughput. |

### 11.3.4 Case 4: aggressor NR TDD UL victim SBFD DU

Case 4 considers SBFD as a victim while NR TDD is operating UL in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in for SBFD DL in Table 11.3.4-1 and for SBFD UL in Table 11.3.4-2.

Table 11.3.4-1: Case 4, SBFD DL

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and scenario 6 | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput respectively for different BS Tx powers, ranging (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid-shifts (5% to 100%), and antenna configurations. |
| Indoor -> Indoor Scenario 3 and Scenario 9 |  |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 |  |  |
| Urban Hotspot -> Urban Hotspot Scenario 2 | FR1 | Some companies’ results show DL throughput degradation is observed only for cell edge throughput due to inter-UE CLI for different grid-shifts (5% to 100%), BS Tx powers (46 dBm to 53 dBm) and for all antenna configurations. The less grid-shift, the larger degradation due to shorter UE-to-UE distance. However more companies show that there is no observed interference for cell edge throughput and cell average throughput for 100% grid-shift, 49 dBm BS Tx power and antenna configuration 2. |

Table 11.3.4-2: Case 4, SBFD UL

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 and FR2-1 | No observed throughput degradation on the SBFD UL for both average throughput and cell edge throughput respectively for different BS Tx powers, ranging (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid-shifts (5% to 100%), and antenna configurations. |
| Indoor -> Indoor Scenario 3 and Scenario 9 |  |  |
| Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8 |  |  |
| Urban Hotspot -> Urban Hotspot Scenario 2 |  |  |

### 11.3.5 General remarks on coexistence findings

For the above cases where no throughput degradation has been observed, no additional coexistence measures are required for SBFD deployment. However, for other cases where throughput degradation has been observed, interference mitigation techniques might need to be considered.