3GPP TSG-RAN WG4 Meeting # 109	R4-2319405

# Chicago, US, 13th - 17th, Nov, 2023

Source: 	Samsung

Title: 	LS on 2Tx-TxD capability and 4Tx-TxD capability

Agenda Item:	8.3.1.2

Document for:	Discussion

- Introduction

The discussion paper address the following issues.

- In terms of 4Tx-TxD capability, whether a FS(per band per band combination) feature can be utilized for single band (None-CA) and whether it is a general approach in RAN2, in case RAN4 targets both single band(Non-CA) and CA.

- In terms of 2Tx-TxD capability, what is the interaction between txDiversity-r16(for single band) and the new FS signalling (for CA)

- What is the interaction between 2Tx-TxD capability and 4Tx-TxD capability.

- Discussion

Background

In last meeting, two LS were sent out to RAN2 regarding TxD capability. [1] seeks for RAN2’s help to introduce a FS 2Tx-TxD capability for UE to indicate the support of 2Tx-TxD for a band within a band combination, as mentioned the legacy txDiversity-r16 was defined as per band in Rel-16. While [2] clarifies the legacy txDiversity-r16 is for 2Tx only while ask for RAN2’s help to introduce a FS 4Tx-TxD capability. [2] is excerpted as below.

It is understood the main target in RAN4 is the single band (Non-CA) case within Rel-18, FS is considered for future proof in case in future 5Tx transmitting simultaneously are supported (though in our view it is far away).

4Tx-TxD: Whether a “FS” feature can be utilized for single band (None-CA)

One issue identified after this LS sent out is that, it appears RAN4 assumes:

- Either a “FS” feature can also be used for single band (None-CA) →Assumption 1

- Or, RAN4 assumes RAN2 would introduce two separate 4T-TxD capabilities (one “per band” 4T-TxD for single band, one “FS” 4T-TxD for the constituent band within a BC) even without RAN4’s explicitly indication. →Assumption 2

For Assumption 2, we think RAN4 should at least clearly tell RAN2 both Non-CA and CA are targeted.

So seems most likely many RAN4 experts take Assumption 1. With regard to whether a FS feature (which would be put under "FeatureSetcombination") can be utilised for single band (None-CA)”, the following are three joint observations made by three Samsung RAN2 experts.

Observation 1: In principle, FS 4Tx-TxD can be utilised for single band (None-CA) case from signalling perspective (as the size of “BandCombinationList” can be 1), by reporting this BC (including only one band) and create the FS for this BC.

Observation 2: If RAN4 targets both single band (None-CA) and CA, it should be clearly delivered to RAN2 and left RAN2 to determin introduce a single FS capability or two separate capabilities.

Observation 3: If 4T-TxD for CA is far away, introduce a per band capability might be more reasonable.

Further, we would like to point out the scope for FR1_enh is single band (None-CA), CA case with 4Tx in a constituent band calls for 5Tx working simultaneously in our view is far away. Same logic applies for 3Tx-TxD if in Rel-19 3Tx for single band for handheld UE would be introduced.

Proposal 1: It is proposed to re-discuss the target of 4Tx-TxD within Rel-18.

- Target only single band (None CA) →Samsung’s preference

- Target both single band (None CA) and CA

2Tx-TxD

If a new FS capability introduced, the relationship between existing capability (txDiversity-r16) and this new FS capability should be clarified. Three approaches provided for discussion and confirmation, and our RAN2 colleagues think RAN4 shall clarify this.

- Approach 1: new capability is included in a certain band/BC, it will override per band capability for this band/BC. Otherwise, the UE applies per band capability. The presence of new capability signaling requires the support of txDiversity-r16. In case of absence of the signaling follows txDiversity-r16 in the corresponding band. (Samsung’s understanding: this precludes the case: TxD is for single band A, and 1Tx for A within CA_A-B, so we do not think Approach 1 is reasonable though it is the common RAN2 way)

- Approach 2: existing capability is applied to non-CA only. New capability should be applied for CA band combination. It means the absence of new capability signaling means no support of tx diversity in the band/band combination.

- Approach 3: existing capability is applied to non-CA only. New capability should be applied for CA band combination. It means the absence of new capability signaling means no support of tx diversity in the band/band combination. The presence of new capability signaling requires the support of txDiversity-r16 →Samsung’s understanding/preference

Please Note that from signalling overhead perspective, Approach 2 and Approach 3 are the same, but from UE implementation perspective, they are different.

Proposal 2: In terms of 2Tx-TxD, RAN4 discuss the interaction between the existing capability (txDiversity-r16) and this new FS capability. (Above three approaches provided, Approach 3 is our understanding)

The interaction between 2Tx-TxD and 4Tx-TxD

Another issue may require some discussion is what is the interaction between 2Tx-TxD and 4Tx-TxD.

In general we think there is no need to couple these two capabilities, UE can declare the maximum capability according to the implementation, i.e., the presence of 4Tx-TxD signaling does not require the support of 2Tx-TxD. More explicitly, if a UE supports PC1.5 by 4*23dBm PA (4Tx-TxD), and the power fall back 3dB to PC2, it still requires at least 2*23dBmPA working simultaneously (2Tx-TxD). However if UE is equipped with 3*23dBm PA+1*26dBm PA(4Tx-TxD), and the power fall back from PC1.5 to PC2, one kind of UE implementation is only the single 26dBm PA is utilized.

Proposal 3: The presence of 4Tx-TxD signaling does not require the support of 2Tx-TxD signalling.

- 3. Conclusion

Observation 1: In principle, FS 4Tx-TxD can be utilised for single band (None-CA) case from signalling perspective (as the size of “BandCombinationList” can be 1), by reporting this BC (including only one band) and create the FS for this BC.

Observation 2: If RAN4 targets both single band (None-CA) and CA, it should be clearly delivered to RAN2 and left RAN2 to determin introduce a single FS capability or two separate capabilities.

Observation 3: If 4T-TxD for CA is far away, introduce a per band capability might be more reasonable.

Proposal 1: It is proposed to re-discuss the target of 4Tx-TxD within Rel-18.

- Target only single band (None CA) →Samsung’s preference

- Target both single band (None CA) and CA

Proposal 2: In terms of 2Tx-TxD, RAN4 discuss the interaction between the existing capability (txDiversity-r16) and this new FS capability.

- Approach 1: new capability is included in a certain band/BC, it will override per band capability for this band/BC. Otherwise, the UE applies per band capability. The presence of new capability signaling requires the support of txDiversity-r16. In case of absence of the signaling follows txDiversity-r16 in the corresponding band. (Samsung’s understanding: this precludes the case: TxD is for single band A, and 1Tx for A within CA_A-B, so we do not think Approach 1 is reasonable though it is the common RAN2 way)

- Approach 2: existing capability is applied to non-CA only. New capability should be applied for CA band combination. It means the absence of new capability signaling means no support of tx diversity in the band/band combination.

- Approach 3: existing capability is applied to non-CA only. New capability should be applied for CA band combination. It means the absence of new capability signaling means no support of tx diversity in the band/band combination. The presence of new capability signaling requires the support of txDiversity-r16 →Samsung’s understanding/preference

Proposal 3: The presence of 4Tx-TxD signaling does not require the support of 2Tx-TxD signalling.

A clarified LS is provided in annex for reference, which can be modified to be consistent with the conclusions made during the meeting.

- 4. Reference

[1] R4-2317762, draft LS on new per band per BC TxD capability, Huawei, RAN4#108bis

[2] R4-2317617, LS on signalling for 4Tx TxD, vivo, RAN4#108bis

# 5. Annex

3GPP TSG RAN WG4 Meeting #109	R4-231xxxx

Chicago, USA, November 13th – 17th, 2023

Title:                LS on signalling for 2Tx-TxD and 4Tx-TxD

Response to:

Release:		Rel-18

Work Item:		NR_ENDC_RF_FR1_enh2-Core

Source:		RAN4

To:		RAN2

Cc:

Contact person:		Yuanyuan Zhang, tina55 (dot) zhang (at) samsung (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		None

# 1	Overall description

In RAN4#108bis meeting, RAN4 sent a LS (R4-2317617) seeking for RAN2’s help to introduce a new per band per band combination (FS) capability for UE to indicate the support of TxDiversity for 4Tx in the band configured. Meanwhile RAN4 sent a LS (R4-2317762) asking for RAN’2 help to introduce a new FS UE capability to indicate the support of TxDiversity for 2Tx in the band configured.

After further discussion during RAN4#109, RAN4 would like to have following further clarification with RAN2.

- In terms of 4Tx-TxDiversity capability, the assumption in RAN4 of R4-2317617 is that a FS feature can also be utilized for single band(None-CA) in principle, so for future proof RAN4 thought a FS 4Tx-TxDiversity signaling could be considered even the primary target within Rel-18 is Non-CA only.

After further discussion during this meeting, RAN4 confirms that within Rel-18 only single band (None-CA) is the target, and left to RAN2 to determine the suitable granularity.

- In terms of 2Tx-TxDiversity capability, RAN4 discussed the interaction between the existing capability (txDiversity-r16) and the new FS capability, and the following agreement is made.

Existing capability is applied to non-CA only. New capability should be applied for CA band combination. It means the absence of new capability signaling means no support of tx diversity in the band/band combination. The presence of new capability signaling requires the support of txDiversity-r16.

- In terms of the interaction between 2Tx-TxDiversity and 4TxD-TxDiversity, RAN4 concludes that the presence of 4TxD-TxDiversity signaling does not require the support of 2Tx-TxDiversity.

# 2	Actions

To RAN2:

ACTION: 	RAN4 respectfully ask RAN2 to take above clarification into account.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #110	26 Feb – 01 Mar, 2024		        Athens, Greece

TSG-RAN WG4 Meeting #110bis	15 – 19 Apr, 2024		             China