3GPP TSG-RAN WG4 Meeting # 109	R4-2319407

Chicago, USA, November 13th – 17th, 2023

Source: 	Samsung

Title: 	              Reply LS on higher power limit capability for NR-DC

Agenda Item:          11.2.5

Document for:	Discussion/Approval

- Introduction

This paper address the question raised in RAN2 reply LS [1].

- Discussion

In RAN4#108, it was identified that higherPowerLimit-r17 was introduced in TS 38.306/38.331 under CA-ParametersNR for NR inter-band ULCA while no similar IE was added under MRDC-Parameters for inter-band EN-DC. An LS [2] was sent out to RAN2 with the issue identified.

In RAN2#123bis, a reply LS was sent out from RAN2 with the following question seeking for further clarification from RAN4.

- Question:  For clarifying the applicability of the parameter “higherPowerLimit-r17”, RAN2 would like to confirm whether the higher power limit is applicable to NR-DC?

In our understanding, dating back to Rel-17, the higher power limit feature was introduced base on power class 2 and the total power can be extended to 23dBm+26dBm=27.8dBm in order to fully utilize the PA capability. While NR-DC can only support PC3 so far, so clearly this feature is not applicable for NR-DC in Rel-17.

In Rel-18, during the discussion in Cov_enh WI, this feature is extended to PC3 with total power 23dBm+20dBm achievable but only applicable for NR-U inter-band NR-CA and inter-band EN-DC. Two relevant CRs[3][4] were endorsed in RAN4#108bis, some description were excerpted below.

At least till now, extending higher power limit feature to NR-DC has never been raised in RAN4 from our observation, probably because FR1 NR-DC is less attractive from NW implementation perspective.

With above, we propose the following “Yes-No” kind of simple answer to the question. A reply LS is attached in annex.

- Answer: RAN4 confirms that higher power limit capability is not applicable to NR-DC.

# 3. Conclusion

Possible answer to the question raised in [1]

- Answer: RAN4 confirms that higher power limit capability is not applicable to NR-DC.

# 4. Reference

[1] R2-2311441, Reply LS on higher power limit capability for inter-band UL DC, RAN2#123-bis, MediaTek

[2] R4-2313619, Draft LS on higher power limit capability for inter-band UL DC, RAN4#108, MediaTek

[3] R4-2316336_draft CR to TS38.101-3 for Introducing new scenarios for increase higher power limit for ENDC, RAN4#108bis, ZTE Corporation, vivo, Huawei, CHTTL, Xiaomi, Samsung

[4] R4-2315848 draft CR to TS 38.101-1 for Introducing new scenarios for increase higher power limit for CA, RAN4#108bis, vivo, ZTE, Huawei, CHTTL, Samsung, Xiaomi

# 5. Annex

3GPP TSG RAN WG4 Meeting #109	R4-2319407

Chicago, USA, November 13th – 17th, 2023

Title:	Reply LS on higher power limit capability for NR-DC

Response to:	Reply LS on higher power limit capability for inter-band UL DC (R2-2311441)

Release:		Rel-17

Work Item:		Power_Limit_CA_DC

Source:		RAN4

To:		RAN2

Cc:

Contact person:		Yuanyuan Zhang, tina55 (dot) zhang (at) samsung (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		None

1	Overall description

RAN4 respectfully thanks RAN2 for the LS on higher power limit capability for inter-band UL DC in R2-2311441.

RAN4 discussed the following question raised in the LS, and have the following answer based on the group’s common understanding on this topic:

- Question:  For clarifying the applicability of the parameter “higherPowerLimit-r17”, RAN2 would like to confirm whether the higher power limit is applicable to NR-DC?

- Answer: RAN4 confirms that higher power limit capability is not applicable to NR-DC.

2	Actions

To RAN2:

ACTION: 	RAN4 respectfully asks RAN2 to take the above answer into account in their further work.

3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #110	26 Feb – 01 Mar, 2024		Athens, Greece

TSG-RAN WG4 Meeting #110bis	15 – 19 Apr, 2024		                    China