3GPP TSG-RAN WG4 Meeting#109	R4-2319436

# Chicago, IL, US, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0136 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Implementation of two-band Tx switching with dual TAG in an earlier release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_channel_raster_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Requirements for UL Tx switching with dual TAG were introduced for band pairs of 2-4 band combiantions in Rel-18. UL Tx switching with dual TAG can be supported in earlier releases without inter-operability problems for Tx switching between two carriers/bands (no change of 38.214 and no new RRC signaling required for support of band combinations with Tx switching and dual TAG). The applicable requirements for UEs implementing Tx swtiching and dual TAG in an earlier release should be made clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Rel-18 requirements applicable for UE implementing UL Tx switching with dual TAG in an earlier release are specified in Clause 5.2.2: time mask requirements for Tx switching with dual TAG and Clause 5.4: requirements for DL interruption with dual TAG for two bands (the length of the switching period the same for single- and dual TAG). for the respective UL Tx switching configurations specified in Rel-16 (between two carriers) and Rel-17 (between two bands with up to 2T and two contiguous carriers within a band). … |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-18 requirements applicable for UE implementing UL Tx switching with dual TAG for two-band Tx switching band combinations in an earlier release are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 5.2.2	Inter-band CA

Requirements for a Rel-16 UE for additional NR inter-band CA configurations within FR1 compared to TS 38.101-1 of Rel-16 [2] are introduced via this clause.

Table 5.2.2-0: NR inter-band CA UE power class within FR1

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Inter-band CA Power Class 2 | TDD, FDD and TDD | Rel-15 | Table B.4.2-1 |
| Inter-band CA Power Class 3 | FDD, TDD, SDL and TDD, FDD and TDD | Rel-15 |  |

Table 5.2.2-1: NR inter-band CA within FR1

| Feature | DL/UL | Maximum number of bands | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations within NR FR1 | DL | 4 | 5 | A, B, C | TDD, FDD, SDL and FDD, SDL and TDD, FDD and TDD | Rel-15 | Table B.4.2-1 |
|  | UL | 2 | 2 | A | TDD, FDD, FDD and TDD | Rel-15 |  |
|  |  | 1 | 2 | B, C, 2A |  |  |  |
|  |  | 2 | 3 | A, B, C |  |  |  |

Requirements for Rel-16 and Rel-17 UEs for NR inter-band CA configurations with UL Tx switching and dual TAG compared to the respective Rel-16 and Rel-17 versions of TS 38.101-1 are introduced via this clause.

Table 5.2.2-2: NR inter-band CA within FR1 with UL TX switching and dual TAG

| Feature | DL/UL | Maximum number of bands | number of CCs of band pair | CA BW Classes for band pair | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the UL TX switching configuration was introduced for dual TAG) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UL TX switching between two carriers with dual TAG | UL | 2 | 2 | A | FDD, TDD, FDD and TDD | Rel-16 | Time-mask requirements introduced by the Rel-18 WI NR_MC_enh for dual TAG in clause 6.3A.3.3.1 of [2] applicable to requirements in clause 6.3A.3.3.2. |
| UL TX switching between two bands with dual TAG and up to two antennas per band | UL | 2 | 4 | A, B, C | FDD, TDD, FDD and TDD | Rel-17 | Time-mask requirements introduced by the Rel-18 WI NR_MC_enh for dual TAG in clause 6.3A.3.3.1 of [2] applicable to requirements in clauses 6.3A.3.3.2 – 6.3A.3.3.5. |
| NOTE 1: A UE supporting dual TAG for an UL Tx switching band pair of a band combination with Tx switching indicates supportedNumberTAG = ‘n2’ for the said band combination. NOTE 2: A UE supporting a band combination with Tx switching and dual TAG shall meet the requirements in Table 4.2.2-1 and Table 4.2.2-2 (for a UL Tx switching band pair with UL-MIMO and UL intra-band CA) also with dual TAG configured. |  |  |  |  |  |  |  |

## 5.3	Additional NR SUL configurations for NR frequency range 1

Requirements for a Rel-17 UE for additional NR SUL configurations within FR1 compared to TS 38.101-1 of Rel-17 [2] are introduced via this clause.

When a UE is configured with both NR UL and NR SUL carriers in a serving cell with active transmission either on the UL carrier or SUL carrier, the release independent features in clause 5.1 are applicable for the UL carrier and the SUL carrier, respectively.

Table 5.3-1: NR SUL within FR1

| Feature | DL/UL | number of bands | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the SUL configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band SUL configurations within NR FR1 | DL | 3 | 3 | A, C | TDD, FDD, FDD and TDD | Rel-15 | Table B.4.3-1 |
|  | UL | 2 | 3 | A, C | TDD and SUL, FDD and SUL | Rel-15 |  |

## 5.4	Other release independent features for NR frequency range 1

This clause covers requirements for a Rel-15 UE coming from all other release independent features that are not covered under clause 5.1, 5.2 and 5.3, e.g. generic baseband requirements or requirements that are not band/CA/SUL configuration specific.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| RRM requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.1-1 | Rel-16 WI NR_HST introduced band independent RRM requirements: see Table C.1-1 |
| UE demodulation requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.2-1 | Rel-16 WI NR_HST introduced band independent UE demodulation requirements: see Table C.2-1 |
| RF requirements for 4Rx UEs | Rel-15 | Table B.4.10-1, Table B.4.10-2 |  |
| [Transparent Tx diversity] | [Rel-15] | [Table B.4.11-1] | [Rel-17 WI NR_RF_TxD introduced transparent Tx diversity requirements: see Table B.4.11-1] |
| UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference | Rel-15 | Table B.3.3-1 | Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration. |
| RRM enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 2) | Table C.3-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent RRM enhancement: see Table C.3-1 |
| HST-SFN CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 3) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-SFN CA demodulation enhancement: see Table C.4-1 |
| HST-DPS CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-15 (NOTE 4) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-DPS CA demodulation enhancement: see Table C.4-1 |
| UL TX switching between two carriers with dual TAG | Rel-16 (NOTE 5) | Table 8.2.2.2.10-1 | Rel-18 WI NR_MC_enh introduced requirements for UL Tx switching for dual TAG. |
| UL TX switching between two bands with dual TAG and up to two antennas per band | Rel-17 (NOTE 5) | Table 8.2.2.2.10-1 Table 8.2.2.2.10A-1 | Rel-18 WI NR_MC_enh introduced requirements for UL Tx switching for dual TAG. |
| NOTE 1: Rel-15 UEs supporting the high speed train are assumed to read the Rel-16 high speed train scenario information, which is broadcast to all UEs. NOTE 2: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 3: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 4: Rel-15 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 1: A UE supporting dual TAG for an UL Tx switching band pair of a band combination with Tx switching indicates supportedNumberTAG = ‘n2’ for the said band combination. |  |  |  |

## 5.5	Additional Inter-band NR-DC configurations for NR frequency range 1

< end of changes >