# 3GPP TSG-RAN WG4 Meeting # 109				R4-2319438

# Chicago, US, November 13 – 17, 2023

Agenda item:	10

Source: 	Ericsson

Title: 	Input to the Rel-18 RAN4 UE feature list for NR

Document for:	Approval

- Introduction

This contribution includes proposed descriptions for feature groups 28-1 and 33-1 of the RAN4 UE feature list for Rel-18 NR.

- Proposal

- NR_channel_raster_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_channel_raster_enh | 28-1 | Enhanced channel raster | The UE supports the radio requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1 and in TS 38.101-5. | N/A | Yes |  | N/A (not defined) | Per Band | No | FR1 only | The feature is supported for applicable bands in FDD-TDD and FR1/FR2 combinations | Applies only for bands with a 100 kHz channel raster for both TN and NTN. | Mandatory for 1) RedCap UEs from Rel-17 in supported bands. 2) In bands as specified in 38.101-1 and 38.101-5 (based on operator requests) |

- NonCol_intraB_ENDC_NR_CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NonCol_intraB_ENDC_NR_CA | 33-1 | Intra-band non-collocated EN-DC/NR-CA deployment | For a supported intra-band NR CA configuration, the UE supports a relative transmission timing difference between UL carriers as specified in Table 7.5.4-1 and a relative transmission timing difference between DL carrier as specified in Table 7.6.4-2 of TS 38.133. The feature is supported for up to 2-layer MIMO per carrier. | N/A | Yes |  | The UE supports requirements specified in TS38.133 for intra-band contiguous NR CA with collocated carriers: a relative transmission timing difference between UL carriers of 5.21 µs and a relative transmission timing difference between DL carrier of 3 µs. | Per BC | TDD only | FR1 only | N/A. Capability only supported within an TDD band. | [Supported for band n77/n78 only] | Optional |