# 3GPP TSG-RAN WG4 Meeting # 109	R4-2319462

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3772 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Handover with PSCell from SA to EN-DC with target FR2 PSCell in A.7.3.1.6 and A.7.3.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Perf |  |  |  |  |  | Date: |  |  | 2023-11-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is test for HO with PScell from SA to EN-DC with FR2 PScell addition, based on the WF R4-2314396 (WF on work split for FR1-FR2 test cases). Test case; Volunteer company. Handover with PSCell: FR1+FR2 NR-DC to FR1+FR2 NR-DC; Ericsson. Handover with PSCell: NR-SA FR1 to EN-DC with FR2 PSCell; OPPO. Handover with PSCell: EN-DC with FR1 PSCell to EN-DC with FR2 PSCell; Nokia. TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PCell; Ericsson. TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell; Xiaomi. TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PSCell; Huawei. TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PSCell; Nokia. A.5.5.3.X2 Fast SCell Activation and deactivation of SCell in FR2 in inter-band; Qualcomm. A.5.5.X1.Y1 E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell; Apple. |
| Summary of change: |  | Update test cases and also solve testability issues caused by uncalibrated LTE/FR1 link as below: Test; Test Description; Test Scenario; Testability Issue #; Testability Issue. A.7.3.1.6; Handover with PSCell from SA to EN-DC; unknown FR2 target cell; FR1+FR2 LTE+FR2; 1, 2, 3; *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (in multiple occasions) via uncalibrated FR1 *UE required to trigger event B2, which requires evaluation of uncalibrated FR1 cell vs threshold1 and evaluation of uncalibrated LTE cell vs threshold2 *UE required to transmit MR over uncalibrated FR1 link *HO latency delay depends on UE reception of RRC config (HO cmd) on FR1 *UE required to RACH on target LTE PCell. A.7.3.1.7; HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell; FR1+FR2 LTE+FR2; 1, 2, 3; *FR1 cell not guaranteed to be detectable, so UE can fail initial acquisition on NR carrier (and not detect the cell) *UE required to receive RRC config (in multiple occasions) via uncalibrated FR1 *UE required to trigger event B2, which requires evaluation of uncalibrated FR1 cell vs threshold1 and evaluation of uncalibrated LTE cell vs threshold2 *UE required to transmit MR over uncalibrated FR1 link *HO latency delay depends on UE reception of RRC config (HO cmd) on FR1 *UE required to RACH on target LTE PCell. |
| Consequences if not approved: |  | The test cases are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.3.1.6, A.7.3.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change 1>

A.7.3.1.6	Handover with PSCell from SA to EN-DC with unknown FR2 target PScell

A.7.3.1.6.1	Test Purpose and Environment

This test is to verify the PSCell addition delay requirements specified in clause 6.1.5.2 for handover with PSCell from NR SA to EN-DC with sequential processing when the SMTC of target unknown PSCell is present in RRCConnectionReconfiguration and the target PSCell is in FR2.

A.7.3.1.6.2	Test Parameters

The test scenario comprises of three carriers and one cell on each carrier. Cell 1 is the NR PCell, Cell 2 is an inter-RAT E-UTRAN neighbour cell and Cell 3 is an NR neighbour cell, on radio channel 1 in FR1, radio channel 2 in E-UTRAN in FR1 and radio channel 3 in FR2, respectively.

The test consists of three successive time periods, with time durations of T1, T2, T3 and T4 respectively. No gap patterns are configured in the test case.

At the start of time duration T1, the UE does not have any timing information of cell 2 and cell 3, and the UE is only monitoring Cell 1. During T1, only Cell1 is known to the UE.

Starting T2, cell 2 and cell 3 become detectable. The RRC message implying handover with PSCell shall be sent to the UE during period T2 after the UE has reported Event B2 and Event B1. The start of T2 is the instant when the last TTI containing the RRC message implying handover with PSCell is sent to the UE. The handover with PSCell message shall contain Cell 2 as the target cell and Cell 3 as PSCell-to-be added. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added.

The point in time at which the RRC message implying HO (Cell 2) with PSCell (Cell 3) is received at the UE antenna connector defines the start of period T3.

During T3, the UE shall carry out random access (i.e., transmit the PRACH) towards the Cell 2 and Cell 3. The test system shall observe the UE sends PRACH to E-UTRAN Cell 2 and PSCell (Cell 3) during period T3. Reception by the test system of the PRACH preambles defines the end of T3.

During T4, the UE shall send periodic CSI reports in PSCell and the test system shall observe the periodic reporting of CSI for PSCell.

Supported test configurations are shown in table A.7.3.1.6.2-1. General test parameters are provided in Table A.6.3.1.14.1-2. Cell specific test parameters for NR Cell 1, E-UTRAN PCell Cell 2 and NR PScell Cell 3 are provided in Tables A.6.3.1.14.1-3, A.6.3.1.14.1-4 and A.6.3.1.14.1-5 respectively.

Table A.7.3.1.6.2-1: Handover with PSCell from NR SA to EN-DC test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PCell: LTE FDD Target PSCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PCell: LTE TDD Target PSCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target PCell: LTE FDD Target PSCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target PCell: LTE TDD Target PSCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.3.1.6.2-2: General test parameters for handover with PSCell from NR SA to EN-DC

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| RF Channel Number |  |  |  | 1, 2, 3 | Three radio channels are used for this test. One for FR1 source PCell, second for E-UTRA target PCell and third for target NR PSCell |
| Initial | Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Condition | Neighbour cell |  |  | Cell2, Cell3 | Neighbour cell on RF channel number 2 and 3. |
| Final Condition | Active PCell |  |  | Cell2 | E-UTRA PCell on RF channel number 2. |
|  | Active PSCell |  |  | Cell3 | PSCell on RF channel number 3. |
|  | Neighbour Cell |  |  | Cell1 | RF channel number 1 |
| NR measurement quantity |  |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  |  | RSRP |  |
| Event B2 |  | Threshold1 | dBm | As specified in Table A.6.3.1.4-3 | Absolute NR SS-RSRP threshold for event B2 |
|  |  | Threshold2EUTRAN | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
|  |  | Hysteresis | dB | 0 |  |
|  |  | TimeToTrigger | s | 0 |  |
| Event B1 |  | Hysteresis | dB | 0 | Hysteresis for evaluation of event B1. |
|  |  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event B1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  |  | Time to Trigger | S | 0 |  |
| Filter coefficient |  |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF | Non-DRX test |
| Access Barring Information |  |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cell 1 and 2 |  |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| Cell-individual offset for cells on RF channel number 2 |  |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  |  | dB | 0 | Individual offset for cells on carrier frequency of cell3. |
| T1 |  |  | s | 5 |  |
| T2 |  |  | s | ≤5 | During this time the cell 2 and cell 3 shall be known. |
| T3 |  |  | s | 1 | During this time the UE perform HO with PSCell addition. |
| T4 |  |  | s | 0.5 | During this time the UE sends CSI reports for PSCell (Cell 3). |

Table A.7.3.1.6.2-3: Cell specific test parameters for NR Cell 1

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3, 4 | 1 |  |  |
| Duplex mode |  |  | 1, 2 | FDD |  |  |
|  |  |  | 3, 4 | TDD |  |  |
| BWchannel |  | MHz | 1, 2, 3 | 10: NRB,c = 52 |  |  |
|  |  |  | 4 | 40: NRB,c = 106 |  |  |
| PDSCH reference measurement channel |  |  | 1, 2 | SR.1.1 FDD |  |  |
|  |  |  | 3 | SR.1.1 TDD |  |  |
|  |  |  | 4 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1, 2 | CR.1.1 FDD |  |  |
|  |  |  | 3 | CR.1.1 TDD |  |  |
|  |  |  | 4 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1, 2 | TRS.1.1 FDD |  |  |
|  |  |  | 3 | TRS.1.1 TDD |  |  |
|  |  |  | 4 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3, 4 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2, 3 | SSB.1 FR1 |  |  |
|  |  |  | 4 | SSB.2 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2, 3 | -96 |  |  |
|  |  |  | 4 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4 | -100 | -104 | -100 |
| NocNote2 |  | dBm/SCS | 1, 2, 3 | -100 | -104 | -100 |
|  |  |  | 4 | -97 | -101 | -97 |
| Ês/Noc |  | dB | 1, 2, 3, 4 | 12 | 0 | -4 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4 | 12 | 0 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 3 | -88 | -104 | -104 |
|  |  |  | 4 | -85 | -101 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 3 | -59.78 | -73.04 | -70.59 |
|  |  |  | 4 | -53.68 | -66.9448 | -64.49 |
| Propagation condition |  |  | 1, 2, 3, 4 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.7.3.1.6.2-4: Cell specific test parameters for E-UTRA Cell 2

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4 | 2 |  |  |
| Duplex mode |  | 1, 3 | FDD |  |  |
|  |  | 2, 4 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 2, 4 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 2, 4 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4 | 10 MHz: NRB,c = 50 |  |  |
| PRACH ConfigurationNote2 |  | 1, 2 | 4 |  |  |
|  |  | 3, 4 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2 | 10 MHz: R.3 FDD |  |  |
|  |  | 3, 4 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3, 4 | 10 MHz: R.6 FDD |  |  |
| OCNG PatternsNote3 |  | 1, 2 | 10 MHz: OP.10 FDD |  |  |
|  |  | 3, 4 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4 | -98 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4 | -Infinity | 8 | 78 |
| Ês/IotNote6 | dB | 1, 2, 3, 4 | -Infinity | 78 | 78 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4 | -Infinity | -90 | -90 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4 | -Infinity | -90 | -90 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4 | -67.21 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

Table A.7.3.1.6.2-5: Cell specific test parameters for NR Cell 3

| Parameter | Unit | Config | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 |
| E-UTRA Channel Number |  | 1,2, 3, 4 | 1 |  |  |  |
| NR Channel Number |  | 1,2, 3, 4 | 2 |  |  |  |
| Duplex Mode |  | 1,2, 3, 4 | TDD |  |  |  |
| TDD configuration |  | 1,2, 3, 4 | TDDConf.3.1 |  |  |  |
| BWchannel | MHz | 1,2, 3, 4 | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  | 1,2, 3, 4 | 48 |  |  |  |
| Initial BWP Configuration |  | 1,2, 3, 4 | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP Configuration |  | 1,2, 3, 4 | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| PRACH configuration on cell 3 |  | FR2 PRACH configuration 2 | Captured in A.3.8.3.2 |  |  |  |
| TRS Configuration |  | 1,2, 3, 4 | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  | 1,2, 3, 4 | TCI.State.2 |  |  |  |
| PDSCH Reference measurement channel |  | 1,2, 3, 4 | SR.3.3 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1,2, 3, 4 | CR.3.2 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | 1,2, 3, 4 | CCR.3.7 TDD |  |  |  |
| OCNG Patterns |  | 1,2, 3, 4 | OP.3 |  |  |  |
| SSB configuration |  | 1,2, 3, 4 | SSB.2 FR2 |  |  |  |
| SMTC configuration |  | 1,2, 3, 4 | SMTC.2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2, 3, 4 | 120 |  |  |  |
| TRS Configuration |  | 1,2, 3, 4 | TRS.2.1 TDD |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1,2, 3, 4 | CSI-RS.3.1 TDD |  |  |  |
| reportConfigType |  | 1,2, 3, 4 | periodic |  |  |  |
| reportQuantity |  | 1,2, 3, 4 | cri-RI-PMI-CQI |  |  |  |
| CSI reporting periodicity | slot | 1,2, 3, 4 | 40 |  |  |  |
| CSI reporting offset | slot | 1,2, 3, 4 | 4 |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2, 3, 4 | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Propagation condition |  | 1,2, 3, 4 | AWGN |  |  |  |

Table A.7.3.1.6.2-6: OTA related test parameters

| Parameter | Unit | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  | Setup 2a according to clause A.3.15.2.1 |  |  |  |
| Assumption for UE beamsNote 6 |  | Rough |  |  |  |
| Ês Note2 | dBm/SCS | -Infinity | -81 |  |  |
| SSB_RPNote2, Note 4 | dBm/SCS | -Infinity | -81 |  |  |
| BB Note 2, Note 7 | dB | -Infinity | 4.88 |  |  |
| IoNote 2, Note 4 | dBm/95.04 MHz | N/A | -56.41 |  |  |
| Note 1: Void Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |

A.7.3.1.6.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 175 ms Note1 from the beginning of time period T3.

The UE shall start to transmit the PRACH to Cell 3 less than 692 ms Note2 from the beginning of time period T3.

The rate of correct PSCell addition observed during repeated tests shall be at least 90%.

NOTE1:	The handover delay can be expressed as specified in clause 6.1.5.2:

DHOwithPSCell_PCell = RRC procedure delay + Tinterrupt, where

RRC procedure delay = 50 ms.

Tinterrupt = 125 ms.

NOTE2:	The PSCell addition delay can be expressed as follows as specified in clause 6.1.5.2.2:

DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_HO + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms, where:

RRC procedure delay = 50 ms and is specified in clause 12 in TS 38.331 [2].

Tprocessing is as defined as 50ms in the test.

Tsearch_HO is as defined as 80ms in the test.

Tsearch_PSCell is as defined as 480ms in the test.

T∆ is defined as 20ms in the test.

TPSCell_ DU is defined as 10ms in the test.

This gives a total of 692 ms.

# <End of Change 1>

# <Start of Change 2>

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell

A.7.3.1.7.1	Test purpose and environment

The purpose of this test is to verify that the delay of HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell are within the requirements stated in clause 6.1.5.2.2 of TS 36.133 [15] for the case when the E-UTRA PCell and FR2 PSCell are known by the UE at the time of handover with PSCell.

The test consists of three successive time periods with duration of T1, T2, and T3. There are three carriers each with one cell. Before the test starts the UE is connected to Cell 1 (source FR1 PCell) on radio channel 1 (FR1 PCC) and is aware of Cell 2 (target E-UTRA PCell) on radio channel 2 and Cell 3 (FR2 target PSCell) on radio channel 3. The UE is monitoring both cell 2 and cell 3 before receives a RRC message implying handover with PSCell.

At the start of time duration T1, the UE does not have any timing information of Cell 2 and Cell 3. Starting T2, Cell 2 and Cell 3 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins to enable inter-RAT frequency monitoring. The test system shall send a RRC message to the UE to trigger HO (Cell 2) with PSCell (Cell 3) during period T2, after UE has reported Event B2 and Event B1. The handover with PSCell message shall contain Cell 2 as the target cell and Cell 3 as PSCell-to-be added. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added.

The point in time at which the RRC message implying HO (Cell 2) with PSCell (Cell 3) is received at the UE antenna connector defines the start of period T3. During T3, the UE shall carry out random access (i.e., transmit the PRACH) towards the Cell 2 and Cell 3. The test system shall observe the UE sends PRACH to Cell 2 and Cell 3 during period T3. Reception by the test system of the PRACH preambles defines the end of T3.

The test system shall observe the UE sends PRACH to E-UTRAN Cell 2 and PSCell (Cell 3) during period T3.

During T4, the UE shall send periodic CSI reports in PSCell and the test system shall observe the periodic reporting of CSI for PSCell.

Supported test configurations are shown in A.7.3.1.7.1-1. The test parameters for the E-UTRA cell are given in Table A.3.7.2.2-1. The E-UTRA cell once set up is not changed across time. The test parameters for NR cell are given in Tables A.7.3.1.7.1-2, cell-specific parameters in A.7.3.1.7.1-3, A.7.3.1.7.1-4, A.7.3.1.7.1-5 and OTA parameters in A.7.3.1.7.1-6 below.

Table A.7.3.1.7.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | Source FR1 PCell: NR FDD, SSB SCS 15kHz, data SCS 15 kHz, 10 MHz bandwidth Target PCell: LTE FDD, Target PSCell: NR TDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| 2 | Source FR1 PCell: NR FDD, SSB SCS 15kHz, data SCS 15 kHz, 10 MHz bandwidth Target PCell: LTE TDD, Target PSCell: NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.3.1.7.1-2: General Test Parameters for HO with PSCell

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| RF Channel Number |  |  |  | 1, 2, 3 | Three radio channels are used for this test. One for FR1 source PCell, second for E-UTRA target PCell and third for target NR PSCell |
| Initial | Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Condition | Neighbour cell |  |  | Cell2, Cell3 | Neighbour cell on RF channel number 2 and 3. |
| Final Condition | Active PCell |  |  | Cell2 | E-UTRA PCell on RF channel number 2. |
|  | Active PSCell |  |  | Cell3 | PSCell on RF channel number 3. |
|  | Neighbour Cell |  |  | Cell1 | RF channel number 1 |
| NR measurement quantity |  |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  |  | RSRP |  |
| Event B1 |  | Hysteresis | dB | 0 | Hysteresis for evaluation of event B1. |
|  |  | b1-Threshold RSRP | dBm | -93 | Actual RSRP threshold for event B1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  |  | Time to Trigger | S | 0 |  |
| Event B2 |  | b2-Threshold1 | dBm | As specified in Table A.6.3.1.4-3 | Absolute NR SS-RSRP threshold for event B2 |
|  |  | b2-Threshold2EUTRAN | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
|  |  | Hysteresis | dB | 0 |  |
|  |  | TimeToTrigger | s | 0 |  |
| Filter coefficient |  |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF | Non-DRX test |
| Access Barring Information |  |  | - | Not sent | No additional delays in random access procedure |
| PRACH configuration on cell3 |  |  |  | FR2 configuration 2 | Captured in A.3.8.3.2 |
| Time offset between cell 1 and 2 |  |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| Cell-individual offset for cells on RF channel number 2 |  |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  |  | dB | 0 | Individual offset for cells on carrier frequency of cell3. |
| T1 |  |  | s | 5 |  |
| T2 |  |  | s | ≤5 | During this time the cell 2 and cell 3 shall be known. |
| T3 |  |  | s | 1 | During this time the UE perform HO with PSCell addition. |
| T4 |  |  | s | 0.5 | During this time the UE sends CSI reports for PSCell (Cell 3). |

Table A.7.3.1.7.1-3: Cell specific test parameters for Cell 1

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2 | 1 |  |  |
| Duplex mode |  |  | 1, 2 | FDD |  |  |
| BWchannel |  | MHz | 1, 2 | 10: NRB,c = 52 (FDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 2 | SR.1.1 FDD |  |  |
| CORSET reference channel |  |  | 1, 2 | CR.1.1 FDD |  |  |
| TRS configuration |  |  | 1, 2 | TRS.1.1 FDD |  |  |
| OCNG patternNote1 |  |  | 1, 2 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2 | SSB.1 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2 | -96 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2 | -100 | -104 | -100 |
| NocNote2 |  | dBm/SCS | 1, 2 | -100 | -104 | -100 |
| Ês/Noc |  | dB | 1, 2 | 12 | 0 | -4 |
| Ês/IotNote3 |  | dB | 1, 2 | 12 | 0 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -88 | -104 | -104 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2 | -59.78 | -73.04 | -70.59 |
| Propagation condition |  |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.7.3.1.7.1-4: Cell specific test parameters for Cell 2

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2 | 2 |  |  |
| Duplex mode |  | 1, 2 | FDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2 | 10 MHz: NRB,c = 50 |  |  |
| PRACH ConfigurationNote2 |  | 1, 2 | 4 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2 | 10 MHz: R.3 FDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2 | 10 MHz: R.6 FDD |  |  |
| OCNG PatternsNote3 |  | 1, 2 | 10 MHz: OP.10 FDD |  |  |
| PBCH_RA | dB | 1, 2 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2 | -98 |  |  |
| Ês/Noc | dB | 1, 2 | -Infinity | 8 | 78 |
| Ês/IotNote6 | dB | 1, 2 | -Infinity | 78 | 78 |
| RSRPNote6 | dBm/15kHz | 1, 2 | -Infinity | -90 | -90 |
| SCH_RPNote6 | dBm/15kHz | 1, 2 | -Infinity | -90 | -90 |
| IoNote6 | dBm/9MHz | 1, 2 | -67.21 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

Table A.7.3.1.7.1-5: Cell specific test parameters for Cell 3

| Parameter | Unit | Config | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 |
| E-UTRA Channel Number |  | 1,2 | 1 |  |  |  |
| NR Channel Number |  | 1,2 | 2 |  |  |  |
| Duplex Mode |  | 1,2 | TDD |  |  |  |
| TDD configuration |  | 1,2 | TDDConf.3.1 |  |  |  |
| BWchannel | MHz | 1,2 | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  | 1,2 | 48 |  |  |  |
| Initial BWP Configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP Configuration |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| TRS Configuration |  | 1,2 | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  | 1,2 | TCI.State.2 |  |  |  |
| PDSCH Reference measurement channel |  | 1,2 | SR.3.3 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1,2 | CR.3.2 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | 1,2 | CCR.3.7 TDD |  |  |  |
| OCNG Patterns |  | 1,2 | OP.3 |  |  |  |
| SSB configuration |  | 1,2 | SSB.2 FR2 |  |  |  |
| SMTC configuration |  | 1,2 | SMTC.2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 120 |  |  |  |
| TRS Configuration |  | 1,2 | TRS.2.1 TDD |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1,2 | CSI-RS.3.1 TDD |  |  |  |
| reportConfigType |  | 1,2 | periodic |  |  |  |
| reportQuantity |  | 1,2 | cri-RI-PMI-CQI |  |  |  |
| CSI reporting periodicity | slot | 1,2 | 40 |  |  |  |
| CSI reporting offset | slot | 1,2 | 4 |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2 | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Propagation condition |  | 1,2 | AWGN |  |  |  |

Table A.7.3.1.7.1-6: OTA related test parameters

| Parameter | Unit | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  | Setup 2a according to clause A.3.15.2.1 |  |  |  |
| Assumption for UE beamsNote 6 |  | Rough |  |  |  |
| Ês Note2 | dBm/SCS | -Infinity | -81 |  |  |
| SSB_RPNote2, Note 4 | dBm/SCS | -Infinity | -81 |  |  |
| BB Note 2, Note 7 | dB | -Infinity | 4.88 |  |  |
| IoNote 2, Note 4 | dBm/95.04 MHz | N/A | -56.41 |  |  |
| Note 1: Void Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |

A.7.3.1.7.2	Test Requirements

In this test, the UE shall start to transmit the PRACH to E-UTRA Cell 2 less than 105 ms Note1 from the beginning of time period T3.

The UE shall transmit the PRACH to PSCell at latest 137 msNote2 from the beginning of time period T3.

The rate of correct observed PSCell addition delay in HO with PSCell during repeated tests shall be at least 90%.

NOTE1:	The handover delay can be expressed as specified in clause 6.1.5.2:

DHOwithPSCell_PCell = RRC procedure delay + Tinterrupt,

Where RRC procedure delay = 50 ms, Tinterrupt = Tsearch_HO + TIU + Tprocessing is defined in clause 6.1.5.2.1, where

Tsearch = 0 ms,

TIU = 10 ms,

Tprocessing = 45 ms.

Note2:	The PSCell addition delay can be expressed as follows as specified in clause 6.1.5.2:

DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_HO + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms

Where:

TRRC_delay = 50ms

Tprocessing = 45ms

Tsearch_HO = 0 ms

Tsearch_PSCell = 0 ms

T∆ = 20ms

TPSCell_ DU = 1*10+10 = 20 ms

# <End of Change 2>