3GPP TSG-RAN WG4 Meeting # 109								  R4-2319465

Chicago, US, November 13 – 17, 2023

Agenda Item:	8.7.2.3

Source: 	OPPO

Title: 	On RLM and BFD/CBD requirements for FR2_mulitRx_DL

Document for:	Approval

1	Introduction

A WF on Rel-18 FR2_multiRX_DL was approved in RAN4#108bis meeting [1].

In this contribution, we further discuss the remaining issues related to RLM and BFD/CBD requirements for FR2_mulitRx_DL.

2	Discussion

Cell specific RLM and BFD/CBD

Issue 2-1-5: Other aspects of RLM and cell specific BFD/CBD requirements for multi-Rx FFS: For m-DCI scenario, reuse RLM requirements in section 8.1 of TS 38.133, with the clarification that in multi-TRP scenario, the RLM-RS for evaluating out-of-sync and in-sync may originate from two different TRPs.

For m-DCI scenario, we agree to reuse RLM requirements in section 8.1 of TS 38.133. However, about the clarification that in multi-TRP scenario the RLM-RS for evaluating out-of-sync and in-sync may originate from two different TRPs, it seems not necessary to add this in the RLM requirements. Whether and how RLM-RSes are received for evaluation have been agreed to be captured in measurement restriction part.

Proposal 1: For m-DCI scenario, reuse RLM requirements in section 8.1 of TS 38.133, and no need to give any clarification or assumption on whether the RLM-RS originated from two TRPs or not.

TRP specific RLM and BFD/CBD

#### Issue 2-2-1: Updated conditions for TRP specific BFD requirements enhancement for multi-Rx - Both CSI-RSs are not in any CSI-RS resource set with repetition ON - The two CSI-RS resources in the two sets q ̅_0,0 and q ̅_0,1 for beam failure detection are overlapped on the same OFDM symbol. - The CSI-RS in set q ̅_0,0 has same QCL source as the active TCI state of one PDCCH or PDSCH, and the CSI-RS in set q ̅_0,1 has same QCL source as the active TCI state of the other PDCCH or PDSCH - Resources of the active TCI states for the two PDCCHs, or two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report. - [FFS how to capture UE is activated with multi-Rx operation] - [Both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time] - Note: additional conditions are not precluded

The conditions in brackets are generally agreeable for us. TRP seems transparent in RAN4 spec and we prefer to use same or different QCL sources instead of same of different TRPs. We can further discuss the wording in CR stage.

In our views, it seems too restricted that both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time. The beam pair that are being used or reported for transmission of PDCCH/PDSCH could be the selected beams, which are best for simultaneous CSI-RSs measurements at the same time for sure. But it is also possible that other beam pairs not selected before can be used for simultaneous CSI-RSs measurements. Thus, we prefer to remove the condition of ‘Both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time’.

Proposal 2: Remove the condition of ‘Both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time’.

3	Conclusion

Proposal 1: For m-DCI scenario, reuse RLM requirements in section 8.1 of TS 38.133, and no need to give any clarification or assumption on whether the RLM-RS originated from two TRPs or not.

Proposal 2: Remove the condition of ‘Both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time’.

4	Reference

[1] R4-2317332 WF on NR FR2 multi-RX operation RRM requirements (part 1), vivo