3GPP TSG-RAN WG4 Meeting # 109	R4-2319484

# Chicago, US, 13 - 17 November 2023

Agenda Item:	8.22.2.5

Source: 	OPPO

Title: 	Discussion on PRS/SRS bandwidth aggregation

Document for:	Approval

1	Introduction

The feasibility of PRS/SRS bandwidth aggregation has been studied during the SID phase and the corresponding RRM requirements are expected to be specified in the WID phase. Last RAN4 meeting reached some agreements reached in the [1] and there are some open issued in [2]. This contribution will provide our considerations on this topic.

2	Discussion

Issue 3-1-3: Applicable condition in RRC inactive - Proposals - Option 1: OPPO - The scenario that aggregated SRS outside initial UL BWP should be considered when defining RRM requirements in RRC INACTIVE state. - Option 2: Nokia - RAN4 to base RRM requirements for RRC_INACTIVE on the scenario that aggregated carriers with SRS for positioning are located outside the initial UL BWP.

Agreement Positioning SRS bandwidth aggregation is supported for UEs in RRC_CONNECTED. Positioning SRS bandwidth aggregation is supported for UEs in RRC_INACTIVE state. - For the details, Rel-17 positioning SRS configuration for UE in RRC_INACTIVE state outside initial UL BWP can be the starting point

For SRS bandwidth aggregation, RAN1 agreed that the SRS configuration outside initial UL BWP can be the starting point. Consequentially, this scenario should be considered in RAN4 when defining RRM requirements.

Proposal 1: The scenario that aggregated SRS outside initial UL BWP should be considered when defining RRM requirements in RRC_INACTIVE state.

Issue 3-2-1: Conditions for PRS/SRS bandwidth aggregation requirements - Proposals - Option 1: ZTE, OPPO - The conditions for PRS/SRS bandwidth aggregation agreed in RAN1 could be used as baseline for defining RRM requirements in RAN4. - Option 2: HW - Requirements for PRS CA are applicable provided that LMF requests UE to perform joint measurement across aggregated PFL. - Requirements for aggregate measurement are applicable to PRS resources in the resource sets that are indicated to be linked, provided that the alignment conditions defined by RAN1 are met.

Agreement For PRS bandwidth aggregation between PRS in two or three different PFLs, the following are needed for the aggregated PRS resources for a TRP: - The same periodicity and slot offset - The same muting pattern - The same NR-DL-PRS-SFN0-Offset value - UE expects to be configured with PRS resources that maintain a per-symbol uniformly spaced PRS pattern across aggregated bandwidths in frequency domain (Note: It does not preclude dropping some REs in the guardband between two PFLs). - FFS same antenna port from RAN1 perspective Agreement For PRS/SRS bandwidth aggregation between two or three different PFLs/carriers, send a reply LS to request RAN4 to capture the condition of ‘the same RF chain (same antenna)’ in RAN4 specification.

To apply PRS bandwidth aggregation, some conditions are proposed in the last meeting, such as the aggregated PRS/SRS resources should have the same numerology and be located in the same slots/symbols. It is noticed that these conditions, except ‘the same RF chain/antenna’, will be captured in RAN1. And RAN1 send a reply LS [3] to request RAN4 to capture the condition of ‘the same RF chain (same antenna)’ in RAN4 specification. We think that is sufficient for RRM requirements, and there is no need to discuss other conditions.

Proposal 2: Use the conditions for PRS/SRS bandwidth aggregation agreed in RAN1 as the baseline and additional capture ‘the same RF chain (same antenna)’ in RAN4.

Issue 3-2-2: Impact of number of PFLs on PRS/SRS bandwidth aggregation requirements Agreements: - The total measurement period for RSTD and UE Rx-Tx requirements with bandwidth aggregation across all PFLs is defined as - Txxxx,total = Taggregate + Tnon-aggregate - Taggregate is the total measurement period for aggregate measurements (i.e. measurements with bandwidth aggregation) across all PFLs - Taggregate is equal to zero if aggregate measurements are not configured. - Only PRS resources that are aggregated shall be counted in Taggregate - Tnon-aggregate is the total measurement period for non-aggregate measurements (i.e. measurements without bandwidth aggregation) across all PFLs - Tnon-aggregate is equal to the Rel-17 the measurement period requirement, with the modification that only PRS resources that are not aggregated are counted in Tnon-aggregate - Tnon-aggregate is equal to zero if non-aggregate measurements are not configured. - FFS whether additional margin shall be added for the alignment of aggregated and non-aggregated measurements. Agreements: - RAN4 requirements does not need to include the number of groups of PFLs. The number of PFL groups can be larger than 1. This is based on the assistance data received from the LMF. - Tmargin is needed. The value is FFS.

The total measurement period across all PFLs is defined as the sum of Taggregate and Tnon-aggregate. In addition, we think Tmargin = max(Teffect, aggregate, Teffect, non-aggregate) should be added. For non-aggregated PFLs, the measurement period is defined as the sum of each PFL where only the sampling and processing time are counted for the last sample of each PFL. There will be a time gap from the last PRS sample in PFL#1 to the first available PRS sample in PFL#2, and that’s the reason why Teffect is added when summing the measurement periods for multiple PFLs. Similarly, Tmargin is also needed when summing aggregated and non-aggregated parts, and max(Teffect, aggregate, Teffect, non-aggregate) could be considered as the starting point.

Proposal 3: Tmargin = max(Teffect, aggregate, Teffect, non-aggregate) should be added when both Taggregate and Tnon-aggregate are larger than 0.

Issue 3-2-3: Impact of PRS resource dropping on PRS/SRS bandwidth aggregation requirement Agreements: - When the PRS collision with other signals on PRS bandwidth aggregation requirement, the measurement period can be longer, and FFS whether RAN4 to define the UE behaviour in RAN4.

Agreement [RAN1 #114] For the case when PRS in one of aggregated PFL is dropped because of collision with other signals, for LMF based positioning, it is up to UE implementation to perform positioning measurement based on one or more of the PRS resources in the aggregated PFLs. - Note: it is up to RAN4 whether or not to define performance requirements for this case of collision with other signals Agreement [RAN1 #114] In RRC_CONNECTED state, for positioning SRS aggregation across CCs, if SRS in one of aggregated carriers is dropped in a symbol, stop SRS transmission in all aggregated carriers in the same symbol

When one of aggregated PRS resources is dropped due to the collision with other signals, it is up to UE implementation to perform positioning based on the other remaining PRS resources according to RAN1 agreements. The same UE behavior should be reused in RAN4. In the worst case, UE will skip measurement based on all the other PRS resources. Then the measurement period should be extended but the exact extension is not specified in RAN4.

Proposal 4: When one of aggregated PRS resources is dropped due to collision, it is up to UE implementation to perform measurement based on the remaining PRS resources and RAN4 will not specify the exact value of extended measurement period.

3	Conclusion

Based on latest progress in RAN1 and RAN4, this contribution gave our considerations on defining RRM requirements for PRS/SRS bandwidth aggregation and the following proposals:

Proposal 1: The scenario that aggregated SRS outside initial UL BWP should be considered when defining RRM requirements in RRC_INACTIVE state.

Proposal 2: Use the conditions for PRS/SRS bandwidth aggregation agreed in RAN1 as the baseline and additional capture ‘the same RF chain (same antenna)’ in RAN4.

Proposal 3: Tmargin = max(Teffect, aggregate, Teffect, non-aggregate) should be added when both Taggregate and Tnon-aggregate are larger than 0.

Proposal 4: When one of aggregated PRS resources is dropped due to collision, it is up to UE implementation to perform measurement based on the remaining PRS resources and RAN4 will not specify the exact value of extended measurement period.

4	Reference

- R4-2314353, WF on R18 NR positioning – RedCap positioning and PRS/SRS BW aggregation, Ericsson

- R4-2317207, Topic summary for [108bis][215] NR_pos_enh2_part1, Ericsson