3GPP TSG-RAN WG4 Meeting # 109							R4-2319493

Chicago, US, November 13 – 17, 2023

Agenda Item:	8.30.3.1

Source:	OPPO

Title:	On remaining RRM issues for SL CA

Document for:	Discussions

1	Introduction

In RAN4#108bis meeting, a WF was approved for SL-CA RRM and some issues were identified for further study in [1].

#### Issue 1-1: Constraints on interruptions to WAN at SL carrier addition/release due to paging and SIB reception - Option 1(Ericsson, LGE, OPPO, Nokia): Add the constraints on interruptions to WAN at SL carrier addition/release during WAN paging or system information reception - Option 2(Xiaomi, QC): Do not introduce constraints on interruptions to WAN at SL carrier addition/release during WAN paging or SIB reception. Agreement: FFS if interruptions to WAN at SL carrier addition/release is allowed during WAN paging or system information reception FFS whether the scenario is valid that SL carrier addition/release can collide with WAN paging or SIB reception in RRC_IDLE/RRC_INACTIVE mode and whether LS is needed. FFS if RLF and CBD should be protected. #### Issue 1-2: Other constraints on interruptions to WAN at SL carrier addition/release FFS: - Option 1(Ericsson): For SL mode 2, interruptions due to SL component carriers addition/release shall not apply while: - T310 timer is running for RLF on PCell - performing candidate beam detection on PCell/serving cell as specfied in section 8.5.5. and 8.5.6

In this contribution, we provide our further views on remaining issues of SL-CA RRM requirements.

2	Discussions

In LTE spec TS 36.133, RAN4 did not consider the constraints on any interruption requirements during WAN paging or SIB reception, including interruptions to WAN due to Sidelink Carrier Aggregation.

In NR, RAN4 agreed on interruption requirements to WAN carriers when NR SL UE performs SL carrier addition/release using the RRCConnectionReconfiguration message, and not to specify the exact interruptions location. However, whether constraints on interruptions during WAN paging or SIB reception shall be introduced still needs to be clarified.

As for Mode 2 UE, UE could be in idle/inactive or connected mode.

During RRC idle/inactive mode, it can happen that SL CA addition or release are colliding with receiving paging or SIB for UE. The carrier set for SL CA is configured by the SL UE in IDLE/INACTIVE mode, which can be included in pre-configuration or SIB12. It is up to the SL UE to decide when to add/release carriers based on configured carrier set. So, the interruption requirement is needed.

Pre-configuration:

sl-PreconfigFreqInfoList, sl-PreconfigFreqInfoListSizeExt This field indicates the NR sidelink communication and/ or NR sidelink discovery configuration some carrier frequency(ies). In this release, only one SL-FreqConfig can be configured in the list sl-PreconfigFreqInfoList.

SIB12:

sl-FreqInfoList, sl-FreqInfoListSizeExt This field indicates the NR sidelink communication/discovery configuration on some carrier frequency (ies). In this release, only one entry can be configured in thesl-FreqInfoList.

For this case, the similar constraints for interruption due to SL CA addition/release can also apply. Thus, we propose to introduce the applicability rule for interruptions to WAN due to SL CA.

Proposal 1: Introduce the applicability for interruptions to WAN due to Sidelink Carrier Aggregation during WAN paging or SIB reception.

Proposal 2: The proposed text can be considered for CR of interruption requirements:

When NR SL UE performs SL carrier addition/release using the RRCConnectionReconfiguration message, the interruptions in this clause shall not apply when one of the following conditions is met:

-	While receiving paging,

-	While receiving system information.

During RRC connected mode, network can avoid RRC (re)configuration for SL CA to collide with RLF and CBD. At least the interruption can be avoided based on network control. There seems no necessity to define constraints on interruption requirements during different procedures.

Proposal 3: Not consider constraint on interruptions to WAN at SL carrier addition/release during RLF and CBD procedure.

3	Summary

In this contribution we have discussed the remaining issues for supporting sidelink CA. Based on the discussion, we have made following proposals:

Proposal 1: Introduce the applicability for interruptions to WAN due to Sidelink Carrier Aggregation during WAN paging or SIB reception.

Proposal 2: The proposed text can be considered for CR of interruption requirements:

When NR SL UE performs SL carrier addition/release using the RRCConnectionReconfiguration message, the interruptions in this clause shall not apply when one of the following conditions is met:

-	While receiving paging,

-	While receiving system information.

Proposal 3: Not consider constraint on interruptions to WAN at SL carrier addition/release during RLF and CBD procedure.

References

R4-2317375, “WF on SL evolution RRM requirements – SL CA and co-channel co-existence”, OPPO