# 3GPP TSG-RAN WG4 Meeting # 109	R4-2319498

# Chicago, US, November 13 – 17, 2023

Agenda item:	11.3.4

Source: 	OPPO

Title: 	Discussion on left issues for MRTD-MTTD requirements in ENDC and NEDC

Document for:	Discussion

Introduction

This is to discuss the left issues related to reply LS on update for “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306 (R2-2309218).

Discussion

The signaling “asyncIntraBandENDC” was introduced for intra-band EN-DC, while it is further extended to cover inter-band EN-DC with overlapping frequency.

[…]

Further, the signaling “asyncIntraBandENDC” now only cover EN-DC UE reporting. RAN4 would like to ask RAN2 to update this signaling description to cover NE-DC reporting.

asyncIntraBandENDC

Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC.

interBandMRDC-WithOverlapDL-Bands-r16

Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE).

Currently, asyncIntraBandENDC is for FDD-FDD only, while the interBandMRDC-WithOverlapDL-Bands-r16 covers both FDD-FDD and TDD-TDD.

Based on previous agreements in RAN4, for inter-band EN-DC with overlapping DL bands or intra-band co-located EN-DC, asyncIntraBandENDC cannot apply for TDD-TDD case, where only sync operation is assumed. The same logic can apply for NE-DC. The requirements of TDD-TDD case are independent from whether UE indicates support of ‘asyncIntraBandENDC’.

We propose the updates for MTTD and MRTD requirements of TDD-TDD NE-DC based on the endorsed CR R4-2317400.

Thus, to align with current endorsed CR, the capability ‘asyncIntraBandENDC’ can only apply for cases including:

1) FDD- FDD intra-band co-located EN-DC,  and

2) FDD- FDD inter-band EN-DC/NE-DC with overlapping DL bands operating in co-located and non-co-located deployment.

Observation 1: The capability of asyncIntraBandENDC can be extended to FDD- FDD inter-band NE-DC with overlapping DL bands.

Observation 2: The capability ‘asyncIntraBandENDC’ can only apply for FDD-FDD intra-band co-located EN-DC and FDD-FDD inter-band EN-DC/NE-DC with overlapping DL bands operation.

About whether to extend the capability to NEDC for inter-band case only, or intra-band case as well, RAN4 needs to discuss. For intra-band co-located NE-DC,no requirements of MTTD/MRTD for intra-band NE-DC are defined. It may be subject to no such RF band combination of intra-band NE-DC. RAN4 can further comfirm whether to complete the requirement for intra-band NE-DC.

Proposal 1: RAN4 to discuss whether to define MTTD/MRTD requirements for intra-band NE-DC.

Proposal 2: RAN4 to discuss whether to extend the capability of asyncIntraBandENDC to NEDC for inter-band case only, or intra-band case as well.

In our view, the similar logic for TDD-TDD can be reused NE-DC. For intra-band co-located EN-DC or inter-band EN-DC/NE-DC with overlapping DL bands, only sync operation is assumed for TDD-TDD case. The capability ‘interBandMRDC-WithOverlapDL-Bands-r16’ can apply for FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands.

Observation 3: For intra-band co-located EN-DC or inter-band EN-DC/NE-DC with overlapping DL bands, only sync operation is assumed for TDD-TDD case.

Observatino 4: The capability ‘interBandMRDC-WithOverlapDL-Bands-r16’ can apply for FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands.

We propose to complete TDD-TDD case for NE-DC, like EN-DC.

Proposal 3: Complete the requirements of MTTD/MRTD for TDD-TDD inter-band sync NE-DC with overlapping DL bands .

Conclusion

In this contribution, we have the following observations:

Observation 1: The capability of asyncIntraBandENDC can be extended to FDD- FDD inter-band NE-DC with overlapping DL bands.

Observation 2: The capability ‘asyncIntraBandENDC’ can only apply for FDD-FDD intra-band co-located EN-DC and FDD-FDD inter-band EN-DC/NE-DC with overlapping DL bands operation.

Observation 3: For intra-band co-located EN-DC or inter-band EN-DC/NE-DC with overlapping DL bands, only sync operation is assumed for TDD-TDD case.

Observatino 4: The capability ‘interBandMRDC-WithOverlapDL-Bands-r16’ can apply for FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands.

We also have the following proposals:

Proposal 1: RAN4 to discuss whether to define MTTD/MRTD requirements for intra-band NE-DC.

Proposal 2: RAN4 to discuss whether to extend the capability of asyncIntraBandENDC to NEDC for inter-band case only, or intra-band case as well.

Proposal 3: Complete the requirements of MTTD/MRTD for TDD-TDD inter-band sync NE-DC with overlapping DL bands .

Reference

[1] R4-2317400. On MRTD/MTTD requirement for inter-band non-collocated EN-DC/NE-DC (R16), Apple

[2] R4-231xxxx, CR for MRTD/MTTD requirement for EN-DC/NE-DC (R16), OPPO