3GPP TSG-RAN WG4 Meeting # 109                                   R4-2319522

# Chicago, US, November 13 – November 17, 2023

Agenda item:	8.9.4

Source:	China Telecom

Title:	Discussion on RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG

Document for:	Discussion

- Introduction

In RAN4 #108bis meeting, RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG were discussed and the WF [1] was approved.

In this paper, we present our views about RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG issues captured in the WF [1].

- Discussion

Test cases for Case 1

Status in the WF [1]:

Issue 4-2-1: [Case 1] Which topics shall RAN4 RRM define test cases for Pre-MG and concurrent MG (Case 1) - Way forward - Option 1: - Pre-MG + Pre-MG, Pre-MG + Type-2 MG and Pre-MG + Type-1 MG. - Option 2: - Pre-MG + Pre-MG, Pre-MG + Type-2 MG. - Option 3: - Concurrent MG and 1 Pre-MG activation / deactivation in same FR - Pre-MGs with activation / deactivation in the same FR. - Option 4: - Pre-MG (de)activation - Pre-MG (de)activation is based on the associated MO or the associated pre-configured status - In non-simultaneous case, the (de)activation delay is same as in Rel-17 - In simultaneous case, the (de)activation delay is extended by 2ms - Collision handling - Deactivated pre-MG is not considered in collision - Dynamic collision including at least scenario 1 and 2

In the core part, Pre-MG + Pre-MG, and Pre-MG + Type-2 MG are mainly discussed, thus at least test cases for Pre-MG + Pre-MG, and Pre-MG + Type-2 MG shall be defined primarily.

Proposal 1: Test cases for Pre-MG + Pre-MG, and Pre-MG + Type-2 MG shall be defined primarily.

Status in the WF [1]:

Issue 4-2-2: [Case 1] Test cases combination for Case 1 - Way forward - Option 1: - RAN4 to define Pre-MG and Con-MGs test cases with the following combinations: - Pre-MG Combination 1: Pre-MG+MG without gap collision - Pre-MG Combination 2: Pre-MG+MG with gap collision - Pre-MG Combination 3: (Pre-MG1, Pre-MG2): (Deactivation, Deactivation) <-> (Activation, Activation) without gap collision - Pre-MG Combination 4: (Pre-MG1, Pre-MG2): (Deactivation, Deactivation) <-> (Activation, Activation) with gap collision - Pre-MG Combination 5: (Pre-MG1, Pre-MG2): (Activation, Deactivation) <-> (Deactivation, Activation) without gap collision - RAN4 not to define the test cases to verify the collision during the transition period between Pre-MG activation/deactivation and MG occasion - Option 2: - TC1: pre-MG + pre-MG, autonomous non-simultaneous (de)activation, FR1 - TC2: pre-MG + pre-MG, NW controlled non-simultaneous (de)activation, FR1 - TC3: pre-MG + pre-MG, autonomous simultaneous (de)activation, FR1 - TC4: pre-MG + pre-MG, NW controlled simultaneous (de)activation, FR1 - TC5: pre-MG + pre-MG, autonomous non-simultaneous (de)activation, FR2 - TC6: pre-MG + pre-MG, NW controlled non-simultaneous (de)activation, FR2 - TC7: pre-MG + pre-MG, autonomous simultaneous (de)activation, FR2 - TC8: pre-MG + pre-MG, NW controlled simultaneous (de)activation, FR2 - TC9: pre-MG + type-2 MG, dynamic collision scenario 1, autonomous (de)activation, FR1 - TC10: pre-MG + type-2 MG, dynamic collision scenario 1, NW controlled (de)activation, FR1 - TC11: pre-MG + type-2 MG, dynamic collision scenario 2 autonomous (de)activation, FR1 - TC12: pre-MG + type-2 MG, dynamic collision scenario 2, NW controlled (de)activation, FR1 - TC13: pre-MG + type-2 MG, dynamic collision scenario 1, autonomous (de)activation, FR2 - TC14: pre-MG + type-2 MG, dynamic collision scenario 1, NW controlled (de)activation, FR2 - TC15: pre-MG + type-2 MG, dynamic collision scenario 2 autonomous (de)activation, FR2 - TC16: pre-MG + type-2 MG, dynamic collision scenario 2, NW controlled (de)activation, FR2 - Option 3: - for gap combination, it is propsed to define test for the combination with 2 gaps for UE which does not support per-FR gap, and define test for the combination with 3 gaps cross all FRs for UE which supports per-FR gap - it is proposed to define test to verify the gap collision handling for concurrent gap with Pre-MG and concurrent gap with NCSG - it is proposed to define test for dynamic collision with Pre-MG - Option 4: - 1-1-1 Event triggered reporting test on intra-frequency in FR1 with concurrent gap and autonomous activation/deactivation of Pre-MG for - 1-1-2 Event triggered reporting test on intra-frequency in FR1 with concurrent gap and network-controlled activation/deactivation of Pre-MG for - 1-1-3 Event triggered reporting test on intra-frequency in FR2 with concurrent gap and autonomous activation/deactivation of Pre-MG - 1-1-3 Event triggered reporting test on intra-frequency in FR2 with concurrent gap and network-controlled activation/deactivation of Pre-MG

For the scenario of Pre-MG + Pre-MG, Pre-MGs activation/deactivation procedure are mainly discussed in the core part, it was agreed to define definitions for simultaneous and non-simultaneous multiple Pre-MGs activation/deactivation. Besides, the test case of autonomous activation/deactivation and network-controlled activation/deactivation were designed in Rel-17, FR1 and FR2 are also considered, we can follow the logic of Rel-17 test case design. Therefore, for Pre-MG + Pre-MG, it’s proposed to design test cases with consideration of simultaneous and non-simultaneous multiple Pre-MGs activation/deactivation, autonomous activation/deactivation and network-controlled activation/deactivation, FR1 and FR2.

Proposal 2: For Pre-MG + Pre-MG, it’s proposed to design test cases with consideration of simultaneous and non-simultaneous multiple Pre-MGs activation/deactivation, autonomous activation/deactivation and network-controlled activation/deactivation, FR1 and FR2.

For the scenario of Pre-MG + Type-2 MG, collision handling for dynamic collisions are mainly discussed in the core part, 4 scenarios are considered and the solutions in scenario 1/2/3 have been agreed in previous meetings. Considering the workload, in our views, test cases of collision handling for dynamic collisions in scenario 1 can be designed primarily, with consideration of autonomous activation/deactivation and network-controlled activation/deactivation, FR1 and FR2.

Proposal 3: For Pre-MG + Type-2 MG, test cases of collision handling for dynamic collisions in scenario 1 can be designed primarily, with consideration of autonomous activation/deactivation and network-controlled activation/deactivation, FR1 and FR2.

Test cases for Case 2

Status in the WF [1]:

Issue 4-3-1: [Case 2] Which topics shall RAN4 RRM define test cases for NCSG and concurrent MG (Case 2) - Way forward - Option 1: - NCSG + NCSG, NCSG + Type-2 MG and NCSG + Type-1 MG. - Option 2: - NCSG + NCSG, NCSG + Type-2 MG. - Option 3: - 1 Concurrent MG and 1 NCSG in the same FR - 2 NCSGs (non-overlapping) in the same FR - Measurement for deactivated SCells with 1 Concurrent MG (w/o gap or with NCSG, as being discussed for Rel-17 maintenance). - Option 4: - Collision definition - Total NCSG duration, including both the VILs and the ML are considered in the proximity - [Deactivated SCell measurement - Deactivated SCell is measured in NCSG regardless of the gap association]

In the core part, NCSG + NCSG, and NCSG + Type-2 MG are mainly discussed, thus at least test cases for NCSG + NCSG, and NCSG + Type-2 MG shall be defined primarily.

Proposal 4: Test cases for NCSG + NCSG, and NCSG + Type-2 MG shall be defined primarily.

Status in the WF [1]:

Issue 4-3-2: [Case 2] Test cases combination for Case 2 - Way forward - Option 1: - TC1: Measurement with NCSG on deactivated SCC without collision(NCSG+MG) - TC2: Measurement with NCSG on deactivated SCC with collision(NCSG+MG) - TC3: Inter-frequency measurement with NCSG without collision (NCSG+NCSG) - TC4: Inter-frequency measurement with NCSG with collision (NCSG+NCSG) - TC5: Intra-frequency measurement with NCSG without collision(NCSG+NCSG) - TC6: Intra-frequency measurement with NCSG with collision(NCSG+NCSG) - Option 2: - TC1: NCSG + type-2 MG, with collision, FR1 - TC2: NCSG + NCSG, with collision, FR1 - TC3: NCSG + type-2 MG, with collision, FR2 - TC4: NCSG + NCSG, with collision, FR2 - [TC5: NCSG + type-2 MG, for deactivated SCC measurement, FR1] - [TC6: NCSG + type-2 MG, for deactivated SCC measurement, FR2] - Option 3: - 1-1-1 Event triggered reporting test on intra-frequency in FR1 with concurrent gap and NCSG - 1-1-2 Event triggered reporting test on intra-frequency in FR2 with concurrent gap and NCSG - Option 4: - for gap combination, it is propsed to define test for the combination with 2 gaps for UE which does not support per-FR gap, and define test for the combination with 3 gaps cross all FRs for UE which supports per-FR gap - it is proposed to define test to verify the gap collision handling for concurrent gap with Pre-MG and concurrent gap with NCSG

For the scenario of NCSG + NCSG, test cases for collision between two NCSGs need to be designed in FR1 and FR2.

Proposal 5: For NCSG + NCSG, test cases for collision between two NCSGs need to be designed in FR1 and FR2.

For the scenario of NCSG + Type-2 MG, similarly, test cases for collision between NCSG and Type-2 MG need to be designed in FR1 and FR2. Besides, test cases for deactivated SCell measurement via NCSG need to be designed in FR1 and FR2.

Proposal 6: For NCSG + Type-2 MG, test cases for collision between NCSG and Type-2 MG need to be designed in FR1 and FR2. Besides, test cases for deactivated SCell measurement via NCSG need to be designed in FR1 and FR2.

- Conclusion

The contribution presents our views about RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG, with the following proposals:

Proposal 1: Test cases for Pre-MG + Pre-MG, and Pre-MG + Type-2 MG shall be defined primarily.

Proposal 2: For Pre-MG + Pre-MG, it’s proposed to design test cases with consideration of simultaneous and non-simultaneous multiple Pre-MGs activation/deactivation, autonomous activation/deactivation and network-controlled activation/deactivation, FR1 and FR2.

Proposal 3: For Pre-MG + Type-2 MG, test cases of collision handling for dynamic collisions in scenario 1 can be designed primarily, with consideration of autonomous activation/deactivation and network-controlled activation/deactivation, FR1 and FR2.

Proposal 4: Test cases for NCSG + NCSG, and NCSG + Type-2 MG shall be defined primarily.

Proposal 5: For NCSG + NCSG, test cases for collision between two NCSGs need to be designed in FR1 and FR2.

Proposal 6: For NCSG + Type-2 MG, test cases for collision between NCSG and Type-2 MG need to be designed in FR1 and FR2. Besides, test cases for deactivated SCell measurement via NCSG need to be designed in FR1 and FR2.

- References

- R4-2317305, WF on NR_MG_enh2_part1, MediaTek inc., RAN WG4 Meeting #108bis, October, 2023.