3GPP TSG-RAN WG4 Meeting # 109                                   R4-2319524

# Chicago, US, November 13 – November 17, 2023

Agenda item:	8.34.4

Source:	China Telecom

Title:	Discussion on RRM performance requirements for NES

Document for:	Discussion

- Introduction

In RAN4 #108bis meeting, RRM performance requirements for NES was discussed and the WF [1] was approved. In this paper, we present our views about RRM performance requirements for NR network energy saving.

- Discussion

Performance part related to SSB-less

Status in the WF [1]:

Issue 3-2-1: Test case for FR1 inter-band SSB-less activation/deactivation - Proposals - Option 1: Define test case for FR1 inter-band SSB-less activation. (Nokia, Huawei, Ericsson) - Option 1a: (Huawei) - RAN4 to define test case for FR1 inter-band SSB-less activation/deactivation, and the detailed condition (e.g. RTD/power difference) shall wait for further agreement in core discussion. - Discuss whether to define separate test cases for TRS based/A-TRS based FR1 inter-band SSB-less activation with more conclusion of core requirements. - Option 1b: (Ericsson) - RAN4 to define the following test cases for SSB-less SCell activation with RTD≤CP - TC1: SSB-less SCell Activation and deactivation of known SCell in FR1 in non-DRX

For performance part related to SSB-less, FR1 inter-band SSB-less activation is mainly discussed in core part, TRS/A-TRS is needed for SCell activation when RTD ≤ CP and received power difference is within [X] dB and X is larger than 6. Therefore, test case for FR1 inter-band SSB-less activation based on TRS/A-TRS in non-DRX need to be defined primarily.

Proposal 1: Test case for FR1 inter-band SSB-less activation based on TRS/A-TRS in non-DRX need to be defined primarily.

Whether requirement for FR1 inter-band SSB-less activation is based on TRS or A-TRS is still on discussion, if requirement based on TRS and requirement based on A-TRS are both defined, different test cases for TRS based and A-TRS based FR1 inter-band SSB-less activation can be defined.

Proposal 2: If requirement based on TRS and requirement based on A-TRS are both defined, different test cases for TRS based and A-TRS based FR1 inter-band SSB-less activation can be defined.

- Conclusion

The contribution presents our views on RRM performance requirements for NES, with the following proposals:

Proposal 1: Test case for FR1 inter-band SSB-less activation based on TRS/A-TRS in non-DRX need to be defined primarily.

Proposal 2: If requirement based on TRS and requirement based on A-TRS are both defined, different test cases for TRS based and A-TRS based FR1 inter-band SSB-less activation can be defined.

- References

- R4-2317405, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN WG4 Meeting #108bis, October 2023.