# 3GPP TSG-RAN WG4 Meeting # 109                            R4-2319530

# Chicago, US, November 13 – 17, 2023

# Source:        ZTE Corporation

# Title:           Simulation results for FR2-1 UL 256QAM

# Agenda Item:   8.6.4.1

# Document for:  Discussion

- Introduction

In RAN 97 meeting, a revised WID [1] for NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved, the objective for demodulation is:

UL 256QAM

  - Specify the BS demodulation performance

In RAN4 #108bis meeting, most of the simulation assumptions for defining demodulation requirements were reached[2] based on which we give our simulation results for reference.

- Simulation results

## In RAN4 # 108 meeting[3] Type B mapping is agreed for FR2-1 UL 256QAM demodulation requirement. TDLA and TDLD are the alternatives as channel model for demodulation requirement. The bandwidth and MCS need to be down selected.

## In RAN4 # 108-bis meeting[2] TDLD30-35 was agreed for demodulation requirements and EVM and phase noise are not considered for demodulation requirements. Both PTRS enabled and disabled need to be considered.

## The updated simulation assumptions are listed in table A-1 and the throughput curves are shown in table A-2.

Table 2-1 Simulation results of PUSCH demodulation for FR2-1 UL 256QAM

| Channel | SCS [kHz] /CBW [MHz] | DMRS Config | PTRS | SNR(dB) |
| --- | --- | --- | --- | --- |
| TDLD30-35Low | 60/50 | 1+1 | disabled | 15.9 |
|  |  |  | enabled | 16.8 |
|  |  | 1+0 | disabled | 15.6 |
|  |  |  | enabled | 16.6 |
|  | 120/50 | 1+1 | disabled | 16.2 |
|  |  |  | enabled | 17.1 |
|  |  | 1+0 | disabled | 15.9 |
|  |  |  | enabled | 16.9 |
|  | 120/100 | 1+1 | disabled | 16.3 |
|  |  |  | enabled | 17.0 |
|  |  | 1+0 | disabled | 16.0 |
|  |  |  | enabled | 16.8 |
| TDLD30-35Low | 120/200 | 1+1 | disabled | 16.3 |
|  |  |  | enabled | 17.0 |
|  |  | 1+0 | disabled | 15.9 |
|  |  |  | enabled | 16.9 |

- Reference

RP-222228 Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, Nokia, Xiaomi

R4-2316903 Way Forward 256 QAM BS Demod, Nokia, Nokia Shanghai Bell

R4-2313946 WF for NR_RF_FR2_req_Ph3_Demod, Nokia, Nokia Shanghai Bell

- Annex

Table A-1 Simulation assumption for PUSCH demodulation for 256QAM

| Parameter | FR2-1 |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| DMRS type | type 1 |
| Number of DMRS symbols | 1, 1+1 |
| symbols length | 10 |
| start symbol index | 0 |
| Time domain resource allocation type | type B |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 20 in table 5.1.3.1-2 in TS 38.214[3] (R=682.5/1024) |
| Carrier frequency (GHz) | 30 |
| Propagation condition | TDL-A/D 30ns delay spread, 35Hz Doppler frequency |
| SCS and BW | 60kHz: 50MHz 120kHz: 50, 100, 200MHz |
| PTRS | Disabled Enabled: KPT-RS =2, LPT-RS =1 |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Number of HARQ transmissions and RV sequence | 4, {0,2,3,1} |
| PN model for Tx and Rx | Tx: none Rx: none |
| Testing metric | SNR @70% of maximum throughput |

Table A-2 BS UL 256QAM PUSCH demodulation for FR2-1@39GHz TDLD MCS20 type B mapping