# 3GPP TSG-RAN WG4 Meeting # 109                                 R4-2319531

# Chicago, US, November 13 – 17, 2023

# Title: Discussion on NonCol_intraB_ENDC_NR_CA

# Source: 	ZTE Corporation

# Agenda item:	8.11.4

# Document for:	Approval

# 1 Introduction

A revised WI for intra-band non-collocated ENDC/NRCA was approved in RAN#100 meeting. In RAN4 # 108b the MCS and Rank parameters were down-selected as below[1].

Way forward: Rank/MCS selection Interested companies are encouraged to bring the PDSCH simulation results with impairments in RAN4#109 to decide the MCS index pair and corresponding SNR test point pair to define PDSCH NR-CA demodulation requirements. Rank; MCS table; MCS index. Carrier with lower power; Rank 1; MCS table 1; Option 1: MCS4 (QPSK, 0.30) Option 2: MCS5 (QPSK, 0.37). Carrier with higher power; Rank 2; MCS table 2; Option 1: MCS22 (256QAM, 0.74) Option 2: MCS23 (256QAM, 0.78) Option 3: MCS24 (256QAM, 0.82) Option 4: MCS25 (256QAM, 0.86).

In this contribution, we give our view points about the MCS selection for Rank 1 and Rank 2 based on our simulation results.

# 2	Discussion

In RAN4 # 108bis meeting most of simulation assumptions were reached and the MCS for lower power CC and MCS for higher power CC need further discussion.

Based on the simulation results which is provided in the companion contribution[], we list the SNR gap between higher power CC and lower power CC of different MCS combination in Table 2-1.

Table 2-1 SNR gap for two CC with different MCS combinations

| index | MCS of lower power CC(Rank1) | MCS of higher power CC(Rank2) | Target SNR of lower power CC (dB) | Target SNR of higher power CC(dB) | SNR gap |
| --- | --- | --- | --- | --- | --- |
| 1 | 4(table 1) | 22(table 2) | -3.7 | 20.2 | 23.9dB |
| 2 | 4(table 1) | 23(table 2) | -3.7 | 21.6 | 25.3dB |
| 3 | 4(table 1) | 24(table 2) | -3.7 | 22.4 | 26.1dB |
| 4 | 4(table 1) | 25(table 2) | -3.7 | 23.4 | 27.1dB |
| 5 | 5(table 1) | 22(table 2) | -2.7 | 20.2 | 23.0dB |
| 6 | 5(table 1) | 23(table 2) | -2.7 | 21.6 | 24.4dB |
| 7 | 5(table 1) | 24(table 2) | -2.7 | 22.4 | 25.2dB |
| 8 | 5(table 1) | 25(table 2) | -2.7 | 23.4 | 26.2dB |

From table 2-1 we can find that some SNR gaps are higher than 25dB. We propose the following MCS combinations as alternatives.

MCS4(table1) and MCS22(table2)

MCS5(table1) and MCS22(table2)

MCS5(table1) and MCS23(table2)

Proposal 1. To consider the following MCS combination as alternatives for demodulation requirement.

MCS4(table1) and MCS22(table2)

MCS5(table1) and MCS22(table2)

MCS5(table1) and MCS23(table2)

# 3 Conclusion

Proposal 1. To consider the following MCS combination as alternatives for demodulation requirement.

MCS4(table1) and MCS22(table2)

MCS5(table1) and MCS22(table2)

MCS5(table1) and MCS23(table2)

# 4	Reference

[1]  R4-2316917, WF on NonCol_intraB_ENDC_NR_CA_Demod, Ericsson