# 3GPP TSG-RAN WG4 Meeting # 109                                 R4-2319532

# Chicago, US, November 13 – 17, 2023

# Title: Simulation results for intra-band non-collocated NR-CA

# Source: 	ZTE Corporation

# Agenda item:	8.11.4

# Document for:	Approval

# 1 Introduction

A revised WI for intra-band non-collocated ENDC/NRCA was approved in RAN#100 meeting. In RAN4 # 108b the MCS and Rank parameters were down-selected as below[1].

Way forward: Rank/MCS selection Interested companies are encouraged to bring the PDSCH simulation results with impairments in RAN4#109 to decide the MCS index pair and corresponding SNR test point pair to define PDSCH NR-CA demodulation requirements. Rank; MCS table; MCS index. Carrier with lower power; Rank 1; MCS table 1; Option 1: MCS4 (QPSK, 0.30) Option 2: MCS5 (QPSK, 0.37). Carrier with higher power; Rank 2; MCS table 2; Option 1: MCS22 (256QAM, 0.74) Option 2: MCS23 (256QAM, 0.78) Option 3: MCS24 (256QAM, 0.82) Option 4: MCS25 (256QAM, 0.86).

In this contribution, we give our simulation results for reference.

# 2	Discussion

In RAN4 # 108bis meeting most of simulation assumptions were reached and the MCS for lower power CC and MCS for higher power CC need further discussion.

Table 2-1 and Table 2-2 show the target SNR for 30kHz and 15kHz SCS with different MCS and rank. The simulation assumption and normalized throughput curves are appended in the annex.

Table 2-1 Target SNR with 30kHz SCS for different MCS and rank

| index | MCS | layer | Targer SNR (dB) |
| --- | --- | --- | --- |
| 1 | 4(table 1) | 1 | -3.7 |
| 2 | 5(table 1) | 1 | -2.7 |
| 3 | 22(table 2) | 2 | 20.2 |
| 4 | 23(table 2) | 2 | 21.6 |
| 5 | 24(table 2) | 2 | 22.4 |
| 6 | 25(table 2) | 2 | 23.4 |

Table 2-2 Target SNR with 15kHz SCS for different MCS and rank

| index | MCS | layer | Targer SNR (dB) |
| --- | --- | --- | --- |
| 1 | 4(table 1) | 1 | -3.7 |
| 2 | 5(table 1) | 1 | -2.6 |
| 3 | 22(table 2) | 2 | 20.4 |
| 4 | 23(table 2) | 2 | 21.2 |
| 5 | 24(table 2) | 2 | 22.6 |
| 6 | 25(table 2) | 2 | 23.4 |

# 4	Reference

[1]  R4-2316917, WF on NonCol_intraB_ENDC_NR_CA_Demod, Ericsson

- Annex

Table A-1 simulation parameter assumption

| Parameter | value |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | Static channel model defined in B.1.1 in TS 38.101-4 |
| Maximum number of HARQ transmission | 1 |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Tx antenna configuration and rank | 2Tx2Rx Rank1 for carrier with lower power Rank2 for carrier with higher power |
| MCS table and MCS index | MCS4, MCS5 (table 1) MCS22, MCS23, MCS24, MCS25(table2) |
| Test metric | 70% of the maximum throughput |

Table A-2 Normalized throughput curves for 30kHz SCS with different MCS

Table A-3 Normalized throughput curves for 15kHz SCS with different MCS