# 3GPP TSG-RAN WG4 Meeting # 109                                   R4-2319536

# Chicago, USA, Nov 13 – Nov 17, 2023

# Title: 	Discussion on NCR-MT demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	8.28.6

# Document for:	Approval

# 1 Introduction

During the last RAN4 meeting[1], some conclusions has been reached for NCR-MT demodulation requirements. In this contribution, we want to share some simulation results for NCR-MT which include PDSCH, PDCCH requirements and FRC tables.

# 2 Discussion

- Sub-topic 1-1 Others

Issue 1-3-1: Which FRC can be reused for NCR-MT - Agreement: - FRCs need to be based on IAB-MT style testing. - For test cases in which the corresponding FRCs are available in IAB-MT specification, reuse the FRC from IAB-MT specification. - For the remaining test cases, companies are encouraged to provide FRCs.

As we discussed before last meeting, we provide the detailed FRCs in following tables.

## 2.1 PDSCH requirements

According to previous meeting WF [2], PDSCH requirements as shown in following table for FR1 and FR2.

### 2.1.1 FR1

Table 2-1. FR1 PDSCH performance requirements

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |

Table 2-2. FRC parameters for FR1 PDSCH performance requirements

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | Case 1/2 | Case 3/4 | Case 5/6 | Case 7/8 |
| Channel bandwidth | MHz | 10 | 10 | 40 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 30 | 30 |
| Allocated resource blocks | PRBs | 52 | 52 | 106 | 106 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 13 | 4 | 13 |
| Modulation |  | QPSK | 16QAM | QPSK | 16QAM |
| Target Coding Rate |  | 0.3 | 0.48 | 0.3 | 0.48 |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  | 4096 | 13064 | 8456 | 26632 |
| Transport block CRC per Slot |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  | 1 | 2 | 2 | 4 |
| Binary Channel Bits Per Slot |  | 13728 | 27456 | 27984 | 55968 |

### 2.1.2 FR2

Table 2-3. FR1 PDSCH performance requirements

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | 70% max throughput 1% BLER |

Table 2-4. FRC parameters for FR2 PDSCH performance requirements, QPSK

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | Case 1 |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 13 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.3 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  | 5504 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 18282 |

## 2.2 PDCCH requirements

### 2.2.1 FR1

Table 2-5. FR1 PDCCH performance requirements

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x2 Low | Available in 38.101-4 |
| 2 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x4 Low | Available in 38.101-4 |
| 3 | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 Low | Available in 38.101-4 |
| 4 | 10 | 48 | 2 | 4 | TDLA30-10 | 1x4 Low | Available in 38.101-4 |
| 5 | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 Low | Available in 38.101-4 |
| 6 | 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low | -7.3 |
| 7 | 40 | 102 | 1 | 2 | TDLA30-10 | 1x2 Low | Available in 38.174 |
| 8 | 40 | 102 | 1 | 2 | TDLA30-10 | 1x4 Low | Available in 38.174 |
| 9 | 40 | 102 | 1 | 4 | TDLA30-10 | 1x2 Low | Available in 38.174 |
| 10 | 40 | 102 | 1 | 4 | TDLA30-10 | 1x4 Low | Available in 38.174 |
| 11 | 40 | 90 | 1 | 8 | TDLA30-10 | 2x2 Low | Available in 38.174 |
| 12 | 40 | 90 | 1 | 8 | TDLA30-10 | 2x4 Low | Available in 38.174 |

Table 2-6. FR1 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | Test 1/2 | Test 3/4 | Test 5/6 | Test 7/8 | Test 9/10 | Test 11/12 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 24 | 48 | 48 | 102 | 102 | 102 |
| CORESET time domain allocation |  | 2 | 2 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 39 | 52 | 52 | 41 | 53 | 53 |

### 2.2.2 FR2

Table 2-7. FR1 PDCCH performance requirements

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | 60 | 1 | 2 | TDLA30-75 | 1x2 Low | Available in 38.174 |
| 2 | 100 | 60 | 1 | 4 | TDLA30-75 | 1x2 Low | Available in 38.174 |
| 3 | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 Low | Available in 38.174 |

Table 2-8. FR2 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | M-FR2-A.3.4-1 | M-FR2-A.3.4-2 | M-FR2-A.3.4-3 |
| Subcarrier spacing | kHz | 120 | 120 | 120 |
| CORESET frequency domain allocation |  | 60 | 60 | 60 |
| CORESET time domain allocation |  | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 40 | 56 | 56 |

## 2.3 CQI requirements

### 2.3.1 FR1

Table 2-9. CQI reporting definition test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |  | Test 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |  | 10 |  |  |  |  |
| Duplex Mode |  |  |  | FDD |  |  |  | FDD |  |  |  |  |
| Subcarrier spacing |  |  | kHz | 15 |  |  |  | 15 |  |  |  |  |
| SNR |  |  | dB | 8 | 9 | 14 | 15 | 5 | 6 | 11 |  | 12 |
| Propagation channel |  |  |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  | 2×4 with static channel specified in Annex B.1 |  |  |  |  |
| Beamforming Model |  |  |  |  |  |  |  |  |  |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  | Periodic |  |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  | 2 |  |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  | FD-CDM2 |  |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  | 1 |  |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) |  |  |  | Row 3,(6) |  |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  | 13 |  |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  | 5/1 |  |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  | Periodic |  |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  | Table 2 |  |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  | cri-RI-PMI-CQI |  |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  | Not configured |  |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  | Not configured |  |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  | Wideband |  |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  | Wideband |  |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  | 8 |  |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  | 1111111 |  |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |  |  | 5/0 |  |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  | Not configured |  |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  | typeI-SinglePanel |  |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  | 1 |  |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  | Not configured |  |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  | 010000 |  |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  | N/A |  |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  | PUCCH |  |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |  |  | 8 |  |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  | 1 |  |  |  |  |
| Measurement channel |  |  |  | TBD |  |  |  | TBD |  |  |  |  |

Table 2-10. CQI reporting definition test

| Parameter |  | Unit | Test 5 |  |  | Test 6 |  | Test 7 |  | Test 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |  |  |  | 40 |  |  |  |
| Subcarrier spacing |  | kHz | 30 |  |  |  |  | 30 |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |
| SNR |  | dB | 8 | 9 | 14 |  | 15 | 5 | 6 | 11 | 12 |
| Propagation channel |  |  | AWGN |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 2x2 |  |  |  |  | 2x4 |  |  |  |
| Beamforming Model |  |  | TBD |  |  |  |  | TBD |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 2 |  |  |  |  | 2 |  |  |  |
|  | CDM Type |  | FD-CDM2 |  |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 3,(6,-) |  |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |  |  |  |  | 10/1 |  |  |  |
| ReportConfigType |  |  | Periodic |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  | Table 2 |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  | RB | 16 |  |  |  |  | 16 |  |  |  |
| Csi-ReportingBand |  |  | 1111111 |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  | slot | 10/9 |  |  |  |  | 10/9 |  |  |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |  |  |  | typeI-SinglePanel |  |  |  |
|  | Codebook Mode |  | 1 |  |  |  |  | 1 |  |  |  |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | Not configured |  |  |  |  | Not configured |  |  |  |
|  | CodebookSubsetRestriction |  | 010000 |  |  |  |  | 010000 |  |  |  |
|  | RI Restriction |  | N/A |  |  |  |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | 9.5 |  |  |  |  | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  | 1 |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  | TBD |  |  |  |  | TBD |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 [28] are left up to test implementation, if transmitted or needed. |  |  |  |  |  |  |  |  |  |  |  |

Table 2-10. Fixed Reference Channels for FR1 CSI reporting

| Reference channel |  |  |  | Test 1/2/3/4 | Test 5/6/7/8 |
| --- | --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 6240 | 12720 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |
| 0 | OOR | OOR | OOR |  | N/A |
| 1 | 0.1523 | 0 | QPSK | 1480 | 2976 |
| 2 | 0.3770 | 1 |  | 2408 | 4744 |
| 3 | 0.8770 | 3 |  | 5504 | 11016 |
| 4 | 1.4766 | 5 | 16QAM | 9224 | 18960 |
| 5 | 1.9141 | 7 |  | 12040 | 24576 |
| 6 | 2.4063 | 9 |  | 15112 | 30728 |
| 7 | 2.7305 | 11 | 64QAM | 16896 | 34816 |
| 8 | 3.3223 | 13 |  | 20496 | 42016 |
| 9 | 3.9023 | 15 |  | 24576 | 49176 |
| 10 | 4.5234 | 17 |  | 28168 | 57376 |
| 11 | 5.1152 | 19 |  | 31752 | 65576 |
| 12 | 5.5547 | 21 | 256QAM | 34816 | 69672 |
| 13 | 6.2266 | 23 |  | 38936 | 79896 |
| 14 | 6.9141 | 25 |  | 43032 | 88064 |
| 15 | 7.4063 | 27 |  | 46104 | 94248 |

### 2.3.2 FR2

Table 2-11. CQI reporting definition test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 3D1S1U |  |  |  |
| Special Slot Configuration |  |  |  | 10D+2G+2U |  |  |  |
| SNRBB |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex I.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex I.3.1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/4 |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 1.75 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | M-FR2-A.3.5-2 |  |  |  |
| Note 1: The same requirements are applicable to with different UL-DL patterns. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 [28] are left up to test implementation, if transmitted or needed. Note 3: If the IAB-MT reports in an available uplink reporting instance at slot #n based on CQI estimation at a downlink slot not later than slot#(n-4), this reported CQI cannot be applied at the gNB downlink before slot#(n+4). |  |  |  |  |  |  |  |

Table 2-12. Fixed Reference Channels for FR2 CSI reporting

| Reference channel |  |  |  | Test 1/2 |
| --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 66 |
| Number of consecutive PDSCH symbols |  |  |  | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 |
| Overhead for TBS determination |  |  |  | 6 |
| Available RE-s |  |  |  | 7590 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |
| 0 | OOR | OOR | OOR | N/A |
| 1 | 0.1523 | 0 | QPSK | 1800 |
| 2 | 0.2344 | 0 |  | 1800 |
| 3 | 0.3770 | 2 |  | 2856 |
| 4 | 0.6016 | 4 |  | 4480 |
| 5 | 0.8770 | 6 |  | 6528 |
| 6 | 1.1758 | 8 |  | 8712 |
| 7 | 1.4766 | 11 | 16QAM | 11016 |
| 8 | 1.9141 | 13 |  | 14343 |
| 9 | 2.4063 | 15 |  | 17928 |
| 10 | 2.7305 | 18 | 64QAM | 20496 |
| 11 | 3.3223 | 20 |  | 25104 |
| 12 | 3.9023 | 22 |  | 29192 |
| 13 | 4.5234 | 24 |  | 33816 |
| 14 | 5.1152 | 26 |  | 38936 |
| 15 | 5.5547 | 28 |  | 42016 |
| NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |

Proposal 1. Propose to consider all the above tables as NCR-MT demodulation requirements FRCs .

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for NCR-MT demodulation requirements , The conclusions are:

Proposal 1. Propose to consider all the above tables as NCR-MT demodulation requirements FRCs .

# 4 References

- R4-2316923 WF for NCR-MT demodulation requirements. ZTE Corporation.

- R4-2313923 WF for NCR-MT demodulation requirements. ZTE Corporation.