# 3GPP TSG-RAN WG4 Meeting # 109                                   R4-2319544

# Chicago, USA, Nov 13 – Nov 17, 2023

# Title: 	Simulation results for BS demodulation performance for less than 5MHz

# Source: 	ZTE Corporation

# Agenda item:	8.14.6.2

# Document for:	Discussion

# 1 Introduction

This contribution contains initial simulation results for the <5MHz WID [1]. The results are provided to aid in determining where new requirements can/should be introduced.

# 2 Discussion

## 2.1 PUSCH requirements

Based on previous meeting’s WF [2], we evaluated the PUSCH performance between 25PRB and 12PRB.

Table 1. PUSCH simulation results for 25PRB and 12PRB

| Test num. | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  |  |  |  |  | 25PRB | 12PRB |
| 1 | QPSK, 0.30 | TDLB100-400 | 1x2, ULA Low | 70 | -5.5 | -5.1 |
| 2 | 16QAM,0.48 | TDLC300-100 | 1x2, ULA Low | 70 | 7.7 | 7.2 |
| 3 | 64QAM, 0.55 | TDLA30-10 | 1x2, ULA Low | 70 | 9.1 | 9.2 |
| 4 | 256QAM, 0.67 | TDLA30-10 | 1x2, ULA Low | 70 | 15.2 | 15.1 |

Observation 1. There is no significant performance difference between 25PRB and 12PRB for PUSCH.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for less than 5MHz demodulation requirements, The conclusions are:

Observation 1. There is no significant performance difference between 25PRB and 12PRB for PUSCH.

# 4 References

- RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, RAN#95e.

- R4-2306924, WF on NR_FR1_lessthan_5MHz_BW_demod, Nokia, Nokia Shanghai Bell.