# 3GPP TSG-RAN WG4 Meeting # 109                                   R4-2319545

# Chicago, USA, Nov 13 – Nov 17, 2023

# Title: 	Discussion on NR DSS demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	8.36.2

# Document for:	Approval

# 1 Introduction

In RAN 96e meeting, a revised WID on Enhancement of NR Dynamic spectrum sharing (DSS) was approved [1] for R-18.

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2] Specify, if any, necessary UE performance requirements for above core objectives: - Specify necessary UE demodulation performance requirements (RAN4)

Also, during the last RAN4 meeting[2], some conclusions has been reached for DSS scenario. In this contribution, we want to share analysis on demodulation requirement aspects from RAN4 perspective based on the listed feature from RAN1 perspective.

# 2 Discussion

## 2.1 Scenarios for DSS

Regarding UE capabilities for eDSS enhancement, three features were defined in RAN1 perspective, which are 52-1, 52-1a and 52-1b, As shown in following table.

Table 1. UE features for NR PDCCH reception in symbols with LTE CRS REs

|  | 52-1 | 52-1a | 52-1b |
| --- | --- | --- | --- |
| Feature group | Reception of NR PDCCH candidates overlapping with LTE CRS REs | Reception of NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns | NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot |
| Components | Reception of NR PDCCH candidates that overlap with LTE CRS REs within a NR carrier using 15 kHz SCS 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM pattern by configuration of one CRS rate matching pattern via lte-CRS-ToMatchAround 2) Reception of a NR PDCCH candidate in REs that overlap with LTE CRS: candidate value set {a) when at least one symbol of the NR PDCCH candidate is not overlapped with LTE CRS, b) when some or all of symbols of NR PDCCH candidate overlap with LTE CRS} 3) Reception of NR PDCCH candidates that overlap with LTE CRS REs on the X-th symbols of an NR slot. Candidate values for X: {only 2nd symbol, 1st and 2nd symbols} 4) NR PDCCH that overlaps with LTE CRS REs is in Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or USS that are monitored within the first 3 OFDM symbols of a slot | 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM patterns by configuration of one or multiple non-overlapping CRS rate matching patterns via lte-CRS-PatternList1-r16 if the UE supports FG 14-1 or lte-CRS-PatternList3-r18 if the UE supports FG 52-2 | 1) NR PDCCH that overlaps with LTE CRS REs is in Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or USS that are monitored within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot |
| prerequisite feature groups | 5-28 | 52-1, at least one of {14-1, 52-2} | 52-1, 22-12 |
| Note | For component 2, RAN1 considers support value b) in component 2 only if RAN4 performance requirements for value b) are not defined |  |  |

And in last meeting, we discussed the typical scenario for eDSS based on RAN1 features. In general, from RAN4 perspective, we can define demodulation requirements for 5-21 at least.

As for FG52-1, we noticed that made a illustration for PDCCH candidate symbol is overlapped with LTE CRS in component 2. As was mentioned above, RAN1 considers support value b) in component 2 only if RAN4 performance requirements for value b) are not defined. In other words, we just need to define requirements for value a) when at least one symbol of the NR PDCCH candidate is not overlapped with LTE CRS, as shown in following Figure 1.

Figure 1. Typical scenarios for NR dynamic spectrum sharing

- Scenario :

  - On the OFDM symbols (i.e. symbol#1, symbol#2) colliding with LTE-CRS, only one PDCCH DMRS RE and four DMRS RE colliding with LTE-CRS, additional PDCCH symbol#2 not colliding with LTE-RS.

  - 4 CRS ports for LTE

  - Symbol #0: LTE PDCCH/PCFICH

  - Symbol #1: NR PDCCH overlapping with LTE CRS

  - Symbol #2: NR PDCCH

Thus, above detailed assumptions could be considered for DSS scenario.

Proposal 1. Considering the following detailed assumptions for DSS scenario.

- 2-symbol CORESET

- Basic assumptions

  - Rate matching pattern

    - Single non-overlapping CRS rate matching patterns for PDCCH demodulation.

  - 4 CRS ports for LTE

  - Symbol #0: LTE PDCCH/PCFICH

  - Symbol #1: NR PDCCH overlapping with LTE CRS

  - Symbol #2: NR PDCCH

## 2.2 Simulation assumption

And for simulation assumptions for DSS scenario, we think aggregation level 2 could be considered at least. Indeed this is a worst case for test cases. Also, it’s better to consider other aggregation levels, i.e. aggregation level 4 and 8.

Proposal 2. Considering aggregation level 2, 4, 8 for DSS requirements.

Table 1. Simulation assumptions for DSS scenario

| - Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLC300-100 |
| Antenna configuration | 1x2, 1x4; 2x2, 2x4 |
| CRS | 4 port CRS without power boosted |
| DCI payload (excluding CRC) | 52 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 REGs |

Table 2. minimum performance for DSS scenario

| Test number | Bandwidth (MHz) | Symbols with PDCCH | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 2 | 2 | TDLA30-10 | 1x2, 1x4 Low | 1 |  |
| 2 | 10 | 2 | 4 | TDLA30-10 | 1x2, 1x4 Low | 1 |  |
| 3 | 10 | 2 | 8 | TDLC300-100 | 2x2, 2x4 Low | 1 |  |

Proposal 3. Considering above table 1 and table 2 as simulation assumptions and minimum performance requirements for DSS scenario.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for DSS demodulation requirements , The conclusions are:

Proposal 1. Considering the following detailed assumptions for DSS scenario.

- 2-symbol CORESET

- Basic assumptions

  - Rate matching pattern

    - Single non-overlapping CRS rate matching patterns for PDCCH demodulation.

  - 4 CRS ports for LTE

  - Symbol #0: LTE PDCCH/PCFICH

  - Symbol #1: NR PDCCH overlapping with LTE CRS

  - Symbol #2: NR PDCCH

Proposal 2. Considering aggregation level 2, 4, 8 for DSS requirements.

Proposal 3. Considering above table 1 and table 2 as simulation assumptions and minimum performance requirements for DSS scenario.

# 4 References

- RP-221622, Revised WID on Enhancement of NR Dynamic spectrum sharing (DSS), RAN#96e.

- R4-2317000. WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements, Ericsson