# 3GPP TSG-RAN WG4 Meeting #109                                  R4-2319546

# Chicago, USA, Nov 13 – Nov 17, 2023

# Title: 	Discussion on ATG performance general aspects

# Source: 	ZTE Corporation

# Agenda item:	8.13.8.1

# Document for:	Approval

# 1 Introduction

- In RAN4#108bis meeting, ATG demodulation requirements were discussed and a WF was agreed[1]. In this paper, we will provide the detailed discussions on ATG general issue.

# 2 Discussion

TDD pattern

One FFS issue of how to introduce new TDD pattern for ATG PUSCH requirements in last meeting are listed in following:

Agreement: - For new incremental ATG PDSCH, introduce test cases of new TDD pattern configuration 30D4S6U. - PDSCH requirements are optional based on UE capability. - FFS on how to introduce new TDD pattern configuration 30D4S6U in ATG PUSCH requirements.

Regarding the TDD pattern, even if same requirements are applicable to FDD and TDD with different UL-DL pattern in legacy pattern. But for ATG scenario, because different frequency will have different Doppler shift. So the legacy rule is not applicable anymore. Thus, we need to consider how to introduce new TDD pattern in ATG PUSCH requirement. In our contribution [2], we gave a practicable alternative about how to introduce new TDD pattern in ATG PUSCH requirements, as shown in following table :

Table: 8.2.14.4.2-1 Test parameters for testing testing PUSCH under ATG conditions (BS type 1-O)

| Parameter |  | FDD | TDD |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Default TDD UL-DL pattern (Note 1) |  |  | 30 KHz SCS: 7D1S2U, S=6D:4G:4U 30D4S6U,S= 14G |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | {0} |  |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |  |
| Time | PUSCH mapping type | A |  |
| domain | Start symbol | 0 |  |
| resource assignment | Allocation length | 14 |  |
| Frequency | RB assignment | Full applicable test bandwidth |  |
| domain resource assignment | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| Note 1: The same requirements are applicable to TDD with different UL-DL pattern |  |  |  |

Proposal 1. For TDD pattern, propose to considering following practicable alternative to introduce new TDD pattern .

| Parameter |  | FDD | TDD |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Default TDD UL-DL pattern (Note 1) |  |  | 30 KHz SCS: 7D1S2U, S=6D:4G:4U 30D4S6U,S= 14G |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | {0} |  |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |  |
| Time | PUSCH mapping type | A |  |
| domain | Start symbol | 0 |  |
| resource assignment | Allocation length | 14 |  |
| Frequency | RB assignment | Full applicable test bandwidth |  |
| domain resource assignment | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| Note 1: The same requirements are applicable to TDD with different UL-DL pattern |  |  |  |

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for 8Rx , The conclusions are:

Proposal 1. For TDD pattern, propose to considering following practicable alternative to introduce new TDD pattern .

| Parameter |  | FDD | TDD |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Default TDD UL-DL pattern (Note 1) |  |  | 30 KHz SCS: 7D1S2U, S=6D:4G:4U 30D4S6U,S= 14G |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | {0} |  |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |  |
| Time | PUSCH mapping type | A |  |
| domain | Start symbol | 0 |  |
| resource assignment | Allocation length | 14 |  |
| Frequency | RB assignment | Full applicable test bandwidth |  |
| domain resource assignment | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| Note 1: The same requirements are applicable to TDD with different UL-DL pattern |  |  |  |

# 4 References

- R4-2316991, WF on NR_ATG_Demod, RAN4#108bis meeting.

- R4-231xxxx, Draft CR to TS38.141-2: Introduction of PUSCH requirements and FRCs for ATG performance requirements. ZTE Corporation.