# 3GPP TSG-RAN WG4 Meeting # 109                               R4-2319547

# Chicago, US, November 13 – 17, 2023

# Title: 	Discussion on ATG UE demodulation

# Source: 	ZTE Corporation

# Agenda item:	8.13.8.2

# Document for:	Approval

# 1 Introduction

In RAN#99e meeting[1], the revised work item on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package.

This contribution shares our views for the open issue for ATG DL demodulation requirements.

# 2	Discussion

During RAN4 # 108bis meeting, some conclusions which need further discussion for ATG DL demodulation are listed below[2].

Issue 2-1-2: Test scope for PDCCH - Agreement: - To consider legacy PDCCH requirements for ATG PDCCH requirements to cover all AL. - The following test cases are for down-selection: (if necessary) - 1T2R FDD: Test number 1, 3 and 5 in 5.3.2.1.1 - 2T2R FDD: Test number 3 in 5.3.2.1.2 - 1T2R TDD: All test cases in 5.3.2.2.1 - 2T2R TDD: All test cases in 5.3.2.2.2 - 1T4R FDD: Test number 1, 3 and 5 in 5.3.3.1.1 - 2T4R FDD: Test number 3 in 5.3.3.1.2 - 1T4R TDD: All test cases in 5.3.3.2.1 - 2T4R TDD: All test cases in 5.3.3.2.2

By checking the specification, test cases for 1Tx include the aggregation level of 2, 4 and 16, test case for 2Tx include the aggregation level of 4 and 8. In order to cover both TDD/FDD, 2Rx/4Rx, aggregation level 2/4/8/16, the above test cases need to be considered for ATG PDCCH requirements.

Proposal 1. Not to down-select test cases reached in RAN4 # 108bis meeting for ATG PDCCH requirements.

# 3 Conclusion

In this contribution, we give some conclusions for ATG UE demodulation. The conclusion is:

Proposal 1. Not to down-select test cases reached in RAN4 # 108bis meeting for ATG PDCCH requirements.

# 4	Reference

[1]  RP-230279, Revised WID on Air-to-ground network for NR, CMCC

[2]  R4-2316991, WF on NR_ATG_Demod, CMCC