3GPP TSG-RAN WG4 Meeting # 109														R4-2319569

Chicago, USA, November 13 –  November 17, 2023

Source:				Ericsson

Title:					NTN enhancements – System parameters updates

Agenda item:		8.26.1.1

Document for:		Approval

# Introduction

This document discusses some updates needed to the previously agreed system parameters, this to properly specify the NTN Ka-band.

# Discussion

## Background and issue

In RAN4#107 meeting, RAN4 agreed ([2]) on the following NR-ARFCN values for NTN bands n510, n511 and n512:

Table 1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |

Nevertheless, in last RAN4#108-bis meeting, a draft CR ([3]) on system parameters for SAN was endorsed specifying the following GSCN ranges:

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

Comparing those 2 tables, it looks obvious that the NR-ARFCN needs to be extanded to cover the uplink frequency ranges of FR2-NTN bands n510, ,511 and n512.

## Proposal

The following proposal will address the above identified issue:

Proposal: Extend the NR-ARFCN parameters table with the following range:

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279165 |

Our companion draft CR ([4]) captures the proposed update, based on the draft CR ([3]) endorsed in last RAN4#108-bis.

# Conclusion

In this contribution, we identified an issue with the currently specified system parameters for the NTN Ka-bands and make the following proposal:

Proposal: Extend the NR-ARFCN parameters table with the following range:

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279165 |

# Reference

- RP-230809, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2314934, WF for system parameters on Ka band, Thales

- R4-2316888, Draft CR on TS 38.108 for Clause 5 - Operating bands and channel arrangement, Thales

- R4-2319570, NTN enhancement: draft CR to TS 38.108 NTN Ka-band - system parameters udpate, Ericsson