3GPP TSG-RAN WG4 Meeting # 109						R4-2319581

Chicago, USA, November 13 –  November 17, 2023

Source:			Ericsson

Title:			Spectrum less than 5 MHz – BS RF conformance considerations

Agenda item:		8.14.3

Document for:		Approval

# Introduction

The Work Item to add NR support for dedicated spectrum less than 5 MHz for FR1 was revised ([1]) in RAN#99. This WI introduces new channel bandwidth(s) narrower than 5 MHz.

In last RAN4#108-bis meeting, RAN4 endorsed draft CRs to TS 38.141-1.

Based on further analysis of  LTE and MSR test configurations and test signals used in those TCs, this contribution is further discussing the test signal to be used in NR test configurations after 3 MHz channel bandwidth has been specified.

# Discussion

## LTE test configurations and test signal

The following Table 1 lists the different test configurations specified in TS 36.141 and the signals used to build those test configurations.

Table 1: TS 36.141 - Test configurations and test signals

| Test Configuration | Usage | Test signal at the lower edge | Test signal at the upper edge | Test signal inside the block |
| --- | --- | --- | --- | --- |
| ETC1 | Contiguous E-UTRA only | Narrowest | 5 MHz | 5 MHz |
| ETC2 | contiguous CA occupied BW | 5 MHz | 5 MHz |  |
| ETC3 | non-contiguous | 5 MHz | 5 MHz |  |
| ETC4 | Multi band |  |  |  |
| ETC5 | Multi band |  |  |  |
| ETC7 | E-UTRA + NB-IoT SA | NB-IoT | 5 MHz |  |
| ETC8 | E-UTRA + NB-IoT IB | 5 MHz | 5 MHz | 5 MHz |
| ETC9 | E-UTRA + NB-IoT GB | 10 MHz | 10 MHz | 10 MHz |

Observation1: Except ETC1, all E-UTRA test configurations are built with a 5 MHz channel bandwidth signal.

## MSR test configurations and test signal

The following Table 2 lists the different test configurations with E-UTRA specified in TS 37.141 and the signals used to build those test configurations.

Table 2: TS 37.141 - Test configurations and test signals

| Test Configuration | Usage | Test signal at the lower edge | Test signal at the upper edge | Test signal inside the block |
| --- | --- | --- | --- | --- |
| TC2 | E-UTRA multicarrier operation | Narrowest | 5 MHz | 5 MHz |
| NTC2 | E-UTRA multicarrier non-contiguous operation | 5 MHz | 5 MHz |  |
| TC3 | UTRA and E-UTRA multi RAT operation | UTRA | 5 MHz |  |
| NTC3 | UTRA and E-UTRA multi RAT non-contiguous operation | UTRA | 5 MHz |  |
| TC4 | BC2 transmitter operation | GSM | 5 MHz | 5 MHz |
| NTC4 | Non-contiguous multi RAT operations with GSM for the transmitter | GSM | 5 MHz | 5 MHz |
| TC5 | BC2 receiver operation | GSM | 5 MHz |  |
| NTC5 | Non-contiguous multi RAT operations with GSM for the receiver | GSM | 5 MHz |  |
| TC6 | Single carrier for receiver tests | NA | NA | narrowest |
| TC11 | E-UTRA and NB-IoT standalone multi-carrier operation | NB-IoT | 5 MHz | 5 MHz |
| TC13 | GSM and E-UTRA and NB-IoT standalone multi-carrier operation | GSM | NB-IoT | 5 MHz |
| TC14 | UTRA and E-UTRA and NB-IoT standalone multi-carrier operation | NB-IoT | 5 MHz |  |
| TC15 | GSM and E-UTRA with NB-IoT in-band multi-carrier operation | GSM | 5 MHz |  |
| TC16 | UTRA and E-UTRA with NB-IoT in-band multi-carrier operation | UTRA | 5 MHz |  |
| TC17 | E-UTRA and E-UTRA with NB-IoT in-band multi-carrier operation | 5 MHz | 5 MHz |  |
| TC18 | GSM and E-UTRA with NB-IoT guard-band multi-carrier operation | GSM | 10 MHz |  |
| TC19 | UTRA and E-UTRA with NB-IoT guard-band multi-carrier operation | UTRA | 10 MHz |  |
| TC20 | E-UTRA and E-UTRA with NB-IoT guard-band multi-carrier operation | 10 MHz | 10 MHz |  |
| TC21 | Contiguous operation in CS16, 18, 19 | 10 or 5 MHz | 10 or 5 MHz |  |
| NTC21 | Non-contiguous operation in CS16, 18, 19 | 10 or 5 MHz | 10 or 5 MHz |  |
| TC22 | Contiguous operation in CS17 | 10 or 5 MHz | 10 or 5 MHz |  |

Observation2: Except TC2 and TC6, all E-UTRA test configurations are built with a E-UTRA 5 or 10 MHz channel bandwidth signal.

## Conclusion

Based on above, we make the following observation:

Observation3: Only when testing a contiguous spectrum allocation and when NB-IoT operating in-band is not supported, the E-UTRA test signal used to build the TC has the narrowest supported channel bandwidth. In all other cases, 5 MHz channel bandwidth E-UTRA signal is used (or 10 MHz for NB-IoT guard band).

Following the same principle for NR testing, now that 3 MHz channel bandwidth, we would then make the following proposal:

Proposal1: Build NRTC1 with the narrowest supported channel bandwidth NR signal if NB-IoT is not supported and with 5 MHz channel BW signal when NB-IoT is supported. Build all other test configurations with a 5 MHz channel bandwidth signal.

Note that our companion draft CR to TS 38.141-1 ([3]) and CR to TS 37.141 ([4]) are based on the above proposal.

Further considering the channel filtering aspects and the spectrum utilization, testing with a 5 MHz channel bandwidth NR signal might even be more stringent comparing to testing with a 3 MHz channel bandwidth NR signal. We would then make the following proposal which would even simplify more all NR testing aspects.

Proposal2: RAN4 should further consider building all NR test configurations (even NRTC1 when NB-IoT is not supported) using a 5 MHz channel bandwidth NR signal.

If this proposal2 is accepted, our companion draft CR to TS 38.141-1 ([3]) should be revised, only removing the proposed changes to sub-clause 4.7.2.

# Conclusion

In this contribution we further analyzed the E-UTRA test configurations specified in TS 36.141 and TS 37.141 and made the following observations and proposals:

Observation1: Except ETC1, all E-UTRA test configurations are built with a 5 MHz channel bandwidth signal.

Observation2: Except TC2 and TC6, all E-UTRA test configurations are built with a E-UTRA 5 or 10 MHz channel bandwidth signal.

Observation3: Only when testing a contiguous spectrum allocation and when NB-IoT operating in-band is not supported, the E-UTRA test signal used to build the TC has the narrowest supported channel bandwidth. In all other cases, 5 MHz channel bandwidth E-UTRA signal is used (or 10 MHz for NB-IoT guard band).

Proposal1: Build NRTC1 with the narrowest supported channel bandwidth NR signal if NB-IoT is not supported and with 5 MHz channel BW signal when NB-IoT is supported. Build all other test configurations with a 5 MHz channel bandwidth signal.

Proposal2: RAN4 should further consider building all NR test configurations (even NRTC1 when NB-IoT is not supported) using a 5 MHz channel bandwidth NR signal.

# References

- RP-231425, WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Anterix, T-Mobile USA

- R4-2310408, WF on system parameters for less than 5MHz channel bandwidth, Nokia

- R4-2319582, Draft CR to TS 38.141: Introduction of 3 MHz channel bandwidth.