3GPP TSG-RAN WG4 Meeting # 109	R4-2319633

Chicago, US, Nov. 13 – Nov. 17, 2023

Agenda Item: 8.32.1

Source: MediaTek Inc.

Title: Discussion on RRM core requirements for R18 SL relay

Document for: Discussion

- Introduction

RAN4#108bis had some discussion on the RRM requirements for NR SL relay [1]. There is one open issue. We will further discuss the open issue in this contribution.

- Discussion

In last meeting, RAN4 discussed whether to define new requirements for remote UE in R18 multi-path scenarios.

Issue 1-1-2: Interruptions in multi-path relay scenario Agreement: Understanding of R17 specificaiton for U2N scenario: - The remote UE can cause interruption on its serving cell due to the RRC reconfiguration. - FFS: Need to align the understanding of the existing RAN2/4 specification whether or not the remote UE can cause interruption on its cell due to the SL DRX operation. For R18 multi-path scenario: - No new interruption requiremet for remote UE on its serving cell due to the RRC reconfiguration. - The remote UE can cause interruption on its serving cell due to the SL DRX operation, and further discuss whether or not to define new requirement after companies are aligned on the understanding of R17 specificaiton.

Some company proposed that the legacy interruption requirements due to SL-DRX operation defined in R17 does not consider remote UE because in R17 remote UE is out of coverage and there is no serving cell. But we have different view. In our understanding, the following three scenarios are all supported in R17. Remote UE may be in coverage, may be not. So R17 remote UE shall follow legacy requirements. There is no difference between R17 and R18 when it comes to interruption requirements due to SL DRX operation.

Observation 1: Legacy interruption requirements due to SL-DRX operation defined in R17 are applicable to R17 remote UE as the scenarios remote UE in coverage or not are all considered in R17 SL relay.

Proposal 1: No need to define new interruption requirements for remote UE due to SL-DRX operation. The legacy requirements apply.

- Summary

In this paper, we have some discussion on R18 SL relay. We have the following observation and proposal:

Observation 1: Legacy interruption requirements due to SL-DRX operation defined in R17 are applicable to R17 remote UE as the scenarios remote UE in coverage or not are all considered in R17 SL relay.

Proposal 1: No need to define new interruption requirements for remote UE due to SL-DRX operation. The legacy requirements apply.

4 Reference

- R4-2317383, WF on SL Relay RRM core/performance requirements, RAN4#108bis, Oct. 13 – 19, 2023.