3GPP TSG-RAN WG4 Meeting #109	R4-2319659

Chicago, USA, 13 - 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to add DC_1A-3A-5A-7A_n28A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_xBLTE_2BNR_yDL2UL |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding new combination |  |  |  |  |  |  |  |  |
| Summary of change: |  | DC_1A-3A-5A-7A_n28A-n78A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New combination is not included in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.5, 6.2B.4.2.3.5, 7.3B.3.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

#### 5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)

Table 5.5B.4.5-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A_n28A-n78A | DC_1A_n28A DC_3A_n28A DC_5A_n28A DC_7A_n28A DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-5A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-5A-7A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-5A-7A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-5A-7A_n40A-n78A DC_1A-3A-5A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-3A-7A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-3A-7A-8A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A-8A-40A_n78A DC_1A-3A-7A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-7A-8A-40A_n78(2A) DC_1A-3A-7A-8A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-7A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-3A-7A-20A_n28A-n78A2,3,6,7 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-7A-20A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A-20A-38A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-7A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-7A-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-7C-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-3A-7A-28A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_3A_n5A DC_3A_n40A DC_7A_n5A DC_7A_n40A DC_28A_n5A DC_28A_n40A |
| DC_1A-3A-7A-28A_n5A-n78A DC_1A-3A-7C-28A_n5A-n78A DC_1A-3C-7A-28A_n5A-n78A DC_1A-3C-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-3A-7A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3C-7A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-7A-28A_n38A-n78A | DC_1A_n78A8 DC_3A_n78A8 DC_28A_n78A8 |
| DC_1A-3A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-3A-8A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-8A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-8A-20A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-7A-20A-28A-32A_n3A DC_1A-7C-20A-28A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-7A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |
| DC_1A-8A_n3A-n28A-n77A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A-11A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-42A_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42A_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42C_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42C_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_2A-5A-7A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_5A_n2A DC_5A_n78A DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-7A-12A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_12A_n2A DC_12A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-7A-66A-71A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A DC_71A_n2A DC_71A_n78A |
| DC_3A-7A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-7A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-8A-20A-32A-38A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_7A-20A-28A-32A-38A_n1A | DC_20A_n1A DC_28A_n1A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 8: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

---Text omitted---

###### 6.2B.4.2.3.5	ΔTIB,c for EN-DC six bands

Table 6.2B.4.2.3.5-1: ΔTIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |  |  |  |  |
| DC_1-3-5-7_n28-n78 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.8 |
| DC_1-3-5-7_n40-n77 DC_1-3-5-7-7_n40-n77 | 0.6 | 0.6 | 0.6 | 0.5 | 0.31 | 0.81 |
| DC_1-3-5-7_n40-n78 | 0.6 | 0.6 | 0.6 | 0.5 | 0.31 | 0.81 |
| DC_1-3-7-8_n28-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-7-8-32_n78 | 0.6 | 0.6 | 0.6 | 0.6 | - | 0.8 |
| DC_1-3-7-8-40_n78 | 0.6 | 0.6 | 0.5 | 0.6 | 0.31 | 0.81 |
| DC_1-3-7-20_n8-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-7-20_n28-n78 | 0.7 | 0.7 | 0.7 | 0.6 | 0.6 | 0.8 |
| DC_1-3-7-20-32_n78 | 0.7 | 0.7 | 0.7 | 0.4 | - | 0.8 |
| DC_1-3-7-20-38_n78 | 0.7 | 0.7 | - | 0.6 | - | 0.8 |
| DC_1-3-7-20_n38-n78 | 0.6 | 0.6 | - | 0.6 | - | 0.8 |
| DC_1-3-7-28_n3-n78 | 0.7 | 0.7 | 0.7 | 0.6 | 0.7 | 0.8 |
| DC_1-3-7-28_n5-n40 | 0.6 | 0.6 | 0.8 | 0.6 | 0.6 | 0.9 |
| DC_1-3-7-28_n7-n78 | 0.7 | 0.7 | 0.7 | 0.6 | 0.7 | 0.8 |
| DC_1-3-7-28_n40-n78 | 0.6 | 0.6 | 0.8 | 0.3 | 0.9 | 0.8 |
| DC_1-3-7-28_n38-n78 | 0.7 | 0.7 | 0.7 | 0.6 | 0.7 | 0.8 |
| DC_1-3-8-11_n28-n77 | 0.6 | 0.8 | 0.6 | 0.9 | 0.6 | 0.8 |
| DC_1-3-8-20-28_n78 | 0.3 | 0.3 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7-20-28-32_n3 | 0.6 | 0.6 | 0.6 | 0.6 | - | 0.6 |
| DC_1-7-20-38_n3-n78 | 0.6 | 0.7 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_1-8_n3-n28-n77-n79 | 0.6 | 0.6 | 0.8 | 0.6 | 0.8 | 0.8 |
| DC_1-8-11_n3-n28-n77 | 0.6 | 0.6 | 0.8 | 0.9 | 0.6 | - |
| DC_1-8-42_n3-n28-n77 | 0.6 | 0.6 | 0.8 | 0.8 | - | 0.8 |
| DC_2-5-7-66_n2-n78 | 0.5 | 0.3 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_2-7-12-66_n2-n78 | 0.6 | 0.6 | 0.8 | 0.5 | 0.5 | 0.8 |
| DC_2-7-66-71_n2-n78 | 0.5 | 0.5 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_3-7-8-40_n1-n78 | 0.6 | 0.5 | 0.6 | 0.32 | 0.6 | 0.82 |
| DC_7-8-20-32-38_n1 | - | 0.6 | 0.6 | - | - | 0.7 |
| DC_7-20-28-32-38_n1 | - | 0.6 | 0.6 | - | - | 0.7 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |  |  |  |

---Text omitted---

##### 7.3B.3.3.5	ΔRIB,c for EN-DC six bands

Table 7.3B.3.3.5-1: ΔRIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |  |  |  |  |
| DC_1A-3-5-7_n28-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-5-7_n40-n77 DC_1-3-5-7-7_n40-n77 | 0.2 | 0.2 | 0.2 | - | 0.41 | 0.51 |
| DC_1-3-5-7_n40-n78 | 0.2 | 0.2 | 0.2 | - | 0.41 | 0.51 |
| DC_1-3-7-8_n28-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-7-8-32_n78 | 0.2 | 0.2 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-7-8-40_n78 | 0.2 | 0.2 | - | 0.2 | 0.41 | 0.51 |
| DC_1-3-7-20_n8-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-7-20_n28-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-7-20-32_n78 | 0.2 | - | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-7-20-38_n78 | 0.7 | 0.7 | - | 0.6 | - | 0.8 |
| DC_1-3-7-20_n38-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-7-28_n3-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-7-28_n5-n40 | - | - | 0.3 | 0.2 | 0.2 | 0.8 |
| DC_1-3-7-28_n7-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-7-28_n38-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-7-28_n40-n78 | - | - | 0.3 | 0.2 | 0.8 | 0.5 |
| DC_1-3-8-11_n28-n77 | 0.2 | 0.3 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_1-3-8-20-28_n78 | - | - | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-20-28-32_n3 | - | - | 0.2 | 0.2 | - | - |
| DC_1-7-20-38_n3-n78 | 0.6 | 0.6 | 0.2 | 0.4 | - | 0.8 |
| DC_1-8_n3-n28-n77-n79 | 0.3 | 0.3 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_1-8-11_n3-n28-n77 | 0.2 | 0.2 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_1-8-42_n3-n28-n77 | 0.2 | 0.2 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_2-5-7-66_n2-n78 | 0.3 | - | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-12-66_n2-n78 | 0.3 | 0.3 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-66-71_n2-n78 | 0.3 | 0.5 | 0.5 | - | 0.3 | 0.5 |
| DC_3-7-8-40_n1-n78 | 0.2 | - | 0.2 | 0.42 | 0.2 | 0.52 |
| DC_7-8-20-32-38_n1 | - | 0.2 | 0.2 | - | - | - |
| DC_7-20-28-32-38_n1 | - | 0.2 | 0.2 | - | 0.2 | - |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 3: “-” denotes ΔRIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |  |  |  |

---End of changes---