3GPP TSG-RAN WG4 Meeting #109	R4-2319662

Chicago, USA, 13-17 November 2023

Source:	Ericsson

Title:	TP for 38.718-03-01 to include CA_n3-n5-n79

Agenda item:			7.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38.718-03-01 to include CA_n3-n5-n79.

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n3-n5-n79

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n5-n79 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3+n5+n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n5A-n79A | CA_n3A_n5A CA_n3A_n79A CA_n5A_n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n5-n79, the TIB,c and RIB,c values are reused from CA_n3-n28-n79 and are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n5-n79 | 0.3 | 0.5 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n5-n79 | - | 0.2 | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

Co-existence studies shows that IMD3 and IMD4 caused by UL CA_n3-n5 Tx may fall into band n79 Rx.

Co-existence studies shows that IMD3 and IMD4 caused by UL CA_n3-n79 Tx may fall into band n5 Rx.

Co-existence studies shows that IMD4 caused by UL CA_n5-n79 Tx may fall into band n3 Rx.

#### 5.x.2.2	REFSENS requirements

Table 5.x.2.2-1 lists the MSD required for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n3A-n8A-n79A and CA_n3-n28-n79 are reused.

Table 5.x.2.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n5-n79 | n3 | N/A | 5 | N/A | 1877.5 | 8.8 | FDD | IMD4 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 860 | 15.3 | FDD | IMD3 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 890 | 10.3 | FDD | IMD4 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | FDD | N/A |
|  | n3 | 1782.5 | 5 | 25 | 1875.5 | N/A | FDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4420 | 15.7 | FDD | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n5 | 846 | 5 | 25 | 891 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4494 | 9.4 | FDD | IMD4 |

---End of changes---

# Reference

[1] 		RP-231860, Revised WID:Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2) , ZTE Corporation