| 3GPP TR 37.718-21-11 V0.9.0 (2023-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Dual Connectivity of 2 bands LTE inter-band CA and 1 NR band (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Background	8

4.1	TR Maintenance	8

5	DC of 2 bands LTE inter-band CA and 1 NR band within FR1: Specific Band Combination Part	8

5.x	DC_a-b_nc	8

5.1	DC_1-(n)7	9

5.2	DC_3-(n)7	10

5.3	DC_28-(n)7	11

5.4	DC_1-26_n78	12

5.5	DC_3-26_n78	13

5.6	DC_7-26_n78	15

5.7	DC_1A-8A_n7A	16

5.8	DC_3-8_n78, DC_3-3-8_n78	18

5.9	DC_7-8_n78, DC_7-7-8_n78	21

5.10	DC_1-3_n26	23

5.11	DC_1-7_n26	24

5.12	DC_3-7_n26	25

5.13	DC_3-41_n1	27

5.14	DC_8-41_n78	28

5.15	DC_20-41_n1	29

5.16	DC_20-41_n78	30

5.17	DC_1-7_n1	31

5.18	DC_1-7_n20	33

5.19	DC_1-8_n20	37

5.20	DC_3-20_n3	41

5.21	DC_3-32_n7	44

5.22	DC_8-28_n3	47

5.23	DC_20-32_n7	52

5.24	DC_7-8_n7	54

5.25	DC_3-5_n40	56

5.28	DC_20-28_n78	61

5.29	DC_1-28_n20	66

5.30	DC_1-5_n40	69

5.31	DC_1-3_n1	73

5.32	DC_1-20_n1	75

5.33	DC_7-8_n20	78

5.34	DC_7-28_n20	83

5.35	DC_5-7_n40	88

5.36	DC_20-(n)3	90

5.37	DC_3-8_n7	91

5.38	DC_3-67_n3	95

5.39	DC_67-(n)3	96

5.40	DC_20-67_n3	97

5.41	 DC_3-8_n41	98

5.42	DC_2-4_n78	102

5.43	DC_28-38_n78	105

5.44	DC_4-7_n78	108

5.45	DC_66-71_n25	110

5.46	DC_2-71_n7	111

5.47	DC_2-71_n77	112

5.48	DC_7-71_n77	113

5.49	DC_66-71_n77	114

5.50	DC_7-12_n77	115

5.51	DC_7-12_n25	117

5.52	DC_7-66_n12	118

5.53	DC_7-71_n25	119

5.54	DC_12-66_n7	120

5.55	DC_66-71_n7	121

5.56	DC_1A-3A_n105A	122

5.57	DC_1A-7A_n105A	125

5.58	DC_3A-7A_n105A	127

5.59	DC_8-39_n40	129

5.60	DC_66-71_n2	131

5.61	 DC_8-39_n41	134

5.62	DC_7-66_n2	135

5.63	DC_3-40_n77	138

5.64	DC_5-40_n77	139

5.65	DC_5-40_n78	140

5.66	DC_2-5_n41	141

5.67	DC_2-7_n12	142

5.68	DC_5-7_n25	146

5.69	DC_5-66_n25	147

5.70	DC_5-66_n41	148

5.71	DC_7-71_n12	149

5.72	DC_12-71_n2	150

5.73	DC_12-71_n77	151

5.74	DC_66-71_n12	152

5.75	DC_3-28_n1	153

5.76	DC_4-5_n78	154

5.77	  DC_8-39_n79	155

5.78	DC_3-7_n79, DC_3-3-7_n79, DC_3-7-7_n79, DC_3-3-7-7_n79	157

5.79	DC_3-5_n28	158

5.80	DC_5-7_n28	159

5.81	DC_1-5_n28	160

Annex A (informative): Change history	162

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for Dual Connectivity of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) under Rel-18 time frame. The purpose is to gather the relevant background information and studies in order to address Dual connectivity (DC) band combinations of 3 different bands DL with 2 different bands UL (2 different LTE bands and 1 NR band) for the Rel-18 band combinations. The actual requirements are added to the corresponding technical specification.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	Background

The present document is a technical report for Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA and 1 NR band under Rel-18 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	DC of 2 bands LTE inter-band CA and 1 NR band within FR1: Specific Band Combination Part

5.x	DC_a-b_nc

5.x.1	Configurations for DC

<Editor’s note: it is required to use the same table format as in TS 38.101-3>

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |

5.x.2	Co-existence studies

<Text will be added.>

5.x.3	∆TIB and ∆RIB values

<Editor’s note: it is required to use the same table format as in TS 38.101-3>

Table 5.X.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_a-b_nc |  |  |  |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.X.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_a-b_nc |  |  |  |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.x.4	Reference sensitivity exceptions

< Editor’s note: text will be added only for reference sensitivity exceptions for intermodulation interference due to dual uplink operation for DC in NR FR1 involving three bands >

5.1	DC_1-(n)7

### 5.1.1	Operating bands for EN-DC

Table 5.1.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-(n)7 | CA_1-7 | n7 | No |

5.1.2	Configuration for DC

Table 5.1.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-(n)7AA | DC_1A_n7A | CA_1A-7A | n7A |

### 5.1.3		∆TIB and ∆RIB values

For DC_1-(n)7, the TIB,c and RIB,c values are reused from DC_1_n7 and are given in the tables below.

Table 5.1.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-(n)7 | 0.5 | 0.6 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.1.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-(n)7 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.1.4	REFSENS requirements

There are no IMD impact from UL 1_7 affecting DL band 1 or band n7.

5.2	DC_3-(n)7

### 5.2.1	Operating bands for EN-DC

Table 5.2.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-(n)7 | CA_3-7 | n7 | No |

5.2.2	Configuration for DC

Table 5.2.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-(n)7AA | DC_3A_n7A | CA_3A-7A | n7A |
| DC_3C-(n)7AA | DC_3A_n7A | CA_3C-7A | n7A |

### 5.2.3		∆TIB and ∆RIB values

For DC_3-(n)7, the TIB,c and RIB,c values are reused from DC_3_n7 and are given in the tables below.

Table 5.2.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-(n)7 | 0.5 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.2.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-(n)7 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.2.4	REFSENS requirements

There are possible IMD4 impact from UL 3_n7 affecting band 7 MSD values based on Skyworks discussion paper R4-2215523: MSD for DC_3A_(n)7AA.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 7 | N/A | 5 | N/A | 2647.5 | 6.9 | IMD4 |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |

5.3	DC_28-(n)7

### 5.3.1	Operating bands for EN-DC

Table 5.3.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 28-(n)7 | CA_28-7 | n7 | No |

5.3.2	Configuration for DC

Table 5.3.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_28A-(n)7AA | DC_28A_n7A | CA_28A-7A | n7A |

### 5.3.3		∆TIB and ∆RIB values

For DC_28-(n)7, the TIB,c and RIB,c values are reused from DC_28_n7 and are given in the tables below.

Table 5.3.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28-(n)7 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.3.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28-(n)7 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.3.4	REFSENS requirements

There are no IMD impact from UL 28_7 affecting DL band 28 or band n7.

5.4	DC_1-26_n78

### 5.4.1	Operating bands for EN-DC

Table 5.4.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-26_n78 | CA_1-26 | n78 | No |

5.4.2	Configuration for DC

Table 5.4.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-26A_n78A | DC_1A_n78A DC_26A_n78A | CA_1A-26A | n78A |

### 5.4.3		∆TIB and ∆RIB values

For DC_1-26_n78, the TIB,c and RIB,c values are reused from DC_1-5_n78 and are given in the tables below.

Table 5.4.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-26_n78 | 0.3 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.4.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-26_n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.4.4	REFSENS requirements

There are IMD3 impact from UL 26_n78 affecting DL band 1.

There are IMD5 impact from UL 1_n78 affecting DL band 26.

MSD values are reused from DC_1A-5A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-26A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 26 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 26 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |

5.5	DC_3-26_n78

### 5.5.1	Operating bands for EN-DC

Table 5.5.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-26_n78 | CA_3-26 | n78 | No |

5.5.2	Configuration for DC

Table 5.5.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-26A_n78A DC_3C-26A_n78A | DC_3A_n78A DC_26A_n78A | CA_3A-26A | n78A |

### 5.5.3		∆TIB and ∆RIB values

For DC_3-26_n78, the TIB,c and RIB,c values are reused from DC_3-5_n78 and are given in the tables below.

Table 5.5.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-26_n78 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.5.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-26_n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.5.4	REFSENS requirements

There are IMD3 impact from UL 26_n78 affecting DL band 3. MSD values are reused from DC_5A_n3A-n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-26A_n78A DC_3C-26A_n78A | 3 | 1767 | 5 | 25 | 1862 | 15.7 | IMD3 |
|  | 26 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | N/A |

5.6	DC_7-26_n78

### 5.6.1	Operating bands for EN-DC

Table 5.6.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-26_n78 | CA_7-26 | n78 | No |

5.6.2	Configuration for DC

Table 5.6.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-26A_n78A DC_7C-26A_n78A | DC_7A_n78A DC_26A_n78A | CA_7A-26A | n78A |

### 5.6.3		∆TIB and ∆RIB values

For DC_7-26_n78, the TIB,c and RIB,c values are reused from DC_5-7_n78 and are given in the tables below.

Table 5.6.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-26_n78 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.6.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-26_n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.6.4	REFSENS requirements

There are IMD2 and IMD5 impact from UL 7_n78 affecting DL band 26. MSD values are reused from DC_5A-7A_n78A.

There are IMD2 impact from UL 26_n78 affecting DL band 7. MSD values are reused from DC_5A-7A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-26A_n78A DC_7C-26A_n78A | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | 26 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | 26 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | 26 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |

5.7	DC_1A-8A_n7A

### 5.7.1	Configurations for DC

Table 5.7.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-8A_n7A | DC_8A_n7A DC_1A_n7A |

### 5.7.2	Co-existence studies

For 2UL/3DL(DC_1A-8A_n7A  with UL DC_1A_n7A) UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.239.2-1.

Table 5.7.2-1: Harmonic and IMD analysis for DC_1A_n7A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

As we can see from the above table，for 3DL_DC_1A-8A_n7A with 2UL_1A_n7A，Two-tone 5th order IMD products |2*fy -3*fx |may fall into DL reception frequency of Band 8.

### 5.7.3	∆TIB and ∆RIB values

The requirements of ∆TIB values in Table 6.2.5-3: ΔTIB,c (three bands)  “CA_1-7-8 , CA_1-7-7-8”from TS36.101 [2] can be applied for DC_1-8_n7.

The requirements of ∆RIB values in Table 7.3.1-1B: ΔRIB,c (three bands) “CA_1-7-8 , CA_1-7-7-8” from TS36.101 [2] can be applied for DC_1-8_n7.

Table 5.7.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-8_n7 | 0.5 | 0.6 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.7.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-8_n7 | - | 0.2 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.7.4	Reference sensitivity exceptions

Table 5.7.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from DC_1A-8A_n7A.

Table 5.7.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-8A_n7A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
|  | 8 | 882.5 | 5 | 25 | 927.5 | 1.0 | IMD5 |

5.8	DC_3-8_n78, DC_3-3-8_n78

5.8.1	Configurations for DC

Table 5.8.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-8B_n78A5 | DC_3A_n78A DC_8A_n78A DC_8B_n78A |
| DC_3A-3A-8B_n78A5 | DC_3A_n78A DC_8A_n78A DC_8B_n78A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

5.8.2	Co-existence studies

Co-existence was studied for DC_3A-8A_n78A in Rel-15 and the results are captured in TR 37.863-02-01. Based on the study for the impact on the third band, the 3rd order IMD generated by dual uplink of Band 8 + Band n78 may fall into own Rx of band 3.

5.8.3	∆TIB and ∆RIB values

For DC_3A-8B_n78A, DC_3A-3A-8B_n78A, the TIB,c and RIB,c values are already covered by the DC_3-8_n78, DC_3-3-8_n78, as in the tables below.

Table 5.8.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-8_n78 DC_3-3-8_n78 | 0.6 | 0.6 | 0.8 |

Table 5.8.3-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-8_n78 DC_3-3-8_n78 | 0.2 | 0.2 | 0.5 |

5.8.4	Reference sensitivity exceptions

For the MSD requirements for IMD interference of DC_3A-8B_n78A and DC_3A-3A-8B_n78A, the requirements for DC_3A-8A_n78A and DC_3A-3A-8A_n78A can be referred as below.

Table 5.8.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-8B_n78A DC_3A-3A-8B_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |

The UL triple beat analysis for the three uplink CCs from the DC_8B_n78A impact to the 3rd DL band is provided below.

Based on the WF R4-2220556, the UL TB analysis focus on the potential de-sense when the following 1st order triple beat product fallls into the Rx frequency range of the third band:

TB1 = |f1+f2-f3|

TB2 = |f1-f2+f3|

TB3 = |f1-f2-f3|

TB4 = f1+f2+f3

In the above equations, f1 is the center frequency of the non-UL CA uplink resource block (RB), and f2 and f3 are the center frequencies of each of the UL CA CC uplink RB, with the consideration of 1RB case. See figure below.

Figure 5.9.4-1: UL triple beat of DC_8B_n78A

Here we provide a approximate checking on the TB impact in the below table.

Table 5.9.4-2: The impacted frequency range of 1st order TB of UL DC_8B_n78A

| UL carriers | f1_low | f1_high | min ¦f2-f3¦ | max ¦f2-f3¦ | min ¦f2+f3¦ | max ¦f2+f3¦ |
| --- | --- | --- | --- | --- | --- | --- |
| frequqncy (MHz) | 3300 | 3800 | ~0 | ~20 | ~1770 | ~1820 |
| TB1 = ¦f1+f2-f3¦ | min TB1 | max TB1 | TB2 = ¦f1-f2+f3¦ | min TB2 | max TB2 |  |
| frequqncy limit (MHz) | ~3300 | ~3820 |  | ~3280 | ~3800 |  |
| TB3 = ¦f1-f2-f3¦ | min TB3 | max TB3 | TB4 = ¦f1+f2+f3¦ | min TB4 | max TB4 |  |
| frequqncy limit (MHz) | ~1480 | ~2030 |  | ~5070 | ~5620 |  |
| Note: assume f2 > f3 |  |  |  |  |  |  |

Based on Table 5.9.4-2, TB3 might fall into Rx frequency range of the 3rd band, band 3, so additional MSD might be needed. The additional MSD requirement is provided in table below.

Table 5.9.4-3: The MSD requirement for 1st order TB of UL DC_8B_n78A

| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-8B_n78A | 3 | N/A | 5 | N/A | 1845 | 4.1 | FDD | IMD3 |
|  | 8 | 900 | 10 | 1 (RBSTART=35) | 945 | N/A | FDD | N/A |
|  |  | 910 | 10 | 1 (RBSTART=30) | 955 | N/A | FDD | N/A |
|  | n78 | 3660 | 10 | 25 (RBSTART=0) | 3660 | N/A | TDD | N/A |

The detail MSD analysis is in R4-2310410. Note that the UL RB setting of the CA_8B configuration is further updated to eliminate band 8 self-desense, and it also doesn’t create overlap of TB and IMD3 in band 3.

However, based on the agreed WF R4-2310316, it is found that if TB frequency is composed of the frequency sum of the 2 discrete RBs in the contiguous UL CA, there is no need to specify the TB test configuration as the requirement can already be verified by the fallback 2UL IMD3. Hence, the MSD requirement proposed in table 5.9.4-3 will be a reference for the TR, and will not need to be specified in the TS specification.

5.9	DC_7-8_n78, DC_7-7-8_n78

5.9.1	Configurations for DC

Table 5.9.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-8B_n78A5 | DC_7A_n78A DC_8A_n78A DC_8B_n78A |
| DC_7A-7A-8B_n78A5 | DC_7A_n78A DC_8A_n78A DC_8B_n78A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

5.9.2	Co-existence studies

Co-existence was studied for DC_7A-8A_n78A in Rel-16 and the results are captured in TR 37.716-21-11. Based on the study for the impact on the third band,

- 2nd and 5th order IM generated by dual uplink of Band 7 and n78 may fall into Band 8

- 2nd order IM generated by dual uplink of Band 8 and n78 may fall into Band 7

5.9.3	∆TIB and ∆RIB values

For DC_7A-8B_n78A, DC_7A-7A-8B_n78A, the TIB,c and RIB,c values are already covered by the DC_7-8_n78, as in the tables below.

Table 5.9.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-8_n78 DC_7-7-8_n78 | 0.5 | 0.6 | 0.8 |

Table 5.9.3-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-8_n78 DC_7-7-8_n78 | 0 | 0.2 | 0.5 |

5.9.4	Reference sensitivity exceptions

For the MSD requirements for IMD interference of DC_7A-8B_n78A and DC_7A-7A-8B_n78A, the requirements for DC_7A-8A_n78A can be referred as below.

Table 5.9.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-8B_n78A DC_7A-7A-8B_n78A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 28 | IMD2 |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |

The UL triple beat analysis for the three uplink CCs from the DC_8B_n78A impact to the 3rd DL band is provided below.

Based on the WF R4-2220556, the UL TB analysis focus on the potential de-sense when the following 1st order triple beat product fallls into the Rx frequency range of the third band:

TB1 = |f1+f2-f3|

TB2 = |f1-f2+f3|

TB3 = |f1-f2-f3|

TB4 = f1+f2+f3

In the above equations, f1 is the center frequency of the non-UL CA uplink resource block (RB), and f2 and f3 are the center frequencies of each of the UL CA CC uplink RB, with the consideration of 1RB case. See figure below.

Figure 5.9.4-1: UL triple beat of DC_8B_n78A

Here we provide an approximate checking on the TB impact in the below table.

Table 5.9.4-2: The impacted frequency range of 1st order TB of UL DC_8B_n78A

| UL carriers | f1_low | f1_high | min ¦f2-f3¦ | max ¦f2-f3¦ | min ¦f2+f3¦ | max ¦f2+f3¦ |
| --- | --- | --- | --- | --- | --- | --- |
| frequqncy (MHz) | 3300 | 3800 | ~0 | ~20 | ~1770 | ~1820 |
| TB1 = ¦f1+f2-f3¦ | min TB1 | max TB1 | TB2 = ¦f1-f2+f3¦ | min TB2 | max TB2 |  |
| frequqncy limit (MHz) | ~3300 | ~3820 |  | ~3280 | ~3800 |  |
| TB3 = ¦f1-f2-f3¦ | min TB3 | max TB3 | TB4 = ¦f1+f2+f3¦ | min TB4 | max TB4 |  |
| frequqncy limit (MHz) | ~1480 | ~2030 |  | ~5070 | ~5620 |  |
| Note: assume f2 > f3 |  |  |  |  |  |  |

Based on Table 5.9.4-2, there is no TB falls into the Rx frequency range of the 3rd band, band 7, so it can be concluded that no additional MSD is needed.

5.10	DC_1-3_n26

### 5.10.1	Operating bands for EN-DC

Table 5.10.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n26 | CA_1-3 | n26 | No |

5.10.2	Configuration for DC

Table 5.10.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n26A | DC_1A_n26A DC_3A_n26A | CA_1A-3A | n26A |
| DC_1A-3C_n26A | DC_1A_n26A DC_3A_n26A DC_3C_n26A | CA_1A-3A CA_1A-3C | n26A |

### 5.10.3		∆TIB and ∆RIB values

For DC_1-3_n26, the TIB,c and RIB,c values are reused from DC_1-3_n5 and are given in the tables below.

Table 5.10.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-3_n26 | 0.3 | 0.3 | 0.3 |

Table 5.10.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-3_n26 | 0 | 0 | 0 |

### 5.10.4	REFSENS requirements

Reusing the coexistence study results from DC_1-3_n5, there is no need to define MSD values for DC_1-3_n26.

5.11	DC_1-7_n26

### 5.11.1	Operating bands for EN-DC

Table 5.11.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-7_n26 | CA_1-7 | n26 | No |

5.11.2	Configuration for DC

Table 5.11.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A_n26A | DC_1A_n26A DC_7A_n26A | CA_1A-7A | n26A |
| DC_1A-7C_n26A | DC_1A_n26A DC_7A_n26A DC_7C_n26A | CA_1A-7A CA_1A-7C | n26A |

### 5.11.3		∆TIB and ∆RIB values

For DC_1-7_n26, the TIB,c and RIB,c values are reused from DC_1-7_n5 and are given in the tables below.

Table 5.11.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-7_n26 | 0.5 | 0.6 | 0.3 |

Table 5.11.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-7_n26 | 0 | 0 | 0 |

### 5.11.4	REFSENS requirements

Reusing the coexistence study results from DC_1-7_n5, there is no need to define MSD values for DC_1-7_n26.

5.12	DC_3-7_n26

### 5.12.1	Operating bands for EN-DC

Table 5.12.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n26 | CA_3-7 | n26 | Yes |

5.12.2	Configuration for DC

Table 5.12.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n26A | DC_3A_n26A DC_7A_n26A | CA_3A-7A | n26A |
| DC_3A-7C_n26A | DC_3A_n26A DC_7A_n26A DC_7C_n26A | CA_3A-7A CA_3A-7C | n26A |
| DC_3C-7A_n26A | DC_3A_n26A DC_3C_n26A DC_7A_n26A | CA_3A-7A CA_3C-7A | n26A |
| DC_3C-7C_n26A | DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A | CA_3A-7A CA_3A-7C CA_3C-7A CA_3C-7C | n26A |

### 5.12.3		∆TIB and ∆RIB values

For DC_3-7_n26, the TIB,c and RIB,c values are reused from DC_3-7_n5 and are given in the tables below.

Table 5.12.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-7_n26 | 0.5 | 0.5 | 0.3 |

Table 5.12.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-7_n26 | 0 | 0 | 0 |

### 5.12.4	REFSENS requirements

Based on the discussions in R4-2219700 and reusing the coexistence study results from EN-DC combination DC_3-7_n5, MSD values should be defined as below:

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-7A_n26A DC_3A-7C_n26A DC_3C-7C_n26A DC_3C-7A_n26A | 3 | 1780 | 10 | 50 | 1875 | N/A | FDD | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 |  | IMD2 |
|  | n26 | 845 | 5 | 25 | 890 | N/A |  | N/A |
| DC_3C-7A_n26A DC_3C-7C_n26A | 3 | 1755 | 20 | 1(RBSTART=20) | 1850 | N/A | FDD | N/A |
|  |  | 1774.8 | 20 | 1(RBSTART=79) | 1869.8 | N/A |  |  |
|  | 7 | N/A | 5 | N/A | 2682.5 | 19 | FDD | IMD3 |
|  | n26 | 846.5 | 5 | 25(RBSTART=0) | 891.5 | N/A | FDD | N/A |

5.13	DC_3-41_n1

5.13.1	Configuration for DC

Table 5.13.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-41A_n1A | DC_3A_n1A DC_41A_n1A | CA_3A-41A | n1A |
| DC_3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A | CA_3A-41C | n1A |
| DC_3A-3A-41A_n1A | DC_3A_n1A DC_41A_n1A | CA_3A-3A-41A | n1A |
| DC_3A-3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A | CA_3A-3A-41C | n1A |

5.13.2	Co-existence studies

When uplink is DC_3A_n1A there is IMD5 interfering band 41 downlink.

5.13.3		∆TIB and ∆RIB values

For DC_3-41_n1, the TIB,c and RIB,c values are reused from DC_1_n3-n41 and are given in the tables below.

Table 5.13.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-41_n1 | 0.5 | 0.33 / 0.84 | 0.5 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |  |

Table 5.13.3-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-41_n1 | - | 03 / 0.54 | - |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz |  |  |  |

### 5.13.4	REFSENS requirements

It is proposed to re-use the IMD5 MSD values from already specified configuration DC_1_n3-n41 which is similar to DC_3-41_n1.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-41A_n1A DC_3A-41C_n1A DC_3A-3A-41A_n1A DC_3A-3A-41C_n1A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |

5.14	DC_8-41_n78

5.14.1	Configuration for DC

Table 5.14.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A-41A_n78A | DC_8A_n78A DC_41A_n78A | CA_8A-41A | n78A |
| DC_8A-41C_n78A | DC_8A_n78A DC_41A_n78A DC_41C_n78A | CA_8A-41C | n78A |

5.14.2	Co-existence studies

When uplink is DC_8A_n78A there is IMD2 interfering band 41 downlink.

When uplink is DC_41A_n78A there is IMD2 and IMD5 interfering band 8 downlink.

5.14.3		∆TIB and ∆RIB values

For DC_8-41_n78, the TIB,c and RIB,c values are reused from DC_8-41_n77A and are given in the tables below.

Table 5.14.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8-41_n78 | 0.6 | 0.3 | 0.8 |

Table 5.14.3-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8-41_n78 | 0.2 | - | 0.5 |

### 5.14.4	REFSENS requirements

It is proposed to re-use the IMD2 MSD values from already specified configuration DC_8-41_n77 which is similar to DC_8-41_n78.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-41A_n78A DC_8A-41C_n78A | 8 | 905 | 5 | 25 | 950 | 29.1 | IMD24 |
|  | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | 41 | 2650 | 5 | 25 | 2650 | 28.0 | IMD2 |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.15	DC_20-41_n1

5.15.1	Configuration for DC

Table 5.15.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-41A_n1A | DC_20A_n1A DC_41A_n1A | CA_20A-41A | n1A |
| DC_20A-41C_n1A | DC_20A_n1A DC_41A_n1A DC_41C_n1A | CA_20A-41C | n1A |

5.15.2	Co-existence studies

There is no impact from UL 20_n1 affecting DL band 41.

There are IMD5 impact from UL 41_n1 affecting DL band 20.

### 5.15.3		∆TIB and ∆RIB values

For DC_20-41_n1, the TIB,c and RIB,c values are reused from DC_1-20_n41 and are given in the tables below.

Table 5.15.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_20-41_n1 | 0.3 | 0.51 / 1.22 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |  |

Table 5.15.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_20-41_n1 | 0 | 0 | 0 |

### 5.15.4	REFSENS requirements

MSD values are reused from DC_1A-20A_n7A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-41A_n1A | 20 | 841 | 5 | 25 | 800 | 4.5 | IMD5 |
| DC_20A-41C_n1A | 41 | 2510 | 10 | 50 | 2510 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |

5.16	DC_20-41_n78

5.16.1	Configuration for DC

Table 5.16.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-41A_n78A | DC_20A_n78A DC_41A_n78A | CA_20A-41A | n78A |
| DC_20A-41C_n78A | DC_20A_n78A DC_41A_n78A DC_41C_n78A | CA_20A-41C | n78A |

5.16.2	Co-existence studies

There are IMD2 impact from UL 20_n78 affecting DL band 41.

There are IMD2 and IMD5 impact from UL 41_n78 affecting DL band 20.

### 5.16.3		∆TIB and ∆RIB values

For DC_20-41_n78, the TIB,c and RIB,c values are reused from DC_20_n41-n78 and are given in the tables below.

Table 5.16.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_20-41_n78 | 0.5 | 0.3 | 0.8 |

Table 5.16.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_20-41_n78 | 0 | 0 | 0.5 |

### 5.16.4	REFSENS requirements

MSD value band 41 is reused from DC_20A_n41A-n78A.

MSD value band 20 is reused from CA_n28A-n41A-n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-41A_n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
| DC_20A-41C_n78A | 41 | 2675 | 10 | 50 | 2675 | 29.8 | IMD2 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 20 | 839 | 5 | 25 | 798 | 30.8 | IMD24 |
|  | 41 | 2642 | 10 | 50 | 2642 | N/A | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |

5.17	DC_1-7_n1

5.17.1	Configurations for DC

Table 5.17.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-7A_n1A | DC_1A_n1A DC_7A_n1A |

5.17.2	Co-existence studies

Table 5.17.2-1 lists the Band 7A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.17.2-1: Band 7 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 520 – 650 |  |  | 4420 – 4550 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 3020 – 3220 |  |  | 1270 – 1460 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6920 – 7120 |  |  | 6340 – 6530 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2450 – 2620 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5520 – 5790 |  |  | 3190 – 3440 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1040 – 1300 |  |  | 8840 – 9100 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9420 – 9690 |  |  | 8260 – 8510 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5110 – 5420 |  |  | 8020 – 8360 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 620 – 940 |  |  | 3540 – 3870 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10180 – 10490 |  |  | 11920 – 12260 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10760 – 11080 |  |  | 11340 – 11670 |  |  |

Based on Table 5.17.2-1,

-	2nd order harmonics may fall into Rx frequencies of band 46 and 47.

-	3rd order harmonics may fall into Rx frequencies of band 77.

-	2nd order IMD may fall into Rx frequencies of bands 71 and 79.

-	3rd order IMD may fall into Rx frequencies of bands 32, 45, 50, 51, 75, 76, 91, 92, 93 and 94.

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 22, 26, 27, 28, 29, 42, 43, 44, 46, 48, 49, 67, 68, 71, 77, 78 and 85.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.17.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.17.2-2: 2UL Band 7 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_7_n1.

5.17.3	∆TIB and ∆RIB values

Table 5.17.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-7_n1 | 0.5 | 0.6 | 0.5 |

5.17.4	Reference sensitivity exceptions

No additional requirements compared to fallbacks.

5.18	DC_1-7_n20

5.18.1	Configurations for DC

Table 5.18.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-7A_n20A | DC_1A_n20A DC_7A_n20A |

5.18.2	Co-existence studies

Table 5.18.2-1 lists the Band 1A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.18.2-1: Band 1 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1058 – 1148 |  |  | 2752 – 2842 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2978 – 3128 |  |  | 196 – 316 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4672 – 4822 |  |  | 3584 – 3704 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 – 2000 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 – 5108 |  |  | 516 – 666 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2116 – 2296 |  |  | 5504 – 5684 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 – 6802 |  |  | 4416 – 4566 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1348 – 1528 |  |  | 6818 – 7088 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 – 1464 |  |  | 4036 – 4276 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 – 5428 |  |  | 8512 – 8782 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 – 6546 |  |  | 7424 – 7664 |  |  |

Based on Table 5.18.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 46 and 47.

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69, 77 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77, 78 and 79.

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 23, 46, 65, 66, 71 and 79.

-	5th order IMD may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 50, 51, 74, 75, 76, 77, 91, 92, 93 and 94.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.18.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.18.2-2: 2UL Band 1 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_1_n20.

Table 5.18.2-3 lists the Band 7A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.18.2-3: Band 7 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1638 – 1738 |  |  | 3332 – 3432 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 4138 – 4308 |  |  | 776 – 906 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5832 – 6002 |  |  | 4164 – 4294 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2480 – 2590 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6638 – 6878 |  |  | 74 – 86 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3276 – 3476 |  |  | 6664 – 6864 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8332 – 8572 |  |  | 4996 – 5156 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 758 – 948 |  |  | 9138 – 9448 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2414 – 2644 |  |  | 5776 – 6046 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5828 – 6018 |  |  | 10832 – 11142 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7496 – 7726 |  |  | 9164 – 9434 |  |  |

Based on Table 5.18.2-3,

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69 and 90.

-	2nd order IMD may fall into Rx frequencies of bands 42, 52, 77 and 78.

-	3rd order IMD may fall into Rx frequencies of bands 5, 6, 18, 19, 20, 26, 27, 28, 44, 46, 47, 68 and 77.

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77, 78 and 79.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 7, 8, 14, 18, 19, 20, 26, 27, 28, 38, 41, 44, 46, 47, 53, 68, 69 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.18.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.18.2-4: 2UL Band 7 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_7_n20.

5.18.3	∆TIB and ∆RIB values

Table 5.18.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-7_n20 | 0.5 | 0.6 | 0.3 |

Table 5.18.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-7_n20 | 0.2 | 0.1 | 0 |

5.18.4	Reference sensitivity exceptions

No additional exceptions required compared to fallbacks.

5.19	DC_1-8_n20

5.19.1	Configurations for DC

Table 5.19.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-8A_n20A | DC_1A_n20A DC_8A_n20A |

5.19.2	Co-existence studies

Table 5.19.2-1 lists the Band 1A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.19.2-1: Band 1 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1058 – 1148 |  |  | 2752 – 2842 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2978 – 3128 |  |  | 196 – 316 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4672 – 4822 |  |  | 3584 – 3704 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 – 2000 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 – 5108 |  |  | 516 – 666 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2116 – 2296 |  |  | 5504 – 5684 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 – 6802 |  |  | 4416 – 4566 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1348 – 1528 |  |  | 6818 – 7088 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 – 1464 |  |  | 4036 – 4276 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 – 5428 |  |  | 8512 – 8782 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 – 6546 |  |  | 7424 – 7664 |  |  |

Based on Table 5.19.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 46 and 47.

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69, 77 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77, 78 and 79.

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 23, 46, 65, 66, 71 and 79.

-	5th order IMD may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 50, 51, 74, 75, 76, 77, 91, 92, 93 and 94.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.19.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.19.2-2: 2UL Band 1 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_1_n20.

Table 5.19.2-3 lists the Band 8A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.19.2-3: Band 8 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 18 – 83 |  |  | 1712 – 1777 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 898 – 998 |  |  | 749 – 844 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2592 – 2692 |  |  | 2544 – 2639 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 860 – 935 |  |  | 822 – 872 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778 – 1913 |  |  | 1581 – 1706 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 36 – 166 |  |  | 3424 – 3554 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472 – 3607 |  |  | 3376 – 3501 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2413 – 2568 |  |  | 2658 – 2828 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 666 – 826 |  |  | 916 – 1081 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208 – 4363 |  |  | 4352 – 4522 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256 – 4416 |  |  | 4304 – 4469 |  |  |

Based on Table 5.19.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 7, 41 and 90.

-	3rd order harmonics may fall into Rx frequencies of bands 3, 38, 41, 69 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 7, 8, 13, 14, 20, 28, 38, 38, 41, 44, 67, 68, 69 and 90.

-	4th order IMD may fall into Rx frequencies of bands 3, 9, 22, 33, 35, 37, 39, 42, 43, 48, 49, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 7, 8, 12, 13, 14, 17, 20, 28, 29, 41, 44, 53, 67, 68, 79, 85 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.19.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.19.2-4: 2UL Band 8 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_8_n20.

5.19.3	∆TIB and ∆RIB values

Table 5.19.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-8_n20 | 0.3 | 0.4 | 0.4 |

5.19.4	Reference sensitivity exceptions

No additional exceptions required compared to fallbacks.

5.20	DC_3-20_n3

5.20.1	Configurations for DC

Table 5.20.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-20A_n3A | DC_3A_n3A2 DC_20A_n3A |
| NOTE 2: Only single switched UL is supported |  |

5.20.2	Co-existence studies

Table 5.20.2-1 lists the Band 20A + Band n3A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.20.2-1: Band 20 and Band n3 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 848 – 953 |  |  | 2542 – 2647 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 14 – 121 |  |  | 2558 – 2738 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3374 – 3509 |  |  | 4252 – 4432 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 802 – 892 |  |  | 1690 – 1805 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 711 – 876 |  |  | 4268 – 4523 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1696 – 1906 |  |  | 5084 – 5294 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4206 – 4371 |  |  | 5962 – 6217 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5978 – 6308 |  |  | 1543 – 1738 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3406 – 3691 |  |  | 834 – 1074 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7672 – 8002 |  |  | 5038 – 5233 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6794 – 7079 |  |  | 5916 – 6156 |  |  |

Based on Table 5.20.2-1,

-	2nd order harmonics may fall into Rx frequencies of band 46.

-	3rd order harmonics may fall into Rx frequencies of bands 22, 38, 41, 42, 48, 49, 69, 77, 78 and 90.

-	2nd order IMD may fall into Rx frequencies of bands 5, 6, 7, 8, 18, 19, 26, 27, 38, 41, 69 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 7, 38, 41, 42, 52, 69, 77, 78, 79 and 90.

-	4th order IMD may fall into Rx frequencies of bands 3, 5, 6, 9, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 33, 35, 39, 44, 46, 67, 68, 79 and 85.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 8, 18, 19, 22, 24, 26, 27, 42, 43, 46, 47, 48, 49, 77 and 78.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.20.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.20.2-2: 2UL Band 20 + Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd Harmonic, IMD4, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_20_n3.

5.20.3	∆TIB and ∆RIB values

Table 5.20.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-20_n3 | 0.3 | 0.3 | 0.3 |

5.20.4	Reference sensitivity exceptions

Table 5.204-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-20A_n3A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |

5.21	DC_3-32_n7

5.21.1	Configurations for DC

Table 5.21.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-32A_n7A | DC_3A_n7A |

5.21.2	Co-existence studies

Table 5.21.2-1 lists the Band 3A + Band n7A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.21.2-1: Band 3 and Band n7 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3420 – 3570 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5130 – 5355 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 715 – 860 |  |  | 4210 – 4355 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 850 – 1070 |  |  | 3215 – 3430 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5920 – 6140 |  |  | 6710 – 6925 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1660 – 1835 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2560 – 2855 |  |  | 5715 – 6000 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1430 – 1720 |  |  | 8420 – 8710 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7630 – 7925 |  |  | 9210 – 9495 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8215 – 8570 |  |  | 4270 – 4640 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3930 – 4290 |  |  | 10 – 355 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 – 12065 |  |  | 9340 – 9710 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 – 11280 |  |  | 10130 – 10495 |  |  |

Based on Table 5.21.2-1,

-	2nd order harmonics may fall into Rx frequencies of band 46.

-	3rd order harmonics may fall into Rx frequencies of bands 22, 42, 48, 49, 77 and 78.

-	2nd order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 26, 27, 28, 29, 44, 67, 68 and 85.

-	3rd order IMD may fall into Rx frequencies of bands 5, 6, 8, 18, 19, 26, 27, 42, 46, 47, 52, 77 and 78.

-	4th order IMD may fall into Rx frequencies of bands 7, 11, 21, 24, 32, 38, 41, 45, 46, 47, 50, 51, 69, 74, 75, 76, 90, 91, 92, 93 and 94.

-	5th order IMD may fall into Rx frequencies of bands 77 and 79.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.21.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.21.2-2: 2UL Band 3 + Band n7 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_3_n7.

5.21.3	∆TIB and ∆RIB values

Table 5.21.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-32_n7 | 0.7 | 0 | 0.7 |

5.21.4	Reference sensitivity exceptions

Table 5.21.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-32A_n7A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 32 | N/A | N/A | N/A | 1470 | 10.5 | IMD4 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |

5.22	DC_8-28_n3

5.22.1	Configurations for DC

Table 5.22.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A-28A_n3A | DC_8A_n3A DC_28A_n3A |

5.22.2	Co-existence studies

Table 5.22.2-1 lists the Band 8A + Band n3A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.22.2-1: Band 8 and Band n3 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 795 – 905 |  |  | 2590 – 2700 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 25 – 120 |  |  | 2505 – 2690 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3470 – 3615 |  |  | 4300 – 4485 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 850 – 945 |  |  | 1700 – 1795 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 855 – 1035 |  |  | 4215 – 4475 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1590 – 1810 |  |  | 5180 – 5400 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4350 – 4530 |  |  | 6010 – 6270 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5925 – 6260 |  |  | 1735 – 1950 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3300 – 3595 |  |  | 675 – 930 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 – 8055 |  |  | 5230 – 5445 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 – 7185 |  |  | 6060 – 6315 |  |  |

Based on Table 5.22.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 3 and 46.

-	3rd order harmonics may fall into Rx frequencies of bands 7, 22, 41, 42, 48, 49, 77, 78 and 90.

-	2nd order IMD may fall into Rx frequencies of bands 5, 6, 7, 18, 19, 20, 26, 27, 28, 38, 41, 44, 69 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 7, 22, 38, 41, 42, 43, 48, 49, 69, 77, 78, 79 and 90.

-	4th order IMD may fall into Rx frequencies of bands 3, 5, 6, 8, 18, 19, 26, 27, 46 and 79.

-	5th order IMD may fall into Rx frequencies of bands 2, 3, 5, 6, 8, 9, 12, 13, 14, 17, 18, 19, 20, 22, 25, 26, 27, 28, 29, 33, 35, 36, 37, 39, 42, 44, 46, 48, 49, 52, 67, 68, 77, 78 and 85.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.22.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.22.2-2: 2UL Band 8 + Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, IMD4, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_8_n3.

Table 5.22.2-3 lists the Band 28A + Band n3A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.22.2-3: Band 28 and Band n3 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1496 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2244 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 962 – 1082 |  |  | 2413 – 2533 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 214 – 379 |  |  | 2672 – 2867 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3116 – 3281 |  |  | 4123 – 4318 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 673 – 778 |  |  | 1690 – 1805 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 324 – 534 |  |  | 4382 – 4652 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1924 – 2164 |  |  | 4826 – 5066 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 – 4029 |  |  | 5833 – 6103 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6092 – 6437 |  |  | 1027 – 1282 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3634 – 3949 |  |  | 1176 – 1461 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 – 7888 |  |  | 4522 – 4777 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 – 6851 |  |  | 5529 – 5814 |  |  |

Based on Table 5.22.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 11, 21, 32, 45, 46, 50, 51, 74, 75, 76, 91, 92, 93 and 94.

-	3rd order harmonics may fall into Rx frequencies of bands 1, 4, 10, 22, 23, 42, 48, 49, 65, 66, 77 and 78.

-	2nd order IMD may fall into Rx frequencies of bands 41, 53 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 7, 41, 77 and 90.

-	4th order IMD may fall into Rx frequencies of bands 1, 2, 4, 10, 25, 31, 34, 36, 37, 46, 47, 65, 66, 70, 72, 73, 77, 79, 87 and 88.

-	5th order IMD may fall into Rx frequencies of bands 32, 43, 45, 46, 48, 49, 50, 51, 75, 76, 77, 78, 79, 91, 92, 93 and 94.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.22.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.22.2-4: 2UL Band 28 + Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_28_n3.

5.22.3	∆TIB and ∆RIB values

Table 5.22.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8-28_n3 | 0.6 | 0.5 | 0.3 |

Table 5.22.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8-28_n3 | 0.2 | 0.1 | 0 |

5.22.4	Reference sensitivity exceptions

Table 5.22.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-28A_n3A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 30.4 | IMD24 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.23	DC_20-32_n7

5.23.1	Configurations for DC

Table 5.23.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_20A-32A_n7A | DC_20A_n7A |

5.23.2	Co-existence studies

Table 5.23.2-1 lists the Band 20A + Band n7A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.23.2-1: Band 20 and Band n7 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1638 – 1738 |  |  | 3332 – 3432 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 776 – 906 |  |  | 4138 – 4308 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4164 – 4294 |  |  | 5832 – 6002 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 782 – 912 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 – 86 |  |  | 6638 – 6878 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3276 – 3476 |  |  | 6664 – 6864 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 – 5156 |  |  | 8332 – 8572 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9138 – 9448 |  |  | 758 – 948 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5776 – 6046 |  |  | 2414 – 2644 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 – 11142 |  |  | 5828 – 6018 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 – 9434 |  |  | 7496 – 7726 |  |  |

Based on Table 5.23.2-1,

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69 and 90.

-	2nd order IMD may fall into Rx frequencies of bands 42, 52, 77 and 78.

-	3rd order IMD may fall into Rx frequencies of bands 5, 6, 18, 19, 20, 26, 27, 28, 44, 46, 47, 68 and 77.

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77, 78 and 79.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 7, 8, 14, 18, 19, 20, 26, 27, 28, 38, 41, 44, 46, 47, 53, 68, 69 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.23.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.23.2-2: 2UL Band 20 + Band n7 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_20_n7.

5.23.3	∆TIB and ∆RIB values

Table 5.23.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_20-32_n7 | 0.3 | 0 | 0.7 |

5.23.4	Reference sensitivity exceptions

No addition exceptions required compared to fallbacks.

5.24	DC_7-8_n7

5.24.1	Configurations for DC

Table 5.24.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink configuration |
| --- | --- |
| DC_7A-8A_n7A | DC_7A_n7A DC_8A_n7A |

5.24.2	Co-existence studies

For UL configuration of DC_7A_n7A, only single UL is required. Thus no IMD issue needs to be evaluated. Besides, there is no harmonic issue from DC_7A_n7A to Band 8 DL reception.

For UE coexistence study of Band 8 + Band n7, the 2nd, 3rd and 4th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.24.2-1.

Table 5.24.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 10000 | 10280 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1585 | 1690 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_high – fy_low¦ | ¦2*fx_low – fy_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 670 | 810 | 4085 | 4260 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4260 | 4400 | 5880 | 6055 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 70 | 245 | 6585 | 6830 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5315 | 8380 | 8625 |
| Two-tone 4th order IMD products | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low - 2* fy_high¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3170 | 3310 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fx_low – 4*fy_high¦ | ¦fy_high – 4*fx_low¦ | ¦fy_low – 4*fx_high¦ |
| IMD frequency limits (MHz) | 9085 | 9400 | 950 | 1160 |
| Two-tone 5th order IMD products | ¦2*fx_high - 3*fy_low¦ | ¦2*fx_low - 3*fy_high¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5670 | 5950 | 2255 | 2500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11195 | 6020 | 6230 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9260 | 9540 | 7640 | 7885 |

As we can see from the above table, there could be MSD due to IMD:

-	The 5th order IMD generated by Band 8 and Band n7 may fall into DL reception frequency of Band 8.

But when we consider the implementation of Band 8 duplexer, we can find that such 5th order IMD will not impact the DL reception of Band 8.

The 3rd order harmonic generated by Band 8 will fall into DL reception frequency of Band n7.

5.24.3	∆TIB and ∆RIB values

For DC_7-8_n7, the TIB,c and RIB,c values are given in the following tables.

Table 5.24.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-8_n7 | 0.3 | 0.6 | 0.3 |

Table 5.24.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-8_n7 | 0 | 0.2 | 0 |

5.24.4	Reference sensitivity exceptions

Compared to its fallback modes, no additional MSD requirements for this band combination are needed.

5.25	DC_3-5_n40

### 5.25.1	Configurations for DC

Table 5.25.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_3A-5A_n40A | DC_3A_n40A DC_5A_n40A |

5.25.2	Co-existence studies

For UE coexistence study of Band 3 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.25.2-1

Table 5.25.2-1: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 11500 | 12000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 515 | 690 | 4010 | 4185 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 102 | 1270 | 2815 | 3090 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5720 | 5970 | 6310 | 6585 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2730 | 3055 | 5115 | 5490 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1380 | 1030 | 8020 | 8370 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7430 | 7755 | 8610 | 8985 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7890 | 7415 | 4840 | 4440 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3780 | 3330 | 755 | 330 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10910 | 11385 | 9140 | 9540 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10320 | 10770 | 9730 | 10155 |

For UE coexistence study of Band 5 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.25.2-2

Table 5.25.2-2: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 824 | 849 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 11500 | 12000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1451 | 1576 | 3124 | 3249 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 752 | 602 | 3751 | 3976 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3948 | 4098 | 5424 | 5649 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 72 | 247 | 6051 | 6376 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3152 | 2902 | 6248 | 6498 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4772 | 4947 | 7724 | 8049 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8776 | 8351 | 1096 | 896 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5552 | 5202 | 2053 | 2328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10024 | 10449 | 5596 | 5796 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8548 | 8898 | 7072 | 7347 |

The Rx impacts can be identified as below.

- No IMD products generated by DC_3_n40 uplink fall into own Rx of band 5.

- No IMD products generated by DC_5_n40 uplink fall into own Rx of band 3.

### 5.25.3	∆TIB and ∆RIB values

For DC_3-5_n40, the TIB,c and RIB,c values are reused from DC_3-8_n40 which is similar

Table 5.25.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-5_n40 | 0.5 | 0.3 | 0.5 |

Table 5.25.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-5_n40 | 0 | 0 | 0 |

### 5.25.4	Reference sensitivity exceptions

No additional MSD requirements need to be defined.

## 5.26	DC_1-28_n38

5.26.1	Configuration for DC

Table 5.26.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-28A_n38A | DC_1A_n38A DC_28A_n38A | CA_1A-28A | n38A |

5.26.2	Co-existence studies

There are IMD5 impact from UL 1_n38 affecting DL band 28.

There are no IMD impact from UL 28_n38 affecting DL band 1.

### 5.26.3		∆TIB and ∆RIB values

For DC_1-28_n38, the TIB,c and RIB,c values are reused from DC_1-28_n7 and are given in the tables below.

Table 5.1.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-28_n38 | 0.5 | 0.6 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.1.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-28_n38 | - | 0.2 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.26.4	REFSENS requirements

MSD values are reused from DC_1A-28A_n7A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-28A_n38A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 28 | 710 | 5 | 25 | 765 | 4.5 | IMD5 |
|  | n38 | 2580 | 5 | 25 | 2580 | N/A | N/A |

## 5.27	DC_3-28_n38

5.27.1	Configuration for DC

Table 5.27.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-28A_n38A | DC_3A_n38A DC_28A_n38A | CA_3A-28A | n38A |

5.27.2	Co-existence studies

There are IMD2 and IMD3 impact from UL 3_n38 affecting DL band 28.

There are IMD2 impact from UL 28_n38 affecting DL band 3.

### 5.27.3		∆TIB and ∆RIB values

For DC_3-28_n38, the TIB,c and RIB,c values are reused from DC_3-28_n7 and are given in the tables below.

Table 5.1.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-28_n38 | 0.5 | 0.3 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.1.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-28_n38 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.27.4	REFSENS requirements

MSD values are reused from DC_3A-28A_n7A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-28A_n38A | 3 | 1775 | 5 | 25 | 1870 | 26.0 | IMD2 |
|  | 28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | n38 | 2580 | 5 | 25 | 2580 | N/A | N/A |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 20.0 | IMD21 |
|  | n38 | 2580 | 5 | 25 | 2580 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

5.28	DC_20-28_n78

5.28.1	Configurations for DC

Table 5.28.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_20A-28A_n78A | DC_20A_n78A DC_28A_n78A |

5.28.2	Co-existence studies

Table 5.28.2-1 lists the Band 20A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.28.2-1: Band 20 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2438 – 2968 |  |  | 4132 – 4662 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1576 – 2136 |  |  | 5738 – 6768 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4964 – 5524 |  |  | 7432 – 8462 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 732 – 962 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 714 – 1304 |  |  | 9038 – 10568 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 4876 – 5936 |  |  | 8264 – 9324 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 – 6386 |  |  | 10732 – 12262 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12338 – 14368 |  |  | 148 – 472 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8176 – 9736 |  |  | 4014 – 5104 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 – 16062 |  |  | 6628 – 7248 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 – 13124 |  |  | 9096 – 10186 |  |  |

Based on Table 5.28.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 38, 41, 69 and 90.

-	2nd order IMD may fall into Rx frequencies of bands 7, 38, 41, 53, 69, 77, 79 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 25, 33, 34, 35, 36, 37, 39, 46, 47, 65, 66, 70 and 79.

-	4th order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 44, 46, 47, 67, 68, 79 and 85.

-	5th order IMD may fall into Rx frequencies of bands 31, 72, 73, 77, 79, 87 and 88.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.28.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.28.2-2: 2UL Band 20 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, IMD4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD4 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_20_n78.

Table 5.28.2-3 lists the Band 28A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.28.2-3: Band 28 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1496 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2244 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2552 – 3097 |  |  | 4003 – 4548 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1804 – 2394 |  |  | 5852 – 6897 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4706 – 5296 |  |  | 7303 – 8348 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 603 – 848 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1056 – 1691 |  |  | 9152 – 10697 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 5104 – 6194 |  |  | 8006 – 9096 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 – 6044 |  |  | 10603 – 12148 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12452 – 14497 |  |  | 308 – 988 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8404 – 9994 |  |  | 4356 – 5491 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 – 15948 |  |  | 6112 – 6792 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 – 12896 |  |  | 8709 – 9844 |  |  |

Based on Table 5.28.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 1, 4, 10, 23, 65 and 66.

-	3rd order harmonics may fall into Rx frequencies of bands 11, 21, 32, 45, 50, 51, 74, 75, 76, 91, 92, 93 and 94.

-	2nd order IMD may fall into Rx frequencies of bands 7, 38, 41, 69, 77, 79 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 39, 40, 46, 47, 65, 66, 70 and 79.

-	4th order IMD may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 46, 47, 47, 50, 51, 74, 75, 76, 91, 92, 93 and 94.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 46, 67, 68, 71, 72, 73, 79, 85, 87 and 88.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.28.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.28.2-4: 2UL Band 28 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, IMD4, IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4, IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_28_n78.

5.28.3	∆TIB and ∆RIB values

Table 5.28.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_20-28_n78 | 0.6 | 0.5 | 0.8 |

Table 5.28.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_20-28_n78 | 0 | 0.2 | 0.5 |

5.28.4	Reference sensitivity exceptions

Table 5.284-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-28A_n78A | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | 28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 20 | 849 | 5 | 25 | 808 | 3.8 | IMD5 |
|  | 28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |

5.29	DC_1-28_n20

5.29.1	Configurations for DC

Table 5.29.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-28A_n20A22 | DC_1A_n20A DC_28A_n20A22 |
| NOTE 22: The frequency range in band 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758 - 788 MHz for the DL. |  |

5.29.2	Co-existence studies

Table 5.29.2-1 lists the Band 1A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.29.2-1: Band 1 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1058 – 1148 |  |  | 2752 – 2842 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2978 – 3128 |  |  | 196 – 316 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4672 – 4822 |  |  | 3584 – 3704 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 – 2000 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 – 5108 |  |  | 516 – 666 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2116 – 2296 |  |  | 5504 – 5684 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 – 6802 |  |  | 4416 – 4566 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1348 – 1528 |  |  | 6818 – 7088 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 – 1464 |  |  | 4036 – 4276 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 – 5428 |  |  | 8512 – 8782 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 – 6546 |  |  | 7424 – 7664 |  |  |

Based on Table 5.29.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 46 and 47.

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69, 77 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77, 78 and 79.

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 23, 46, 65, 66, 71 and 79.

-	5th order IMD may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 50, 51, 74, 75, 76, 77, 91, 92, 93 and 94.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.29.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.29.2-2: 2UL Band 1 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_1_n20.

Table 5.29.2-3 lists the Band 28A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.29.2-3: Band 28 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 99 – 159 |  |  | 1535 – 1595 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 544 – 634 |  |  | 931 – 1021 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2238 – 2328 |  |  | 2367 – 2457 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1247 – 1367 |  |  | 1763 – 1883 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 198 – 318 |  |  | 3070 – 3190 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2941 – 3061 |  |  | 3199 – 3319 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2595 – 2745 |  |  | 1950 – 2100 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1030 – 1180 |  |  | 385 – 535 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4031 – 4181 |  |  | 3644 – 3794 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3902 – 4052 |  |  | 3773 – 3923 |  |  |

Based on Table 5.29.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 32, 38, 41, 45, 50, 51, 69, 75, 76, 90, 91, 92, 93 and 94.

-	3rd order harmonics may fall into Rx frequencies of bands 1, 4, 10, 23, 65 and 66.

-	2nd order IMD may fall into Rx frequencies of band 24.

-	3rd order IMD may fall into Rx frequencies of bands 8, 40 and 71.

-	4th order IMD may fall into Rx frequencies of bands 3, 9, 35, 39, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 2, 7, 25, 31, 34, 36, 38, 41, 43, 48, 49, 69, 70, 72, 73, 77, 78, 87, 88 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.29.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.29.2-4: 2UL Band 28 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

The requirements for spurious emission band UE coexistence that exist in 38.101-3 for DC_20_n28 can be reused.

5.29.3	∆TIB and ∆RIB values

Table 5.29.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-28_n20 | 0.3 | 0.6 | 0.6 |

Table 5.29.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-28_n20 | 0 | 0.2 | 0.2 |

5.29.4	Reference sensitivity exceptions

No additional sensitivity exceptions required compared to fallbacks.

5.30	DC_1-5_n40

### 5.30.1	Configurations for DC

Table 5.30.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_1A-5A_n40A | DC_1A_n40A DC_5A_n40A |

### 5.30.2	Co-existence studies

For UE coexistence study of Band 1 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.30.2-1

Table 5.30.2-1: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 11500 | 12000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 | 480 | 4220 | 4380 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1660 | 2620 | 2880 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 | 6360 | 6520 | 6780 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 3640 | 4920 | 5280 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 960 | 640 | 8440 | 8760 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 | 8340 | 8820 | 9180 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 | 7220 | 5620 | 5280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 2940 | 1340 | 960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 | 11580 | 9980 | 10320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 | 11160 | 10360 | 10740 |

For UE coexistence study of Band 5 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.30.2-2

Table 5.216.2-2: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 824 | 849 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 11500 | 12000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1451 | 1576 | 3124 | 3249 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 752 | 602 | 3751 | 3976 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3948 | 4098 | 5424 | 5649 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 72 | 247 | 6051 | 6376 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3152 | 2902 | 6248 | 6498 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4772 | 4947 | 7724 | 8049 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8776 | 8351 | 1096 | 896 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5552 | 5202 | 2053 | 2328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10024 | 10449 | 5596 | 5796 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8548 | 8898 | 7072 | 7347 |

The Rx impacts can be identified as below.

- Co-existence analysis for DC_1_n40 UL shows that 4th IMD may fall in DL band 5.

- Co-existence analysis for DC_5_n40 UL shows that 5th IMD may fall in DL band 1.

### 5.30.3	∆TIB and ∆RIB values

For DC_1-5_n40, the TIB,c and RIB,c values are reused from DC_1-28_n40 which is very similar

Table 5.30.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-5_n40 | 0.6 | 0.3 | 0.5 |

Table 5.30.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-5_n40 | 0 | 0.2 | 0 |

### 5.30.4	Reference sensitivity exceptions

The MSD requirement due to 5th order IMD for DC_5-66_n30 can be reused for DC_1-5_n40.

The MSD requirement due to 4th order IMD for DC_1-8_n40 can be reused for DC_1-5_n40.

Table 6.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-5A_n40A | 1 | 1954 | 5 | 25 | 2144 | 4.0 | IMD5 |
|  | 5 | 832 | 5 | 25 | 877 | N/A | N/A |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | N/A |
|  | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | 8.0 | IMD4 |
|  | n40 | 2385 | 5 | 25 | 2385 | N/A | N/A |

5.31	DC_1-3_n1

### 5.31.1	Configurations for DC

Table 5.31.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n1A | DC_1A_n1A2 DC_3A_n1A |
| NOTE 2: Only single switched UL is supported. |  |

### 5.31.2	Co-existence studies

Table 5.31.2-1 lists the Band 3A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.31.2-1: Band 3 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3420 – 3570 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5130 – 5355 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 135 – 270 |  |  | 3630 – 3765 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1440 – 1650 |  |  | 2055 – 2250 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5340 – 5550 |  |  | 5550 – 5745 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1660 – 1835 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3150 – 3435 |  |  | 3975 – 4230 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 270 – 540 |  |  | 7260 – 7530 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7050 – 7335 |  |  | 7470 – 7725 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5895 – 6210 |  |  | 4860 – 5220 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2190 – 2520 |  |  | 1170 – 1515 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 – 9705 |  |  | 8760 – 9120 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 – 9510 |  |  | 8970 – 9315 |  |  |

Based on Table 5.31.2-1,

-	2nd order harmonics may fall into Rx frequencies of band 22, 42, 46, 47, 48, 49, 77 and 78.

-	3rd order harmonics may fall into Rx frequencies of band 46 and 77

-	2nd order IMD may fall into Rx frequencies of bands 43, 48, 49, 77 and 78.

-	3rd order IMD may fall into Rx frequencies of bands 1, 4, 10, 11, 21, 23, 24, 32, 45, 46, 50, 65, 66, 74, 75, 92 and 94.

-	4th order IMD may fall into Rx frequencies of bands 31, 42, 52, 72, 73, 77, 78, 87 and 88

-	5th order IMD may fall into Rx frequencies of bands 11, 21, 23, 30, 32, 40, 41, 45, 46, 47, 50, 51, 53, 65, 66, 74, 75, 76, 79, 90, 91, 92, 93 and 94.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.31.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.31.2-2: 2UL Band 3 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, 3rd harmonic, IMD3, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 3rd harmonic, IMD3, IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD3 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, 3rd harmonic, IMD3, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_3_n1.

### 5.31.3	∆TIB and ∆RIB values

Table 5.31.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-3_n1 | 0.3 | 0.3 | 0.3 |

### 5.31.4	Reference sensitivity exceptions

Table 5.31.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2150 | 23 | IMD3 |

5.32	DC_1-20_n1

### 5.32.1	Configurations for DC

Table 5.32.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-20A_n1A | DC_1A_n1A2 DC_20A_n1A |
| NOTE 2: Only single switched UL is supported. |  |

### 5.32.2	Co-existence studies

Table 5.32.2-1 lists the Band 20A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.32.2-1: Band 20 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1058 – 1148 |  |  | 2752 – 2842 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 196 – 316 |  |  | 2978 – 3128 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3584 – 3704 |  |  | 4672 – 4822 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 782 – 912 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 516 – 666 |  |  | 4898 – 5108 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2116 – 2296 |  |  | 5504 – 5684 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4416 – 4566 |  |  | 6592 – 6802 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6818 – 7088 |  |  | 1348 – 1528 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4036 – 4276 |  |  | 1254 – 1464 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8512 – 8782 |  |  | 5248 – 5428 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7424 – 7664 |  |  | 6336 – 6546 |  |  |

Based on Table 5.32.2-1,

-	2nd order harmonics may fall into Rx frequencies of band 46 and 47.

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69, 77 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77, 78 and 79.

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 23, 46, 65, 66, 71 and 79.

-	5th order IMD may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 50, 51, 74, 75, 76, 77, 91, 92, 93 and 94.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.32.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.32.2-2: 2UL Band 20 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_20_n1.

### 5.32.3	∆TIB and ∆RIB values

Table 5.32.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-20_n1 | 0.3 | 0.3 | 0.3 |

### 5.32.4	Reference sensitivity exceptions

Table 5.32.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-20A_n1A | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2160 | 6 | IMD4 |

5.33	DC_7-8_n20

### 5.33.1	Configurations for DC

Table 5.33.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A-8A_n20A | DC_7A_n20A DC_8A_n20A |

### 5.33.2	Co-existence studies

Table 5.33.2-1 lists the Band 7A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.33.2-1: Band 7 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1638 – 1738 |  |  | 3332 – 3432 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 4138 – 4308 |  |  | 776 – 906 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5832 – 6002 |  |  | 4164 – 4294 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2480 – 2590 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6638 – 6878 |  |  | 74 – 86 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3276 – 3476 |  |  | 6664 – 6864 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8332 – 8572 |  |  | 4996 – 5156 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 758 – 948 |  |  | 9138 – 9448 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2414 – 2644 |  |  | 5776 – 6046 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5828 – 6018 |  |  | 10832 – 11142 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7496 – 7726 |  |  | 9164 – 9434 |  |  |

Based on Table 5.33.2-1,

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69 and 90.

-	2nd order IMD may fall into Rx frequencies of bands 42, 52, 77 and 78.

-	3rd order IMD may fall into Rx frequencies of bands 5, 6, 18, 19, 20, 26, 27, 28, 44, 46, 47, 68 and 77.

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77, 78 and 79.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 7, 8, 14, 18, 19, 20, 26, 27, 28, 38, 41, 44, 46, 47, 53, 68, 69 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.33.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.33.2-2: 2UL Band 7 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_7_n20.

Table 5.33.2-3 lists the Band 8A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.33.2-3: Band 8 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 18 – 83 |  |  | 1712 – 1777 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 898 – 998 |  |  | 749 – 844 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2592 – 2692 |  |  | 2544 – 2639 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 860 – 935 |  |  | 822 – 872 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778 – 1913 |  |  | 1581 – 1706 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 36 – 166 |  |  | 3424 – 3554 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472 – 3607 |  |  | 3376 – 3501 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2413 – 2568 |  |  | 2658 – 2828 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 666 – 826 |  |  | 916 – 1081 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208 – 4363 |  |  | 4352 – 4522 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256 – 4416 |  |  | 4304 – 4469 |  |  |

Based on Table 5.33.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 7, 41 and 90.

-	3rd order harmonics may fall into Rx frequencies of bands 3, 38, 41, 69 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 7, 8, 13, 14, 20, 28, 38, 38, 41, 44, 67, 68, 69 and 90.

-	4th order IMD may fall into Rx frequencies of bands 3, 9, 22, 33, 35, 37, 39, 42, 43, 48, 49, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 7, 8, 12, 13, 14, 17, 20, 28, 29, 41, 44, 53, 67, 68, 79, 85 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.33.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.33.2-4: 2UL Band 8 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_8_n20.

### 5.33.3	∆TIB and ∆RIB values

Table 5.33.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-8_n20 | 0.3 | 0.6 | 0.6 |

Table 5.33.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-8_n20 | 0 | 0.2 | 0.2 |

### 5.33.4	Reference sensitivity exceptions

Table 5.33.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-8A_n20A | 7 | 2520 | 5 | 25 | 2640 | 21.1 | IMD34,15 |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 7 | 2503 | 5 | 25 | 2623 | N/A | N/A |
|  | n20 | 859 | 5 | 25 | 818 | N/A | N/A |
|  | 8 | N/A | 5 | N/A | 933 | 4.4 | IMD5 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 15: This band is subject to additional IMD3 for which MSD is not specified. |  |  |  |  |  |  |  |

5.34	DC_7-28_n20

### 5.34.1	Configurations for DC

Table 5.34.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A-28A_n20A | DC_7A_n20A DC_28A_n20A |

### 5.34.2	Co-existence studies

Table 5.34.2-1 lists the Band 7A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.34.2-1: Band 7 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1638 – 1738 |  |  | 3332 – 3432 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 4138 – 4308 |  |  | 776 – 906 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5832 – 6002 |  |  | 4164 – 4294 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2480 – 2590 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6638 – 6878 |  |  | 74 – 86 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3276 – 3476 |  |  | 6664 – 6864 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8332 – 8572 |  |  | 4996 – 5156 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 758 – 948 |  |  | 9138 – 9448 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2414 – 2644 |  |  | 5776 – 6046 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5828 – 6018 |  |  | 10832 – 11142 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7496 – 7726 |  |  | 9164 – 9434 |  |  |

Based on Table 5.34.2-1,

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69 and 90.

-	2nd order IMD may fall into Rx frequencies of bands 42, 52, 77 and 78.

-	3rd order IMD may fall into Rx frequencies of bands 5, 6, 18, 19, 20, 26, 27, 28, 44, 46, 47, 68 and 77.

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77, 78 and 79.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 7, 8, 14, 18, 19, 20, 26, 27, 28, 38, 41, 44, 46, 47, 53, 68, 69 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.34.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.34.2-2: 2UL Band 7 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_7_n20.

Table 5.34.2-3 lists the Band 28A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.34.2-3: Band 28 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1496 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2244 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 84 – 159 |  |  | 1535 – 1610 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 544 – 664 |  |  | 916 – 1021 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2238 – 2358 |  |  | 2367 – 2472 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 768 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1247 – 1412 |  |  | 1748 – 1883 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 168 – 318 |  |  | 3070 – 3220 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2941 – 3106 |  |  | 3199 – 3334 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2580 – 2745 |  |  | 1950 – 2160 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1000 – 1180 |  |  | 385 – 580 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4031 – 4196 |  |  | 3644 – 3854 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3902 – 4082 |  |  | 3773 – 3968 |  |  |

Based on Table 5.34.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 11, 21, 32, 38, 41, 45, 50, 51, 69, 74, 75, 76, 90, 91, 92, 93 and 94.

-	3rd order harmonics may fall into Rx frequencies of bands 1, 4, 10, 23, 65 and 66.

-	2nd order IMD may fall into Rx frequencies of band 24.

-	3rd order IMD may fall into Rx frequencies of bands 8, 30, 40 and 71.

-	4th order IMD may fall into Rx frequencies of bands 3, 9, 35, 39, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 1, 2, 4, 7, 10, 25, 31, 34, 36, 38, 41, 43, 48, 49, 65, 66, 69, 70, 72, 73, 77, 78, 87, 88 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.34.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.34.2-4: 2UL Band 28 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

The requirements for spurious emission band UE coexistence that exist in 38.101-3 for DC_20_n28 can be reused.

### 5.34.3	∆TIB and ∆RIB values

Table 5.34.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-28_n20 | 0.3 | 0.6 | 0.6 |

Table 5.34.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-28_n20 | 0 | 0.2 | 0.2 |

5.34.4	Reference sensitivity exceptionsTable 5.344-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-28A_n20A | 7 | 2520 | 5 | 25 | 2640 | 5.9 | IMD5 |
|  | 28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | n20 | 842 | 5 | 25 | 801 | N/A | N/A |
|  | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |
|  | n20 | 859 | 5 | 25 | 818 | N/A | N/A |
|  | 28 | N/A | 5 | N/A | 787 | 17.4 | IMD3 |

5.35	DC_5-7_n40

5.35.1	Configurations for DC

Table 5.35.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-7A_n40A | DC_5A_n40A DC_7A_n40A |

5.35.2	Co-existence studies

For UE coexistence study of Band 5 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 5.35.2-1.

Based on this calculation, we find that there is no impact of harmonics and inter-modulation products from UL DC_5_n40 affecting DL Band 7.

Table 5.35.2-1: Harmonics and IMD analysis of Band 5 + Band n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 11500 | 12000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1451 | 1576 | 3124 | 3249 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 752 | 602 | 3751 | 3976 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3948 | 4098 | 5424 | 5649 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 72 | 247 | 6051 | 6376 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3152 | 2902 | 6248 | 6498 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4772 | 4947 | 7724 | 8049 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8776 | 8351 | 1096 | 896 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5552 | 5202 | 2053 | 2328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10024 | 10449 | 5596 | 5796 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8548 | 8898 | 7072 | 7347 |

For UE coexistence study of Band 7 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 5.35.2-2.

Based on this calculation, we find that there is no impact of harmonics and inter-modulation products from UL DC_7_n40 affecting DL Band 5.

Table 5.35.2-2: Harmonics and IMD analysis of Band 7 + Band n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 11500 | 12000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 100 | 4800 | 4970 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2600 | 2840 | 2030 | 2300 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7300 | 7540 | 7100 | 7370 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5100 | 5410 | 4330 | 4700 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 200 | 540 | 9600 | 9940 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9800 | 10110 | 9400 | 9770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7100 | 6630 | 7980 | 7600 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2200 | 1760 | 3110 | 2700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11700 | 12170 | 12300 | 12680 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11900 | 12340 | 12100 | 12510 |

5.35.3	∆TIB and ∆RIB values

For DC_5-7_n40, TIB,c and RIB,c values are given as follows.

Table 5.35.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5-7_n40 | 0.3 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.35.3-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5-7_n40 | 0.2 | 0.3 | 0.7 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.35.4	Reference sensitivity exceptions

Based on the coexistence study results, there is no need to define additional MSD requirements for this band combination.

5.36	DC_20-(n)3

### 5.36.1	Operating bands for EN-DC

Table 5.36.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_20-(n)3 | CA_3-20 | n3 | DC_(n)3 |

5.36.2	Configuration for DC

Table 5.36.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-(n)3AA | DC_(n)3AA2 DC_20A_n3A | CA_3A-20A | n3A |
| NOTE 2: Only single switched UL is supported |  |  |  |

### 5.36.3		∆TIB and ∆RIB values

For DC_20-(n)3, the TIB,c and RIB,c values are reused from DC_20_n3 and are given in the tables below.

Table 5.36.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_20-(n)3 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.36.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_20-(n)3 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.36.4	REFSENS requirements

There are IMD4 impact from UL 20_n3 affecting DL band 3.

MSD value band n3 is derived from DC_3_n20.

A similar approach to the effect on SCell as DC_3_(n)7 (see R4-2216086) has been used for MSD value band 3.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-(n)3AA | 3 | N/A | 5 | N/A | 1865 | 3 | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |

5.37	DC_3-8_n7

5.37.1	Configurations for DC

Table 5.37.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink configuration |
| --- | --- |
| DC_3A-8A_n7A | DC_3A_n7A DC_8A_n7A |

5.37.2	Co-existence studies

For UE coexistence study of Band 3 + Band n7, the 2nd, 3rd and 4th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.37.2-1.

Table 5.37.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 10000 | 10280 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low - 2* fy_high¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1430 | 1570 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fx_low – 4*fy_high¦ | ¦fy_high – 4*fx_low¦ | ¦fy_low – 4*fx_high¦ |
| IMD frequency limits (MHz) | 8215 | 8570 | 4270 | 4640 |
| Two-tone 5th order IMD products | ¦2*fx_high - 3*fy_low¦ | ¦2*fx_low - 3*fy_high¦ | ¦2*fy_high -3*fx_low¦ | ¦2*fy_low - 3*fx_high¦ |
| IMD frequency limits (MHz) | 3930 | 4290 | 10 | 355 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

As we can see from the above table, there could be MSD due to IMD:

-	The 3rd order IMD generated by Band 3 and Band n7 may fall into DL reception frequency of Band 8.

-	The 4th order IMD generated by Band 3 and Band n7 may fall into DL reception frequency of Band n7.The MSD value for the 3rd order IMD can reuse what have been specified in TS 36.101 Table 7.3.1A-0g for CA_3A-7A-8A. For the 4th order IMD, it has already been studied with its fallback mode.

For UE coexistence study of Band 8 + Band n7, the 2nd, 3rd and 4th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.37.2-2.

Table 5.37.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 10000 | 10280 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1585 | 1690 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_high – fy_low¦ | ¦2*fx_low – fy_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 670 | 810 | 4085 | 4260 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4260 | 4400 | 5880 | 6055 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 70 | 245 | 6585 | 6830 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5315 | 8380 | 8625 |
| Two-tone 4th order IMD products | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low - 2* fy_high¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3170 | 3310 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fx_low – 4*fy_high¦ | ¦fy_high – 4*fx_low¦ | ¦fy_low – 4*fx_high¦ |
| IMD frequency limits (MHz) | 9085 | 9400 | 950 | 1160 |
| Two-tone 5th order IMD products | ¦2*fx_high - 3*fy_low¦ | ¦2*fx_low - 3*fy_high¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5670 | 5950 | 2255 | 2500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11195 | 6020 | 6230 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9260 | 9540 | 7640 | 7885 |

As we can see from the above table, there could be MSD due to IMD:

-	The 5th order IMD generated by Band 8 and Band n7 may fall into DL reception frequency of Band 8.

Besides, harmonic could be a cause for MSD:

-	The 2nd order harmonic generated by Band 8 may fall into DL reception frequency of Band 3.

-	The 3rd order harmonic generated by Band 8 may fall into DL reception frequency of Band n7.

The above IMD issue has been studied for DC_7-8_n7. Specifically, when we consider the implementation of Band 8 duplexer, we can find that such 5th order IMD will not impact the DL reception of Band 8.

The MSD values due to the above two harmonic issues have been specified as in TS 38.101-3 table 7.3B.2.3.1-1.

5.37.3	∆TIB and ∆RIB values

For DC_3-8_n7, the TIB,c and RIB,c values are given in the following tables. (Both are reusing the specified values for DC_3-7_n8 as in TS 38.101-3)

Table 5.37.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-8_n7 | 0.5 | 0.6 | 0.5 |

Table 5.37.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-8_n7 | 0 | 0.2 | 0 |

5.37.4	Reference sensitivity exceptions

The required MSD values are derived from CA_3A-7A-8A.

Table 5.37.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-8A_n7A | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | n7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 18.0 | IMD3 |

5.38	DC_3-67_n3

5.38.1	Configuration for DC

Table 5.38.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-67A_n3A | DC_3A_n3A2 | CA_3A-67A | n3 |
| NOTE 2: Only single switched UL is supported |  |  |  |

5.38.2	Co-existence studies

There are no IMD impact from UL 3_n3 affecting DL band 67.

### 5.38.3	∆TIB and ∆RIB values

For DC_3-67_n3, the TIB,c and RIB,c values are reused from CA_n3-n67 and are given in the tables below.

Table 5.1.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-67_n3 | - | - | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.1.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-67_n3 | 0.3 | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.38.4	REFSENS requirements

Based on the co-existence studies there are no need to define MSD values.

5.39	DC_67-(n)3

5.39.1	Configuration for DC

Table 5.39.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_67A-(n)3AA | DC_(n)3AA2 | CA_3A-67A | n3 |
| NOTE 2: Only single switched UL is supported |  |  |  |

5.39.2	Co-existence studies

There are no IMD impact.

### 5.39.3		∆TIB and ∆RIB values

For DC_67-(n)3, the TIB,c and RIB,c values are reused from CA_n3-n67 and are given in the tables below.

Table 5.1.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_67-(n)3 | - | - | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.1.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_67-(n)3 | 0.3 | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.39.4	REFSENS requirements

Based on the co-existence studies there are no need to define MSD values.

5.40	DC_20-67_n3

5.40.1	Configuration for DC

Table 5.40.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-67A_n3A | DC_20A_n3A | CA_20A-67A | n3 |

5.40.2	Co-existence studies

There are IMD4 impact from UL 20_n3 affecting DL band 67.

### 5.40.3		∆TIB and ∆RIB values

For DC_20-67_n3, the TIB,c and RIB,c values are reused from DC_20_n3-n67 and are given in the tables below.

Table 5.1.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_20-67_n3 | 0.5 | - | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.1.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_20-67_n3 | 0.1 | 0.1 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.40.4	REFSENS requirements

Based on the co-existence studies there is a need to define MSD value.

The MSD value for band n67 is reused from DC_20_n3-n67.

| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20-67_n3 | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | 67 | N/A | 5 | 25 | 746 | 9.4 | IMD4 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |

5.41		DC_3-8_n41

### 5.41.1 	Configuration for DC

Table 5.41.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-8A_n41A | DC_3A_n41A DC_8A_n41A |

### 5.41.2	 Co-existence studies

The harmonic and IMD products caused by UL DC_8_n41 are shown below.

Table 5.41.3-1: The harmonic and IMD products caused by UL DC_8_n41

| DL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 880 | 915 | 2496 | 2690 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1581 | 1810 | 3376 | 3605 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 930 | 666 | 4077 | 4500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4256 | 4520 | 5872 | 6295 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3620 | 3162 | 6752 | 7210 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 50 | 249 | 6573 | 7190 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5136 | 5435 | 8368 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9880 | 9069 | 1164 | 830 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10864 | 11675 | 6016 | 6350 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6310 | 5658 | 2247 | 2740 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9248 | 9900 | 7632 | 8125 |

The harmonic and IMD products caused by UL DC_3_n41 are shown below.

Table 5.41.3-2: The harmonic and IMD products caused by UL DC_3_n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 1710 | 1785 | 2496 | 2690 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1960 | 1422 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8412 | 8950 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

In light of the above co-existence table, we can observe:

Near miss IMD2 products caused by 8+n41 may fall into the its own band n3 Rx

IMD 2 and IMD3 caused by 3+n41 may fall into the its own band n8 Rx

### 5.41.4	 ∆TIB and ∆RIB values

Referring to DC_3A_n8A-n41A, ΔTIB,c and ΔRIB,c for DC_3A-n8A_n41A can be specified as below.

Table 5.41.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-8_n41 | 0.5 | 0.3 | 0.33/0.84 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.41.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-8_n41 | - | - | 03/0.54 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.41.5	 MSD requirements

For UL DC_8A_n41A, the IMD2 due to dual UL of band n41 10MHz CBW and B8 5MHz  CBW may partially overlap the lowest 5MHz DL channel of band 3 leading to a special MSD test point, where the IMD center does not fall into the center of the DL victim in the MSD test configuration.

The estimation of IMD2 interference power level ration between direct hit and adjacent hit for DC_3-8_n41 based on measurement of the IMD2 PSD. The IMD2 interference is approximately 0.9dB lower when integrated at 1807.5MHz/5MHz CBW relative to the direct hit interference level measured at 1802.5MHz/5MHz. This is due to the IMD2 PSD flatness.

Referring to DC_3A_n8A-n41A, the MSD requirement are defined in table 5.41.5-1:

Table 5.41.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ENDC band combination | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) |  |
| DC_3A-8A_n41A | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 26.0 | IMD215 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 25 | IMD2x |
|  | 8 | 882.5 | 5 | 25 | 927.5 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| NOTE 15: This band is subject to IMD3 also which MSD is not specified. NOTE x: In the MSD test configuration, the IMD center does not fall into the DL victim Fc. |  |  |  |  |  |  |  |

5.42	DC_2-4_n78

5.42.1	Operating bands for EN-DC

Table 5.42.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_2-4_n78 | CA_2-4 | n78 |

5.42.2	Configuration for DC

Table 5.42.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-4A_n78A | DC_2A_n78A DC_4A_n78A | CA_2A-4A | n78A |

### 5.42.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.42.3-1 for DC_2A_n78A into band 4 downlink.

Table 5.42.3-1: IMD analysis for DC_2A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1850 | 1910 | 3300 | 3800 |
| DL Frequency [MHz] | 1930 | 1990 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1390 | 1950 | 5150 | 5710 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 100 | 520 | 4690 | 5750 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 | 7620 | 8450 | 9510 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1750 | 2430 | 7990 | 9550 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3900 | 2780 | 10300 | 11420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850 | 9530 | 11750 | 13310 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13350 | 11290 | 4340 | 3600 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7700 | 6080 | 870 | 2050 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 17110 | 10700 | 11440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 15220 | 12150 | 13330 |

The analysis shows the 4th order IMD product falls inside band 4 downlink

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.42.3-2 for DC_4A_n78A into band 2 downlink.

Table 5.42.3-2: IMD analysis for DC_4A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1755 | 3300 | 3800 |
| DL Frequency [MHz] | 2110 | 2155 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1545 | 2090 | 5010 | 5555 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 210 | 4845 | 5890 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7310 | 8310 | 9355 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 1965 | 8145 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3090 | 10020 | 11110 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9065 | 11610 | 13155 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11445 | 3720 | 3040 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6390 | 1335 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16955 | 10140 | 10820 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14910 | 11730 | 12865 |

The analysis shows the 2nd, 4th and 5th order IMD product falls inside band 2 downlink.

Based on Table 5.42.3-1 and Table 5.42.3-2:

- The 4th order IMD product of DC_2_n78 falls inside band 4 downlink

- The 2nd, 4th and 5th order IMD product of DC_4_n78 falls inside band 2 downlink

### 5.42.4		∆TIB and ∆RIB values

For DC_2-4_n78, the TIB,c and RIB,c values are reused from DC_2-66_n78 and are given in the tables below.

Table 5.42.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-4_n78 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.42.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2-4_n78 | 0.3 | 0.3 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.42.5	REFSENS requirements

MSD values are reused from DC_2A-66A_n78A, but band 4 frequencies requires new test points.

Table 5.42.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-4A_n78A | 2 | 1875 | 5 | 25 | 1955 | N/A | N/A |
|  | 4 | 1745 | 5 | 25 | 2145 | 10.3 | IMD4 |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 4 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | 9.1 | IMD4 |
|  | 4 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 2 | 1870 | 5 | 25 | 1950 | 2.1 | IMD5 |
|  | 4 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | N/A |

5.43	DC_28-38_n78

5.43.1	Operating bands for EN-DC

Table 5.43.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_28-38_n78 | CA_28-38 | n78 | No |

5.43.2	Configuration for DC

Table 5.43.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_28A-38A_n78A | DC_28A_n78A DC_38A_n78A | CA_28A-38A | n78A |

### 5.43.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.43.3-1 for DC_28A_n78A into band 38 downlink.

Table 5.43.3-1: IMD analysis for DC_28A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 703 | 748 | 3300 | 3800 |
| DL Frequency [MHz] | 758 | 803 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3097 | 4003 | 4548 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 | 6897 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 | 8348 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 | 10697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 | 9096 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 | 12148 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 | 988 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 | 5491 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 | 9844 |

The analysis shows the 2nd order IMD product falls inside band 38 downlink.

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.43.3-2 for DC_38A_n78A into band 28 downlink.

Table 5.43.3-2: IMD analysis for DC_7A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2570 | 2620 | 3300 | 3800 |
| DL Frequency [MHz] | 2570 | 2620 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 680 | 1230 | 5870 | 6420 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1340 | 1940 | 3980 | 5030 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8440 | 9040 | 9170 | 10220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3910 | 4560 | 7280 | 8830 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2460 | 1360 | 11740 | 12840 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11010 | 11660 | 12470 | 14020 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12630 | 10580 | 7180 | 6480 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 4660 | 1260 | 110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15770 | 17820 | 13580 | 14280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15040 | 16640 | 14310 | 15460 |

The analysis shows the 2nd and 5th order IMD product falls inside band 28 downlink.

Based on Table 5.43.3-1 and Table 5.43.3-2:

- The 2nd order IMD product of DC_28_n78 falls inside band 38 downlink

- The 2nd and 5th order IMD product of DC_38_n78 falls inside band 28 downlink

### 5.43.4		∆TIB and ∆RIB values

For DC_28-38_n78, the TIB,c and RIB,c values are reused from DC_28-41_n78 and are given in the tables below.

Table 5.43.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28-38_n78 | 0.5 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.43.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28-38_n78 | 0.2 | 0.4 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.43.5	REFSENS requirements

MSD values are reused from DC_28A-41A_n78A. The test frequencies are not re-used.

Table 5.43.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A-38A_n78A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 38 | 2582 | 5 | 25 | 2582 | 29.5 | IMD2 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | 28 | 738 | 5 | 25 | 793 | 30.8 | IMD24 |
|  | 38 | 2582 | 5 | 25 | 2582 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | N/A | N/A |

5.44	DC_4-7_n78

5.44.1	Operating bands for EN-DC

Table 5.44.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_4-7_n78 | CA_4-7 | n78 | No |

5.44.2	Configuration for DC

Table 5.44.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_4A-7A_n78A | DC_4A_n78A DC_7A_n78A | CA_4A-7A | n78A |
| DC_4A-7C_n78A | DC_4A_n78A DC_7C_n78A | CA_4A-7C | n78A |

### 5.44.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.44.3-1 for DC_4A_n78A into band 7 downlink.

Table 5.44.3-1: IMD analysis for DC_4A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1755 | 3300 | 3800 |
| DL Frequency [MHz] | 2110 | 2155 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1545 | 2090 | 5010 | 5555 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 210 | 4845 | 5890 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7310 | 8310 | 9355 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 1965 | 8145 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3090 | 10020 | 11110 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9065 | 11610 | 13155 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11445 | 3720 | 3040 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6390 | 1335 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16955 | 10140 | 10820 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14910 | 11730 | 12865 |

The analysis shows there is no IMD issues of this uplink combination.

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.44.3-2 for DC_7A_n78A into band 4 downlink.

Table 5.44.3-2: IMD analysis for DC_7A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 3300 | 3800 |
| DL Frequency [MHz] | 2620 | 2690 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

The analysis shows the 4th order IMD product falls inside band 4 downlink.

Based on Table 5.44.3-1 and Table 5.44.3-2:

- The 4th order IMD product of DC_7_n78 falls inside band 4 downlink

### 5.44.4		∆TIB and ∆RIB values

For DC_4-7_n78, the TIB,c and RIB,c values are reused from DC_7-66_n78 and are given in the tables below.

Table 5.44.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_4-7_n78 | 0.6 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.44.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_4-7_n78 | 0.5 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.44.5	REFSENS requirements

MSD values are reused from DC_7A-66A_n78A. The test frequencies are not re-used.

Table 5.44.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_4A-7A_n78A | 4 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | N/A |

5.45	DC_66-71_n25

### 5.45.1	Configuration for DC

Table 5.45.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A-71A_n25A | DC_66A_n25A DC_71A_n25A |

### 5.45.2	Co-existence studies

There is no IMD impact from UL 71_n25 affecting DL band 66.

There is no IMD impact from UL 66_n25 affecting DL band 71.

### 5.45.3		∆TIB and ∆RIB values

For DC_66-71_n25, the TIB,c and RIB,c values are reused from CA_n25-n66-n71 and are given in the tables below.

Table 5.45.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66-71_n25 | 0.5 | 0.6 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.45.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66-71_n25 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.45.4	REFSENS requirements

No need to specify additional REFSENS exceptions according to the analysis in clause 5.45.2.

5.46	DC_2-71_n7

5.46.1	Configurations for DC

Table 5.46.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-71A_n7A | DC_2A_n7A DC_71A_n7A |

5.46.2	Co-existence studies

Based on co-existence studies of DC_2_n7 and DC_71_n7, own Rx impact of the 3rd band is the followings.

-	2nd and 5th order IMD products generated by DC_2_n7 uplink may fall into own Rx of band 71.

5.46.3	∆TIB and ∆RIB values

For DC_2-71_n7, the TIB,c and RIB,c values are reused from DC_2-71_n41 are given in the tables below.

Table 5.46.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-71_n7 | 0.5 | 0.6 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.46.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2-71_n7 | - | 0.2 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.46.4	Reference sensitivity exceptions

Table 5.46.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_2A-71A_n41A are reused.

Table 5.46.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-71A_n7A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 71 | 676 | 5 | 50 | 630 | 28.7 | IMD24 |
|  | n7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.47	DC_2-71_n77

5.47.1	Configurations for DC

Table 5.47.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-71A_n77A DC_2A-71A_n77(2A) | DC_2A_n77A DC_71A_n77A |

5.47.2	Co-existence studies

Based on co-existence studies of DC_2_n77 and DC_71_n77, own Rx impact of the 3rd band is the followings.

-	3rd and 4th order IMD products generated by DC_71_n77 uplink may fall into own Rx of band 2.

5.47.3	∆TIB and ∆RIB values

For DC_2-71_n77, the TIB,c and RIB,c values are reused from DC_2-71_n78 are given in the tables below.

Table 5.47.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-71_n77 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.47.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2-71_n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.47.4	Reference sensitivity exceptions

Table 5.47.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_2A-71A_n78A are reused.

Table 5.47.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-71A_n77A DC_2A-71A_n77(2A) | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD39 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | N/A |
| NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

5.48	DC_7-71_n77

5.48.1	Configurations for DC

Table 5.48.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-71A_n77A DC_7A-71A_n77(2A) | DC_7A_n77A DC_71A_n77A |

5.48.2	Co-existence studies

Based on co-existence studies of DC_7_n77 and DC_71_n77, own Rx impact of the 3rd band is the followings.

-	2nd and 3rd order IMD products generated by DC_71_n77 uplink may fall into own Rx of band 7.

5.48.3	∆TIB and ∆RIB values

For DC_7-71_n77, the TIB,c and RIB,c values are reused from DC_7-71_n78 are given in the tables below.

Table 5.48.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-71_n77 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.48.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-71_n77 | 0.2 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.48.4	Reference sensitivity exceptions

Table 5.48.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_7A-71A_n78A are reused.

Table 5.48.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-71A_n77A DC_7A-71A_n77(2A) | 7 | 2550 | 5 | 25 | 2670 | 29.6 | IMD21 |
|  | 71 | 680 | 5 | 25 | 634 | N/A | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

5.49	DC_66-71_n77

5.49.1	Configurations for DC

Table 5.49.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A-71A_n77A DC_66A-71A_n77(2A) | DC_66A_n77A DC_71A_n77A |

5.49.2	Co-existence studies

Based on co-existence studies of DC_66_n77 and DC_71_n77, own Rx impact of the 3rd band is the followings.

-	3rd and 4th order IMD products generated by DC_71_n77 uplink may fall into own Rx of band 66.

-    3rd order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 71.

5.49.3	∆TIB and ∆RIB values

For DC_66-71_n77, the TIB,c and RIB,c values are reused from DC_71_n66-n78 are given in the tables below.

Table 5.49.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66-71_n77 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.49.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66-71_n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.49.4	Reference sensitivity exceptions

Table 5.49.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_66A_n71A-n77A and DC_71A_n66A-n78A are reused.

Table 5.49.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-71A_n77A DC_66A-71A_n77(2A) | 66 | 1760 | 5 | 25 | 2160 | 15.5 | IMD39 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | 71 | 686 | 5 | 25 | 640 | 15.3 | IMD311 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | N/A |
| NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

5.50	DC_7-12_n77

5.50.1	Configurations for DC

Table 5.50.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-12A_n77A DC_7A-12A_n77A(2A) | DC_7A_n77A DC_12A_n77A |

5.50.2	Co-existence studies

Based on co-existence studies of DC_7_n77 and DC_12_n77, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_7_n77 uplink may fall into own Rx of band 12.

-     2nd and 3rd order IMD products generated by DC_12_n77 uplink may fall into own Rx of band 7.

5.50.3	∆TIB and ∆RIB values

For DC_7-12_n77, the TIB,c and RIB,c values are reused from DC_7-12_n78 are given in the tables below.

Table 5.50.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-12_n77 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.50.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-12_n77 | 0.2 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.50.4	Reference sensitivity exceptions

Table 5.50.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_7A-12A_n78A are reused.

Table 5.50.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-12A_n77A DC_7A-12A_n77(2A) | 7 | 2542 | 5 | 25 | 2662 | 29.6 | IMD21 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n77 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 12 | 710 | 5 | 25 | 740 | 30.8 | IMD2 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

5.51	DC_7-12_n25

5.51.1	Configurations for DC

Table 5.51.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-12A_n25A | DC_7A_n25A DC_12A_n25A |

5.51.2	Co-existence studies

Based on co-existence studies of DC_7_n25 and DC_12_n25, own Rx impact of the 3rd band is the followings.

-	5th order IMD products generated by DC_7_n25 uplink may fall into own Rx of band 12.

-     2nd order IMD products generated by DC_12_n25 uplink may fall into own Rx of band 7.

5.51.3	∆TIB and ∆RIB values

For DC_7-12_n25, the TIB,c and RIB,c values are reused from DC_7-13_n25are given in the tables below.

Table 5.51.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-12_n25 | 0.5 | 0.3 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.51.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-12_n25 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.51.4	Reference sensitivity exceptions

Table 5.51.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_7A-12A_n2A are reused.

Table 5.51.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-12A_n25A | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
|  | 12 | 701.5 | 5 | 25 | 731.5 | 5.3 | IMD5 |
|  | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | 30.8 | IMD2 |
|  | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |
|  | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |

5.52	DC_7-66_n12

5.52.1	Configurations for DC

Table 5.52.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-66A_n12A | DC_7A_n12A DC_66A_n12A |

5.52.2	Co-existence studies

Based on co-existence studies of DC_7_n12 and DC_66_n12 on own Rx impact of the 3rd band, there is no IMD interference for this band combination.

5.52.3	∆TIB and ∆RIB values

For DC_7-66_n12, the TIB,c and RIB,c values are reused from DC_12_n7-n66 are given in the tables below.

Table 5.52.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-66_n12 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.52.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-66_n12 | 0.5 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.52.4	Reference sensitivity exceptions

There is no IMD interference for this combination, thus there is no MSD requirement.

5.53	DC_7-71_n25

5.53.1	Configurations for DC

Table 5.53.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-71A_n25A | DC_7A_n25A DC_71A_n25A |

5.53.2	Co-existence studies

Based on co-existence studies of DC_7_n25 and DC_71_n25, own Rx impact of the 3rd band is the followings.

-	2nd and 5th order IMD products generated by DC_7_n25 uplink may fall into own Rx of band 71.

5.53.3	∆TIB and ∆RIB values

For DC_7-71_n25, the TIB,c and RIB,c values are reused from DC_7_n2-n71

Table 5.53.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-71_n25 | 0.5 | 0.3 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.53.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-71_n25 | 0.3 | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.53.4	Reference sensitivity exceptions

Table 5.53.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_7A_n2A-n71A are reused.

Table 5.53.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-71A_n25A | 7 | 2530 | 5 | 25 | 2530 | N/A | N/A |
|  | 71 | 676 | 5 | 25 | 630 | 28.7 | IMD24 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.54	DC_12-66_n7

5.54.1	Configurations for DC

Table 5.54.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A-66A_n7A | DC_12A_n7A DC_66A_n7A |

5.54.2	Co-existence studies

Based on co-existence studies of DC_12_n7 and DC_66_n7, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_66_n7 uplink may fall into own Rx of band 12.

5.54.3	∆TIB and ∆RIB values

For DC_12-66_n7, the TIB,c and RIB,c values are reused from DC_12-66_n41 are given in the tables below.

Table 5.54.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12-66_n7 | 0.6 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.54.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12-66_n7 | 0.5 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.54.4	Reference sensitivity exceptions

Table 5.54.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_12A-66A_n41A are reused.

Table 5.54.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-66A_n7A | 12 | 712 | 5 | 25 | 742 | 31 | IMD2 |
|  | 66 | 1773 | 5 | 25 | 2173 | N/A | N/A |
|  | n7 | 2515 | 5 | 25 | 2635 | N/A | N/A |

5.55	DC_66-71_n7

5.55.1	Configurations for DC

Table 5.55.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A-71A_n7A | DC_66A_n7A DC_71A_n7A |

5.55.2	Co-existence studies

Based on co-existence studies of DC_66_n7 and DC_71_n7 on own Rx impact of the 3rd band, there is no IMD interference for this band combination.

5.55.3	∆TIB and ∆RIB values

For DC_66-71_n7, the TIB,c and RIB,c values are reused from DC_66-71_n41 are given in the tables below.

Table 5.55.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66-71_n7 | 0.5 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.55.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66-71_n7 | 0.5 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.55.4	Reference sensitivity exceptions

There is no IMD interference for this combination, thus there is no MSD requirement.

5.56	DC_1A-3A_n105A

5.56.1	Operating bands for EN-DC

Table 5.56.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n105 | CA_1-3 | n105 | No |

5.56.2	Configuration for DC

Table 5.56.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n105A | DC_1A_n105A DC_3A_n105A | CA_1A-3A | n105A |

### 5.56.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.56.3-1 for DC_1A_n105A into band 3 downlink.

Table 5.56.3-1: IMD analysis for DC_1A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 663 | 703 |
| DL Frequency [MHz] | 2110 | 2170 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 | 1217 | 2583 | 2683 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3137 | 3297 | 654 | 514 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 | 4663 | 3246 | 3386 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5057 | 5277 | 9 | 189 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2434 | 2634 | 5166 | 5366 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 | 6643 | 3909 | 4089 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 892 | 672 | 7257 | 6977 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1731 | 1971 | 4614 | 4354 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 | 4792 | 8343 | 8623 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 | 6069 | 7086 | 7346 |

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.56.3-2 for DC_3A_n105A into band 1 downlink.

Table 5.56.3-2: IMD analysis for DC_3A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 663 | 703 |
| DL Frequency [MHz] | 1805 | 1880 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 | 1007 | 2373 | 2488 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2717 | 2907 | 459 | 304 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 | 4273 | 3036 | 3191 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4427 | 4692 | 204 | 399 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2014 | 2244 | 4746 | 4976 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 | 6058 | 3699 | 3894 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1102 | 867 | 6477 | 6137 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1311 | 1581 | 4029 | 3724 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4597 | 7503 | 7843 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 5679 | 6456 | 6761 |

Based on Table 5.56.3-1 and Table 5.56.3-2:

- The 5th order IMD product of DC_1_n105 falls inside band 3 downlink

- The 4th order IMD product of DC_3_n105 falls inside band 1 downlink

### 5.56.4		∆TIB and ∆RIB values

For DC_1-3_n105, the TIB,c and RIB,c values are taken from fallbacks.

Table 5.56.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-3_n105 | 0.3 | 0.3 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.56.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-3_n105 | - | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.56.5	REFSENS requirements

MSD values are reused from DC_1A-3A_n71A and DC_1A-3A_n28A. The test frequencies are not re-used.

Table 5.56.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n105A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 3 | 1760 | 5 | 25 | 1855 | 4 | IMD5 |
|  | n105 | 695 | 5 | 25 | 644 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | 5 | IMD4 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n105 | 695 | 5 | 25 | 644 | N/A | N/A |

5.57	DC_1A-7A_n105A

5.57.1	Operating bands for EN-DC

Table 5.57.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-7_n105 | CA_1-7 | n105 | No |

5.57.2	Configuration for DC

Table 5.57.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A_n105A | DC_1A_n105A DC_7A_n105A | CA_1A-7A | n105A |

### 5.57.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.57.3-1 for DC_1A_n105A into band 7 downlink.

Table 5.57.3-1: IMD analysis for DC_1A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 663 | 703 |
| DL Frequency [MHz] | 2110 | 2170 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 | 1217 | 2583 | 2683 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3137 | 3297 | 654 | 514 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 | 4663 | 3246 | 3386 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5057 | 5277 | 9 | 189 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2434 | 2634 | 5166 | 5366 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 | 6643 | 3909 | 4089 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 892 | 672 | 7257 | 6977 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1731 | 1971 | 4614 | 4354 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 | 4792 | 8343 | 8623 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 | 6069 | 7086 | 7346 |

Based on Table 5.57.3-1:

- The 2nd order IMD product of DC_1_n105 falls inside band 7 downlink

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.57.3-2 for DC_7A_n105A into band 1 downlink.

Table 5.57.3-2: IMD analysis for DC_7A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 663 | 703 |
| DL Frequency [MHz] | 2620 | 2690 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1797 | 3163 | 3273 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4297 | 4477 | 1244 | 1094 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5663 | 5843 | 3826 | 3976 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6797 | 7047 | 581 | 391 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3594 | 3814 | 6326 | 6546 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8163 | 8413 | 4489 | 4679 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 312 | 82 | 9617 | 9297 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 3151 | 6384 | 6094 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5152 | 5382 | 10663 | 10983 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6989 | 7249 | 8826 | 9116 |

The analysis shows there is no IMD issues of this uplink combination.

### 5.57.4		∆TIB and ∆RIB values

For DC_1-7_n105, the TIB,c and RIB,c values are taken from fallbacks.

Table 5.57.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-7_n105 | 0.5 | 0.6 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.57.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-7_n105 | - | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.57.5	REFSENS requirements

MSD values are reused from CA_n1-n7-n28. The test frequencies are not re-used.

Table 5.57.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-7A_n105A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 7 | 2553 | 5 | 25 | 2673 | 30 | IMD2 |
|  | n105 | 698 | 5 | 25 | 647 | N/A | N/A |

5.58	DC_3A-7A_n105A

5.58.1	Operating bands for EN-DC

Table 5.58.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n105 | CA_3-7 | n105 | No |

5.58.2	Configuration for DC

Table 5.58.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n105A | DC_3A_n105A DC_7A_n105A | CA_3A-7A | n105A |

### 5.58.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.58.3-1 for DC_3A_n105A into band 7 downlink.

Table 5.58.3-1: IMD analysis for DC_3A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 663 | 703 |
| DL Frequency [MHz] | 1805 | 1880 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 | 1007 | 2373 | 2488 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2717 | 2907 | 459 | 304 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 | 4273 | 3036 | 3191 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4427 | 4692 | 204 | 399 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2014 | 2244 | 4746 | 4976 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 | 6058 | 3699 | 3894 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1102 | 867 | 6477 | 6137 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1311 | 1581 | 4029 | 3724 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4597 | 7503 | 7843 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 5679 | 6456 | 6761 |

The analysis shows there is no IMD issues of this uplink combination.

A study of the uplink combination affecting the downlink of the third band is shown in Table 5.58.3-2 for DC_7A_n105A into band 3 downlink.

Table 5.58.3-2: IMD analysis for DC_7A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 663 | 703 |
| DL Frequency [MHz] | 2620 | 2690 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1797 | 3163 | 3273 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4297 | 4477 | 1244 | 1094 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5663 | 5843 | 3826 | 3976 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6797 | 7047 | 581 | 391 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3594 | 3814 | 6326 | 6546 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8163 | 8413 | 4489 | 4679 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 312 | 82 | 9617 | 9297 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 3151 | 6384 | 6094 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5152 | 5382 | 10663 | 10983 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6989 | 7249 | 8826 | 9116 |

Based on Table 5.58.3-1 and Table 5.58.3-2:

- The 2nd order IMD product of DC_7_n105 falls inside band 3 downlink

### 5.58.4		∆TIB and ∆RIB values

For DC_3-7_n105, the TIB,c and RIB,c values are taken from fallbacks.

Table 5.58.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-7_n105 | 0.5 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.58.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-7_n105 | - | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.58.5	REFSENS requirements

MSD values are reused from CA_n3-n7-n28. The test frequencies are not re-used.

Table 5.58.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-7A_n105A | 3 | 1780 | 5 | 25 | 1875 | 16.5 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n105 | 675 | 5 | 25 | 624 | N/A | N/A |

5.59	DC_8-39_n40

5.59.1	Configurations for DC

Table 5.59.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A-39A_n40A | DC_8A_n40A DC_39A_n40A |

5.59.2	Co-existence studies

Co-existence analysis for DC_39_n40 UL in TR 37.716-11-11 shows IMD4 and IMD5 produce impact to NR Band 8 DL.

Co-existence analysis for DC_8_n40 UL in TR 37.863-01-01 shows IMD5 produce impact to NR Band n39 DL.

5.59.3	∆TIB and ∆RIB values

For DC_8-39_n40, the TIB,c and RIB,c values are reused from DC_8_n39-n40 and given in the tables below.

Table 5.59.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8-39_n40 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.59.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8-39_n40 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.59.4	Reference sensitivity exceptions

Based on co-existence studies, there is IMD5 products cause by DC_8_n40 will falls into band 39. However, simultaneous Rx/Tx is not supported for DC_39_n40, which means synchronization between band 39 and n40. Therefore no additional MSD is needed to be defined

For the IMD4 and IMD5 MSD caused by DC_39_n40 fall into B8, the MSD values are proposed as follow:

Table 5.59.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-39A_n40A | 8 | 895 | 5 | 25 | 940 | 8.6 | IMD4 |
|  | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n40 | 2370 | 5 | 25 | 2370 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 4.9 | IMD5 |
|  | 39 | 1890 | 5 | 25 | 1890 | N/A | N/A |
|  | n40 | 2370 | 5 | 25 | 2370 | N/A | N/A |

5.60	DC_66-71_n2

5.60.1	Configurations for DC

Table 5.60.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A-71A_n2(2A) | DC_66A_n2A DC_71A_n2A |

5.60.2	Co-existence studies

Table 5.60.2-1 lists the UL configuration of DC_66_n2 2nd to 5th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.60.2-1: UL DC_66_n2 harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 | 70 | 3560 | 3690 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1920 | 2110 | 1510 | 1710 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5410 | 5600 | 5270 | 5470 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3770 | 4020 | 3220 | 3490 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7260 | 7510 | 6980 | 7250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 140 | 400 | 7120 | 7380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5270 | 4930 | 5930 | 5620 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1640 | 1310 | 2310 | 1990 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8690 | 9030 | 9110 | 9420 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8830 | 9160 | 8970 | 9290 |

Based on Table 5.60.2-1, the IMD3 from UL DC_66_n2 may affect DL band 2.

The IMD5 from UL DC_66_n2 may affect DL band 66.

Table 5.60.2-2 lists the UL configuration of DC_71A_n2A 2nd to 5th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.60.2-2: DC_71A_n2A UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 1989 | 2094 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 2652 | 2792 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 3315 | 3490 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1247 | 1152 | 2513 | 2608 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3002 | 3157 | 584 | 454 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4363 | 4518 | 3176 | 3306 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4852 | 5067 | 79 | 244 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6213 | 6428 | 3839 | 4004 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2304 | 2494 | 5026 | 5216 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 942 | 742 | 6977 | 6702 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1606 | 1831 | 4404 | 4154 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4502 | 4702 | 8063 | 8338 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5689 | 5914 | 6876 | 7126 |

Based on Table 5.60.2-1, there is no harmonic or IMD impact from UL 71_n2 affecting DL band 71, 2 and n66.

5.60.3	∆TIB and ∆RIB values

For DC_66-71_n2, the TIB,c and RIB,c values are reused from CA_n2-n66-n71 and are given in the tables below.

Table 5.60.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66-71_n2 | 0.5 | 0.3 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.60.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66-71_n2 | 0.3 | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.60.4	Reference sensitivity exceptions

Compared to its fallback modes, no additional MSD requirements for this band combination are needed.

5.61		DC_8-39_n41

    - 5.61.1	Configurations for DC

Table 5.61.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A-39A_n41A DC_8A-39A_n41C | DC_8A_n41A DC_39A_n41A |

    - 5.61.2	Co-existence studies

In terms of the co-existence studies of the fallbacks of DC_8_n41 and DC_39_n41, which are captured in the TR 37.863-01-01, it can be found that:

- No IMD products produced  by Band 8 and Band n41 that impact the reference sensitivity of LTE band 39.

- No IMD products produced  by Band 39 and Band n41 that impact the reference sensitivity of LTE band 8.

    - 5.61.3	∆TIB and ∆RIB values

For DC_8-39_n41, the TIB,c and RIB,c values are reused from DC_8_n39-n41 and given in the tables below.

Table 5.61.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8-39_n41 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.61.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8-39_n41 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

    - 5.61.4	Reference sensitivity exceptions

According to the co-existence study, no MSD issues need to be specified. Therefore, no additional REFENS requirements need to be specified.

5.62	DC_7-66_n2

5.62.1	Configurations for DC

Table 5.62.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-66A_n2(2A) | DC_7A_n2A DC_66A_n2A |

5.62.2	Co-existence studies

Table 5.62.2-1 lists the UL configuration of DC_7_n2 2nd to 5th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.62.2-1: UL DC_7_n2 harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 590 | 720 | 4350 | 4480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1130 | 1320 | 3090 | 3290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6200 | 6390 | 6850 | 7050 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2980 | 3230 | 5590 | 5860 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8050 | 8300 | 9350 | 9620 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1440 | 1180 | 8700 | 8960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8430 | 8090 | 5140 | 4830 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4010 | 3680 | 730 | 410 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11850 | 12190 | 9900 | 10210 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11200 | 11530 | 10550 | 10870 |

There is no harmonic or IMD impact from UL DC_7_n2 affecting DL band n2, 7 and 66.

Table 5.62.2-2 lists the UL configuration of DC_5_n41 2nd to 5th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.62.2-2: DC_66_n2 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 | 70 | 3560 | 3690 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1920 | 2110 | 1510 | 1710 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5410 | 5600 | 5270 | 5470 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3770 | 4020 | 3220 | 3490 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7260 | 7510 | 6980 | 7250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 140 | 400 | 7120 | 7380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5270 | 4930 | 5930 | 5620 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1640 | 1310 | 2310 | 1990 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8690 | 9030 | 9110 | 9420 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8830 | 9160 | 8970 | 9290 |

The IMD3 from UL_66_n2 may affect DL band 2.

The IMD5 from UL_66_n2 may affect DL band 66.

There is no IMD impact from UL_66_n2 affecting DL band 7.

5.62.3	∆TIB and ∆RIB values

For DC_7-66_n2, the TIB,c and RIB,c values are reused from CA_2-7-66 and are given in the tables below.

Table 5.62.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-66_n2 | 0.5 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.62.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-66_n2 | 0.5 | 0.5 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.62.4	Reference sensitivity exceptions

Compared to its fallback modes, no additional MSD requirements for this band combination are needed.

5.63	DC_3-40_n77

### 5.63.1	Configurations for DC

Table 5.63.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-40A_n77A DC_3A-40C_n77A | DC_3A_n77A DC_40A_n77A |

### 5.63.2	Co-existence studies

Based on co-existence studies of DC_3_n77 and DC_40_n77, own Rx impact of the 3rd band is the followings.

-	2nd and 5th order IMD products generated by DC_3_n77 uplink may fall into own Rx of band 40.

-   2nd and 4th order IMD products generated by DC_40_n77 uplink may fall into own Rx of band 3.

### 5.63.3	∆TIB and ∆RIB values

For DC_3-40_n77, the TIB,c and RIB,c values are reused from DC_3_n40-n77 are given in the tables below.

Table 5.63.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-40_n77 | 0.6 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.63.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-40_n77 | 0.2 | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.63.4	Reference sensitivity exceptions

Table 5.63.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n3A-n40A-n77A and CA_n3-n41-n77 are reused.

Table 5.63.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-40A_n77A DC_3A-40C_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 40 | 2310 | 5 | 25 | 2310 | 29.4 | IMD2 |
|  | n77 | 4030 | 10 | 50 | 4030 | N/A | N/A |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 40 | 2350 | 5 | 25 | 2350 | 5.3 | IMD5 |
|  | n77 | 3755 | 10 | 50 | 3755 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 29.9 | IMD29 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

5.64	DC_5-40_n77

### 5.64.1	Configurations for DC

Table 5.64.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-40A_n77A DC_5A-40C_n77A DC_5A-40A_n77C DC_5A-40C_n77C | DC_5A_n77A DC_40A_n77A |

### 5.64.2	Co-existence studies

Based on co-existence studies of DC_5_n77 and DC_40_n77, own Rx impact of the 3rd band is the followings.

-	3th order IMD products generated by DC_5_n77 uplink may fall into own Rx of band 40.

-   3th and 5th order IMD products generated by DC_40_n77 uplink may fall into own Rx of band 5.

### 5.64.3	∆TIB and ∆RIB values

For DC_5-40_n77, the TIB,c and RIB,c values are reused from DC_5_n40-n77 are given in the tables below.

Table 5.64.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5-40_n77 | 0.6 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.64.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5-40_n77 | 0.2 | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.64.4	Reference sensitivity exceptions

Table 5.64.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n5-n40-n78 and DC_5A_n40A-n77A are reused.

Table 5.64.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5-40_n77 | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | 40 | 2355 | 5 | 25 | 2355 | 13.2 | IMD3 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | 15.2 | IMD34 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.65	DC_5-40_n78

### 5.65.1	Configurations for DC

Table 5.65.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-40A_n78A DC_5A-40C_n78A DC_5A-40A_n78C DC_5A-40C_n78C | DC_5A_n78A DC_40A_n78A |

### 5.65.2	Co-existence studies

Based on co-existence studies of DC_5_n78 and DC_40_n78, own Rx impact of the 3rd band is the followings.

-	No IMD products generated by DC_5_n78 uplink may fall into own Rx of band 40.

-   3th order IMD products generated by DC_40_n78 uplink may fall into own Rx of band 5.

### 5.65.3	∆TIB and ∆RIB values

For DC_5-40_n78, the TIB,c and RIB,c values are reused from DC_5_n40-n78 are given in the tables below.

Table 5.65.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5-40_n78 | 0.6 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.65.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5-40_n78 | 0.2 | 0.4 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.65.4	Reference sensitivity exceptions

Table 5.65.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n5-n40-n78 are reused.

Table 5.65.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5-40_n78 | 5 | 835 | 5 | 25 | 880 | 15.2 | IMD3 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n78 | 3740 | 10 | 50 | 3740 | N/A | N/A |

5.66	DC_2-5_n41

5.66.1	Configurations for DC

Table 5.66.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A_n41A | DC_2A_n41A DC_5A_n41A |
| DC_2A-2A-5A_n41A | DC_2A_n41A DC_5A_n41A |

5.66.2	Co-existence studies

Based on co-existence studies of DC_2_n41 and DC_5_n41, there is no own Rx impact of the 3rd band for this combination.

5.66.3	∆TIB and ∆RIB values

For DC_2-5_n41, the TIB,c and RIB,c values are reused from CA_n2-n5-n41 are given in the tables below.

Table 5.66.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-5_n41 DC_2-2-5_n41 | 0.5 | 0.6 | 0.41 / 0.92 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.66.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2-5_n41 DC_2-2-5_n41 | - | 0.2 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.66.4	Reference sensitivity exceptions

Based on co-existence studies in section 5.66.2, the er is no need to define MSD requirements.

5.67	DC_2-7_n12

5.67.1	Configurations for DC

Table 5.67.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A_n12A | DC_2A_n12A DC_7A_n12A |
| DC_2A-2A-7A_n12A | DC_2A_n12A DC_7A_n12A |

5.67.2	Co-existence studies

Table 5.67.2-1 lists the UL configuration of DC_2A_n12A 2nd to 5th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.67.2-1: UL DC_2A_n12A harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 699 | 716 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1398 | 1432 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2097 | 2148 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 2796 | 2864 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 3495 | 3580 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1211 | 1134 | 2549 | 2626 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2984 | 3121 | 512 | 418 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4399 | 4536 | 3248 | 3342 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4834 | 5031 | 187 | 298 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6249 | 6446 | 3947 | 4058 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2268 | 2422 | 5098 | 5252 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1014 | 886 | 6941 | 6684 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1552 | 1723 | 4332 | 4118 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4646 | 4774 | 8099 | 8356 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5797 | 5968 | 6948 | 7162 |

Based on Table 5.67.2-1, the IMD2 from UL 2_n12, that is, 1*fx +1*fy, may affect DL band 7.

Table 5.67.2-2 lists the UL configuration of DC_7A_n12A 2nd to 5th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.67.2-2: Band 7 and Band n12 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 699 | 716 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1398 | 1432 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2097 | 2148 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 2796 | 2864 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 3495 | 3580 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1871 | 1784 | 3199 | 3286 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4284 | 4441 | 1172 | 1068 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5699 | 5856 | 3898 | 4002 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6784 | 7011 | 473 | 352 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8199 | 8426 | 4597 | 4718 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3568 | 3742 | 6398 | 6572 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 364 | 226 | 9581 | 9284 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2852 | 3043 | 6312 | 6068 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5296 | 5434 | 10699 | 10996 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7097 | 7288 | 8898 | 9142 |

From the above table, there is no IMD impact from UL 7_n12 affecting DL band 7, 2 and n12.

5.67.3	∆TIB and ∆RIB values

For DC_2-7_n12, the TIB,c and RIB,c values are reused from DC_2-7_n28 are given in the tables below.

Table 5.67.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-7_n12 DC_2-2-7_n12 | 0.5 | 0.5 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.67.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2-7_n12 DC_2-2-7_n12 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.67.4	Reference sensitivity exceptions

The MSD value of DC_7A-12A_n2A is reused for DC_2A-7A_n12A and given as follow:

Table 5.59-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-7A_n12A | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | 30.8 | IMD2 |
|  | n12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |

5.68	DC_5-7_n25

5.68.1	Configurations for DC

Table 5.68.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-7A_n25A | DC_5A_n25A DC_7A_n25A |

5.68.2	Co-existence studies

Based on co-existence studies of DC_5_n25 and DC_7_n25, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_5_n25 uplink may fall into own Rx of band 7.

5.68.3	∆TIB and ∆RIB values

For DC_5-7_n25, the TIB,c and RIB,c values are reused from CA_n2-n5-n41 are given in the tables below.

Table 5.68.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5-7_n25 | 0.6 | 0.4 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.68.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5-7_n25 | 0.2 | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.68.4	Reference sensitivity exceptions

Table 5.68.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n2-n5-n41 are reused.

Table 5.68.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-7A_n25A | 5 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | 30.0 | IMD2 |
|  | n25 | 830 | 5 | 25 | 875 | N/A | N/A |

5.69	DC_5-66_n25

5.69.1	Configurations for DC

Table 5.69.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-66A_n25A | DC_5A_n25A DC_66A_n25A |

5.69.2	Co-existence studies

Based on co-existence studies of DC_5_n25 and DC_66_n25, own Rx impact of the 3rd band is the followings.

-	4th order IMD products generated by DC_5_n25 uplink may fall into own Rx of band 66.

5.69.3	∆TIB and ∆RIB values

For DC_5-66_n25, the TIB,c and RIB,c values are reused from DC_5-66_n2 are given in the tables below.

Table 5.69.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5-66_n25 | 0.3 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.69.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5-66_n25 | - | 0.3 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.69.4	Reference sensitivity exceptions

Table 5.69.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_5A-66A_n2A are reused.

Table 5.69.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-66A_n25A | 5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | 66 | 1732 | 5 | 25 | 2132 | 7.2 | IMD4 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |

5.70	DC_5-66_n41

5.70.1	Configurations for DC

Table 5.70.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-66A_n41A | DC_5A_n41A DC_66A_n41A |

5.70.2	Co-existence studies

Based on co-existence studies of DC_5_n41 and DC_66_n41, own Rx impact of the 3rd band is the followings.

-	2nd and 3rd order IMD products generated by DC_66_n41 uplink may fall into own Rx of band 5

5.70.3	∆TIB and ∆RIB values

For DC_5-66_n41, the TIB,c and RIB,c values are reused from CA_n5-n41-n66 are given in the tables below.

Table 5.70.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5-66_n41 | 0.6 | 0.5 | 0.81 / 1.32 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |  |

Table 5.70.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5-66_n41 | 0.2 | 0.5 | 0.51 / 12 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |  |

5.70.4	Reference sensitivity exceptions

Table 5.70.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n5A-n41A-n66A and DC_5A-66A_n7A are reused.

Table 5.70.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-66A_n41A | 5 | 830 | 5 | 25 | 875 | 28.9 | IMD2 |
|  | 66 | 1765 | 5 | 25 | 2165 | N/A | N/A |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | 18.0 | IMD3 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | N/A |

5.71	DC_7-71_n12

5.71.1	Configurations for DC

Table 5.71.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-71A_n12A | DC_7A_n12A |

5.71.2	Co-existence studies

Based on co-existence studies of DC_7_n12, there is no own Rx impact of the 3rd band for this combination.

5.71.3	∆TIB and ∆RIB values

For DC_7-71_n12, the TIB,c and RIB,c values are reused from DC_7-28_n5 are given in the tables below.

Table 5.71.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7-71_n12 | 0.5 | 0.5 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.71.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7-71_n12 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.71.4	Reference sensitivity exceptions

Based on the co-existence study in 5.71.2, there is no need to define any MSD values.

5.72	DC_12-71_n2

5.72.1	Configurations for DC

Table 5.72.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A-71A_n2A | DC_12A_n2A DC_71A_n2A |

5.72.2	Co-existence studies

Based on co-existence studies of DC_12_n2 and DC_71_n2, own Rx impact of the 3rd band is the followings.

-	5th order IMD products generated by DC_71_n2 uplink may fall into own Rx of band 12.

5.72.3	∆TIB and ∆RIB values

For DC_12-71_n2, the TIB,c and RIB,c values are reused from DC_2-5_n71are given in the tables below.

Table 5.72.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12-71_n2 | 0.5 | 0.5 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.72.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12-71_n2 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.72.4	Reference sensitivity exceptions

Table 5.72.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_2A-5A_n71A are reused.

Table 5.72.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-71A_n2A | 12 | 713.5 | 5 | 25 | 743.5 | 4.2 | IMD5 |
|  | 71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |

5.73	DC_12-71_n77

5.73.1	Configurations for DC

Table 5.73.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A-71A_n77A | DC_12A_n77A DC_71A_n77A |

5.73.2	Co-existence studies

Based on co-existence studies of DC_12_n77 and DC_71_n77, own Rx impact of the 3rd band is the followings.

-	5th order IMD products generated by DC_71_n77 uplink may fall into own Rx of band 12.

-	5th order IMD products generated by DC_12_n77 uplink may fall into own Rx of band 71.

5.73.3	∆TIB and ∆RIB values

For DC_12-71_n77, the TIB,c and RIB,c values are reused from CA_n5-n12-n77 are given in the tables below.

Table 5.73.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12-71_n77 | 0.4 | 0.8 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.73.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12-71_n77 | 0.3 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.73.4	Reference sensitivity exceptions

Table 5.73.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n5A-n12A-n77A are reused.

Table 5.73.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-71A_n77A | 12 | 702 | 5 | 25 | 732 | 4.4 | IMD5 |
|  | 71 | 667 | 5 | 25 | 621 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 12 | 701.5 | 5 | 25 | 731.5 | N/A | N/A |
|  | 71 | 690 | 5 | 25 | 644 | 3.9 | IMD5 |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | N/A |

5.74	DC_66-71_n12

5.74.1	Configurations for DC

Table 5.74.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A-71A_n12A | DC_66A_n12A |

5.74.2	Co-existence studies

Based on co-existence studies of DC_66_n12, there is no own Rx impact of the 3rd band for this combination.

5.74.3	∆TIB and ∆RIB values

For DC_66-71_n2, the TIB,c and RIB,c values are reused from DC_5-66_n71 are given in the tables below.

Table 5.74.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66-71_n12 | 0.3 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.74.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66-71_n12 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.74.4	Reference sensitivity exceptions

Based in the co-existence studies in 5.74.2, there is no need to define MSD values.

5.75	DC_3-28_n1

5.75.1	Configurations for DC

Table 5.75.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink configuration |
| --- | --- |
| DC_3C-28A_n1A | DC_3A_n1A DC_3C_n1A DC_28A_n1A |

Note that UL DC_3A_n1A, DC_28A_n1A are already specified for this configuration in the specification.

5.75.2	Co-existence studies

The 2nd, 3rd and 4th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products had already been analyzed and specified for UL configuration DC_3A_n1A and DC_28A_n1A, respectively. Regarding UL configuration DC_3C_n1A, the following analysis is presented.

Table 5.75.2-3: IMD (due to triple beats) analysis for UL configuration DC_3C_n1A

| UE UL carriers | Band 3 case #1 |  | Band 3 case #2 |  | Band n1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | fx1_low | fx2_low | fx1_high | fx2_high | fy_low | fy_high |
| UL frequency (MHz) | 1710 | 1750 | 1745 | 1785 | 1920 | 1980 |
| TB1 | ¦fx1_low-fx2_low+fy_low ¦ | ¦fx1_low-fx2_low+fy_high ¦ | ¦fx1_high-fx2_high+fy_low ¦ | ¦fx1_high-fx2_high+fy_high ¦ |  |  |
|  | 1880 | 1940 | 1880 | 1940 |  |  |
| TB2 | ¦-fx1_low+fx2_low+fy_low ¦ | ¦-fx1_low+fx2_low+fy_high ¦ | ¦-fx1_high+fx2_high+fy_low ¦ | ¦-fx1_high+fx2_high+fy_high¦ |  |  |
|  | 1960 | 2020 | 1960 | 2020 |  |  |
| TB3 | ¦-fx1_low-fx2_low+fy_low ¦ | ¦-fx1_low-fx2_low+fy_high ¦ | ¦-fx1_high-fx2_high+fy_low ¦ | ¦-fx1_high-fx2_high+fy_high ¦ |  |  |
|  | 1540 | 1480 | 1610 | 1550 |  |  |
| TB4 | ¦fx1_low+fx2_low+fy_low ¦ | ¦fx1_low+fx2_low+fy_high ¦ | ¦fx1_high+fx2_high+fy_low ¦ | ¦fx1_high+fx2_high+fy_high ¦ |  |  |
|  | 5380 | 5440 | 5450 | 5510 |  |  |

As we can see from the above table, there is no IMD to band 3, 28 or n1.

5.75.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values for DC_3-28_n1 have already been specified in TS 38.101-3.

5.75.4	Reference sensitivity exceptions

Compared to its fallback modes, no additional MSD requirements for this band combination are needed.

5.76	DC_4-5_n78

### 5.76.1	Configuration for DC

Table 5.76.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink configuration |
| --- | --- |
| DC_4A-5A_n78A | DC_4A_n78A DC_5A_n78A |

5.76.2  Co-existence studies

Based on co-existence studies of DC_4_n78 and DC_5_n78, own Rx impact of the 3rd band is the followings.

-	3nd order IMD products generated by DC_5_n78 uplink may fall into own Rx of band 4.

5.76.3		∆TIB and ∆RIB values

For DC_4-5_n78, the TIB,c values are reused from CA_n3-n5-n78 and are given in the tables below.

Table 5.76.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_4-5_n78 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

For DC_4-5_n78, the RIB,c values are reused from CA_n1-n5-n78 and are given in the tables below.

Table 5.x.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_4-5_n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.76.4	Reference sensitivity exceptions

Reusing the coexistence study results from EN-DC combination CA_n1-n5-n78, MSD values should be defined as below:

Table 5.76.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_4A-5A_n78A | 4 | N/A | 5 | N/A | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |

  - 5.77			DC_8-39_n79

    - 5.77.1	Configurations for DC

Table 5.77.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A-39A_n79A DC_8A-39A_n79C | DC_8A_n79A DC_39A_n79A |

    - 5.77.2	Co-existence studies

In terms of the co-existence studies of the fallbacks of DC_8_n79 and DC_39_n79, which are captured in the TR 37.863-01-01, it can be found that:

- IMD4 products produced  by Band 8 and Band n79 that impact the reference sensitivity of LTE band 39.

- IMD3, IMD4 products produced  by Band 39 and Band n79 that impact the reference sensitivity of LTE band 8.

    - 5.77.3	∆TIB and ∆RIB values

For DC_8-39_n79, the TIB,c and RIB,c values are reused from DC_8_n39-n79 and given in the tables below.

Table 5.77.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8-39_n79 | 0.3 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.77.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8-39_n79 | - | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

    - 5.77.4	Reference sensitivity exceptions

According to the co-existence study, the MSD values are defined as below, in which the corresponding MSD values are reused from the NR 3DL/2UL CA_n8A-n39A-n79A

Table 5.77.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8-39_n79 | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 39 | N/A | 5 | N/A | 1907.5 | 13.8 | IMD4 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 15.1 | IMD3 |
|  | 39 | 1900 | 10 | 50 | 1900 | N/A | N/A |
|  | n79 | 4740 | 40 | 216 | 4740 | N/A | N/A |
|  | 8 | N/A | 5 | N/A | 940 | 7.1 | IMD4 |
|  | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n79 | 4760 | 40 | 216 | 4760 | N/A | N/A |

5.78	DC_3-7_n79, DC_3-3-7_n79, DC_3-7-7_n79, DC_3-3-7-7_n79

5.78.1	Configurations for DC

Table 5.78.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A_n79A5 | DC_3A_n79A DC_7A_n79A |
| DC_3A-3A-7A_n79A5 | DC_3A_n79A DC_7A_n79A |
| DC_3A-7A-7A_n79A5 | DC_3A_n79A DC_7A_n79A |
| DC_3A-3A-7A-7A_n79A5 | DC_3A_n79A DC_7A_n79A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

5.78.2	Co-existence studies

Based on the co-existence studies for the DC_3_n79, DC_7_n79, which are studied in section 6.6 of TR 37.863-01-01 and section 6.1.67 of TR 37.71-11-11, the own Rx impact on the 3rd band is listed below.

- IMD2 and IMD5 generated by dual uplink of Band 3 + Band n79 will fall into own Rx of Band 7.

- IMD2 and IMD5 generated by dual uplink of Band 7 + Band n79 will fall into own Rx of Band 3.

5.78.3	∆TIB and ∆RIB values

For DC_3-7_n79, DC_3-3-7_n79, DC_3-7-7_n79, DC_3-3-7-7_n79, the TIB,c and RIB,c values are provided in the table below.

Table 5.78.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-7_n79 DC_3-3-7_n79 DC_3-7-7_n79 DC_3-3-7-7_n79 | 0.5 | 0.5 | 0.8 |

Table 5.78.3-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-7_n79 DC_3-3-7_n79 DC_3-7-7_n79 DC_3-3-7-7_n79 | - | - | 0.5 |

5.78.4	Reference sensitivity exceptions

Since based on the latest guideline in TR 38.846, the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications. Therefore, both MSD for IMD2 and IMD5 of DC_3A-7A_n79A series are proposed in the table below. The values are reused from the DC_3-41_n79 which is studied in TR 37.716-21-11 with the full set of requirements.

Table 5.78.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-7A_n79A DC_3A-3A-7A_n79A DC_3A-7A-7A_n79A DC_3A-3A-7A-7A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4440 | 10 | 50 | 4440 | N/A | N/A |
|  | 7 | N/A | 5 | N/A | 2670 | 30.2 | IMD2 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4440 | 10 | 50 | 4440 | N/A | N/A |
|  | 7 | N/A | 5 | N/A | 2640 | 5.0 | IMD5 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n79 | 4420 | 10 | 50 | 4420 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1855 | 29.4 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n79 | 4745 | 10 | 50 | 4745 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1840 | 4.8 | IMD5 |

5.79	DC_3-5_n28

5.79.1	Configurations for DC

Table 5.79.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-5A_n28A | DC_3A_n28A DC_5A_n28A |

5.79.2	Co-existence studies

Based on co-existence studies of DC_3_n28 and DC_5_n28, own Rx impact of the 3rd band is the followings:

-	4th order IMD products generated by DC_5_n28 uplink may fall into own Rx of band 3.

5.79.3	∆TIB and ∆RIB values

For DC_3-5_n28, the TIB,c and RIB,c values are reused from DC_3-28_n5 are given in the tables below.

Table 5.79.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-5_n28 | 0.3 | 0.7 | 0.7 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.79.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-5_n28 | - | 0.2 | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.79.4	Reference sensitivity exceptions

Table 5.79.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_3A-28A_n5A are reused.

Table 5.79.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-5A_n28A | 3 | N/A | 5 | N/A | 1829.5 | 8.7 | IMD4 |
|  | 5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |

5.80	DC_5-7_n28

5.80.1	Configurations for DC

Table 5.80.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-7A_n28A | DC_5A_n28A DC_7A_n28A |

5.80.2	Co-existence studies

Based on co-existence studies of DC_7_n28 and DC_5_n28, own Rx impact of the 3rd band is the followings:

-	5th order IMD products generated by DC_5_n28 uplink may fall into own Rx of band 7.

5.80.3	∆TIB and ∆RIB values

For DC_5-7_n28, the TIB,c and RIB,c values are reused from DC_7A-20A_n28A are given in the tables below.

Table 5.80.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5-7_n28 | 0.7 | 0.3 | 0.7 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.80.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5-7_n28 | 0.2 | - | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.80.4	Reference sensitivity exceptions

Table 5.80.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_7A-20A_n28A are reused.

Table 5.80.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-7A_n28A | 5 | 842 | 5 | 25 | 887 | N/A | N/A |
|  | 7 | N/A | 5 | N/A | 2640 | 5.9 | IMD5 |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |

5.81	DC_1-5_n28

5.81.1	Configurations for DC

Table 5.81.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-5A_n28A | DC_1A_n28A DC_5A_n28A |

5.81.2	Co-existence studies

Based on co-existence studies of DC_1_n28 and DC_5_n28, own Rx impact of the 3rd band is the followings:

-	5th order IMD products generated by DC_1_n28 uplink may fall into own Rx of band 5.

-	5th order IMD products generated by DC_5_n28 uplink may fall into own Rx of band 1.

5.81.3	∆TIB and ∆RIB values

For DC_1-5_n28, the TIB,c and RIB,c values are reused from CA_n1-n18-n28 are given in the tables below.

Table 5.81.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-5_n28 | 0.3 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.81.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-5_n28 | - | - | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

5.81.4	Reference sensitivity exceptions

Table 5.81.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n1-n18-n28 are reused.

Table 5.81.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-5A_n28A | 1 | N/A | 5 | N/A | 2123 | 4 | IMD5 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 1 | 1965 | 5 | 25 | 2155 | N/A | N/A |
|  | 5 | N/A | 5 | N/A | 875 | 4.6 | IMD5 |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-8 | RAN4#104-e | R4-2212499 |  |  |  | TR skeleton | 0.0.1 |
| 2022-8 | RAN4#104-e | R4-2212500 |  |  |  | The following approved TPs were implemented: R4-2215008 TP for TR 37 718-21-11 to include DC_1-(n)7 R4-2215009 TP for TR 37 718-21-11 to include DC_3-(n)7 R4-2215010 TP for TR 37 718-21-11 to include DC_28-(n)7 R4-2215011 TP for TR 37 718-21-11 to include DC_1-26_n78 R4-2215012 TP for TR 37 718-21-11 to include DC_3-26_n78 R4-2215013 TP for TR 37 718-21-11 to include DC_7-26_n78 R4-2215031 TP for TR 37.718-21-11 DC_1A-8A_n7A | 0.1.0 |
| 2022-10 | RAN4#104-bis-e | R4-2216246 |  |  |  | The following approved TPs were implemented: R4-2215771 TP for TR 37.718-21-11: support of DC_3A-8B_n78A, DC_3A-3A-8B_n78A R4-2215772 TP for TR 37.718-21-11: support of DC_7A-8B_n78A, DC_7A-7A-8B_n78A R4-2215849 TP for TR 37.718-21-11 to include DC_1_3-n26 R4-2215850 TP for TR 37.718-21-11 to include DC_1_7-n26 R4-2217104 TP for TR 37.718-21-11 to include DC_3_7-n26 R4-2215901 TP for TR 37.718-21-11: Including band combinations DC_3-41_n1 R4-2215902 TP for TR 37.718-21-11: Including band combinations DC_8-41_n78 R4-2216092 TP for TR 37 718-21-11 to include DC_20-41_n1 R4-2216093 TP for TR 37 718-21-11 to include DC_20-41_n78 R4-2216161 TP for TR 37.718-21-11: DC_1-7_n1 R4-2217067 TP for TR 37.718-21-11: DC_1-7_n20 R4-2217068 TP for TR 37.718-21-11: DC_1-8_n20 R4-2217071 TP for TR 37.718-21-11: DC_3-20_n3 R4-2217072 TP for TR 37.718-21-11: DC_3-32_n7 R4-2217075 TP for TR 37.718-21-11: DC_8-28_n3 R4-2217077 TP for TR 37.718-21-11: DC_20-32_n7 R4-2216249 TP for TR 37.718-21-11: update the format of tables for ?TIB and ?RIB values R4-2216591 TP for TR 37.718-21-11 DC_7A-8A_n7A R4-2216625 TP for TR 37.718-21-11 on table templates and error corrections | 0.2.0 |
| 2022-11 | RAN4#105 | R4-2219165 |  |  |  | The following approved TPs were implemented: R4-2218525 TP for TR 37.718-21-11 DC_3A-5A_n40A R4-2218958 TP for 37.718-21-11 to include DC_1-28_n38 R4-2218959 TP for 37.718-21-11 to include DC_3-28_n38 R4-2219282 TP for TR 37.718-21-11: DC_20-28_n78 R4-2219338 TP for TR 37.718-21-11: DC_1-28_n20 R4-2220607 TP for TR 37.718-21-11 DC_1A-5A_n40A R4-2220773 TP for TR 37.718-21-11: DC_1-3_n1 R4-2220774 TP for TR 37.718-21-11: DC_1-20_n1 R4-2220775 TP for TR 37.718-21-11: DC_7-8_n20 R4-2220776 TP for TR 37.718-21-11: DC_7-28_n20 | 0.3.0 |
| 2023-03 | RAN4#106 | R4-2301454 |  |  |  | The following approved TPs were implemented: R4-2220568 TP for TR 37.718-21-11: DC_3-7_n26 R4-2216086 TP for TR 37 718-21-11 to update DC_3-(n)7 R4-2300180 TP for TR 37.718-21-11 to include DC_5-7_n40 R4-2303610 TP for 37.718-21-11 to include DC_20-(n)3 R4-2301769 TP for TR 37.718-21-11 DC_3A-8A_n7A | 0.4.0 |
| 2023-04 | RAN4#106bis-e | R4-2305310 |  |  |  | The following approved TPs were implemented: R4-2306483 TP for TR 37.718-21-11: support of DC_3A_8B_n78A, DC_3A-3A_8B_n78A with UL DC_8B_n78A R4-2305311 TP for TR 37.718-21-11: update the tables for ?TIB and ?RIB values R4-2305576 TP for TR 37.718-21-11: support of DC_7A_8B_n78A, DC_7A-7A_8B_n78A with UL DC_8B_n78A R4-2306491 TP for 37.718-21-11 to include DC_3-67_n3 R4-2306492 TP for 37.718-21-11 to include DC_67-(n)3 R4-2306493 TP for 37.718-21-11 to include DC_20-67_n3 R4-2306652 TP for TR 37.718-21-11 DC_3A-8A_n41A | 0.5.0 |
| 2023-05 | RAN4#107 | R4-2307767 |  |  |  | R4-2310331 TP for 37.718-11-21 to include DC_1A-3A_n105A R4-2310333 TP for 37.718-11-21 to include DC_1A-7A_n105A R4-2310337 TP for 37.718-11-21 to include DC_3A-7A_n105A R4-2310298 TP for TR 37.718-21-11: updated MSD for DC_3A_8B_n78A with UL DC_8B_n78A R4-2310326 TP to TR 37.718-21-11 Addition of DC_2_4_n78 R4-2310327 TP to TR 37.718-21-11 Addition of DC_28_38_n78 R4-2310328 TP to TR 37.718-21-11 Addition of DC_4_7_n78 R4-2308145 TP for TR 37.718-21-11 DC_66A-71A_n25A R4-2310339 TP for TR 37.718-21-11: updates for DC_3A-8A_n41A R4-2310340 TP for 37.718-21-11 to include DC_2A-71A_n7A R4-2309519 TP for 37.718-21-11 to include DC_2A-71A_n77A R4-2309520 TP for 37.718-21-11 to include DC_7A-71A_n77A R4-2310341 TP for 37.718-21-11 to include DC_66A-71A_n77A R4-2309522 TP for 37.718-21-11 to include DC_7A-12A_n77A R4-2309523 TP for 37.718-21-11 to include DC_7A-12A_n25A R4-2309524 TP for 37.718-21-11 to include DC_7A-66A_n12A R4-2309525 TP for 37.718-21-11 to include DC_7A-71A_n25A R4-2309528 TP for 37.718-21-11 to include DC_12A-66A_n7A R4-2309529 TP for 37.718-21-11 to include DC_66A-71A_n7A | 0.6.0 |
| 2023-08 | RAN4#108 | R4-2312324 |  |  |  | The following approved TPs were implemented: R4-2314815 TP for 37.718-21-11 to add DC_2A-7A_n12A R4-2314816 TP to 37.718-21-11 to add DC_66A-71A_n2(2A) R4-2311466 TP to 37.718-21-11to add DC_2A-5A_n41A R4-2311467 TP to 37.718-21-11 to add DC_7A-66A_n2(2A) R4-2314824 TP for TR 37.718-21-11 DC_3-40_n77 R4-2311952 TP for TR 37.718-21-11 DC_5-40_n77 R4-2311953 TP for TR 37.718-21-11 DC_5-40_n78 R4-2314841 TP for 37.718-21-11 to include DC_2A-5A_n41A R4-2314842 TP for 37.718-21-11 to include DC_2A-7A_n12A R4-2313152 TP for 37.718-21-11 to include DC_5A-7A_n25A R4-2314843 TP for 37.718-21-11 to include DC_5A-66A_n25A R4-2314844 TP for 37.718-21-11 to include DC_5A-66A_n41A R4-2314845 TP for 37.718-21-11 to include DC_7A-71A_n12A R4-2314846 TP for 37.718-21-11 to include DC_12A-71A_n2A R4-2314847 TP for 37.718-21-11 to include DC_12A-71A_n77A R4-2314848 TP for 37.718-21-11 to include DC_66A-71A_n12A | 0.7.0 |
| 2023-10 | RAN4#108bis | R4-2316389 |  |  |  | The following approved TPs were implemented: R4-2316334 TP to TR37.718-21-11 to add DC_8A-39A_n40A R4-2316335 TP to TR37.718-21-11 to add DC_8A-39A_n41A and DC_8A-39A_n41C R4-2317672 TP for TR 37.718-21-11 DC_3C-28A_n1A R4-2316390 TP for TR 37.718-21-11: merge the subclauses for DC_2-5_n41 and DC_2-7_n12 | 0.8.0 |
| 2023-11 | RAN4#09 | R4-319702 |  |  |  | The following approved TPs were implemented: R4-2319517 TP for TR 37.718-21-11 to introduce DC_4A-5A_n78A R4-2321896 TP to TR37.718-21-11 to add DC_8A-39A_n79A and DC_8A-39A_n79C R4-2320588 TP for TR 37.718-21-11: support of DC_3_7_n79, DC_3-3-7_n79, DC_3-7-7_n79, DC_3-3-7-7_n79 R4-2319673 TP for 37.718-21-11 to include DC_3A-5A_n28A R4-2320852 TP for 37.718-21-11 to include DC_5A-7A_n28A R4-2321894 TP for 37.718-21-11 to include DC_1A-5A_n28A | 0.9.0 |