3GPP TSG-RAN WG4 Meeting #109	R4-2319728

Chicago, USA, November 13-17, 2023

Agenda Item:	8.13.2.3

Source:	Ericsson

Title:	Final simulation results for ATG non-synchronized scenarios

Type:	Other

Document for:	Information

# 1	Introduction

In the previous meeting, we presented our preliminary simulation results [1] after the offline calibration between the companies. In this document, we present our final co-existence simulation results for non-synchronized scenarios for both non-subarray and subarray antenna models. We focus on the results using the Free Space Pathloss channel model between TN BS and ATG BS.

# 2	Discussion

## Final simulation results for non-synchronized scenarios

This section discusses the impact of boresight angles – 0, 30 and 60 degrees between TN and ATG BS, with no TN wrap for both non-subarray and subarray antenna models. The common simulation settings can be found in the TR [3]. For the results in the following sections, we have assumed uniform distribution for ATG UEs between 3 and 10 km in altitude, 8-column non-subarray TN and ATG BS antennas and 16x1 array ATG UE antennas.

The propagation model used for simulations in this document between ATG BS and TN BS is Free Space Pathloss model.

#### Scenario 5: 4GHz ATG DL interfering TN UL using FSPL model

In Scenario 5, Figure 1 shows the results for the non-subarray antenna configuration. As can be seen from the figure, isolation distance needed to maintain the 5% throughput degradation criteria for ATG network is 290 km for the zero-degree case, 39 km for the thirty-degree case, and 15 km for the sixty-degree case.

Figure 1: Isolation distance for Scenario 5, non-subarray model

Figure 2 shows the results for the Subarray antenna configuration. As can be seen from the figure, isolation distance needed to maintain the 5% throughput degradation criteria for ATG network is 350 km for the zero-degree case, 42 km for the thirty-degree case, and 25 km for the sixty-degree case.

Figure 2: Isolation distance for Scenario 5, Subarray model

Table 1 below summarizes the results as per the agreed template.

Table 1: Simulation results for Scenario 5 – 4GHz ATG DL interfering TN UL using FSPL model

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| Ericsson | Non-Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 290 | 39 | 15 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 60 | 14 | 8 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 350 | 42 | 25 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 70 | 14 | 9 |

#### Scenario 7: 4GHz TN DL interfering ATG UL using FSPL model

In Scenario 7, Figure 3 shows the results for the non-subarray antenna configuration. As can be seen from the figure, isolation distance needed to maintain the 5% throughput degradation criteria for ATG network is more than 500 km for the zero-degree case, 70 km for the thirty-degree case, and 35 km for the sixty-degree case.

Figure 3: Isolation distance for Scenario 7, non-subarray model

Figure 4 shows the results for the Subarray antenna configuration. As can be seen from the figure, isolation distance needed to maintain the 5% throughput degradation criteria for ATG network is more than 500 km for the zero-degree case, 70 km for the thirty-degree case, and 70 km for the sixty-degree case.

Figure 4: Isolation distance for Scenario 7, Subarray model

Table 2 below summarizes the results as per the agreed template.

Table 2: Simulation results for Scenario 7 – 4GHz TN DL interfering ATG UL using FSPL model

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| Ericsson | Non-Subarray | 5% in the whole network | > 500 | 70 | 35 |
|  |  | Average of all users in the whole network | 290 | 41 | 25 |
|  | Subarray | 5% in the whole network | > 500 | 70 | 70 |
|  |  | Average of all users in the whole network | 400 | 47 | 43 |

#### Scenario 14: 2GHz TN DL interfering ATG UL using FSPL model

In Scenario 14, Figure 5 shows the results for the non-subarray antenna configuration the isolation distance needed to maintain the throughput degradation of the ATG network at 5% is, more than 500 km for the zero-degree case, 130 km for the thirty-degree case, and around 90 km for the sixty-degree case.

Figure 5: Isolation distance for Scenario 14, non-subarray model

Figure 6 shows the results for the Subarray antenna configuration. As can be seen from the figure, isolation distance needed to maintain the 5% throughput degradation criteria for ATG network is more than 500 km for the zero-degree case, 140 km for the thirty-degree case, and 120 km for the sixty-degree case.

Table 3 below summarizes the results as per the agreed template.

Table 3: Simulation results for Scenario 14 – 2GHz TN DL interfering ATG UL using FSPL model

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| Ericsson | Non-Subarray | 5% in the whole network | > 500 | 130 | 90 |
|  |  | Average of all users in the whole network | > 500 | 80 | 41 |
|  | Subarray | 5% in the whole network | > 500 | 140 | 120 |
|  |  | Average of all users in the whole network | > 500 | 90 | 70 |

For scenarios 5,7 and 14, the isolation distances that needs to be maintained for the 5% throughput degradation criteria between TN and ATG BS is in hundreds of kilometers using FSPL model for both non-subarray and subarray antenna configurations.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For scenarios 5,7 and 14, the isolation distances that needs to be maintained for the 5% throughput degradation criteria between TN and ATG BS is in hundreds of kilometers using FSPL model for both non-subarray and subarray antenna configurations.

# References

R4-2315445, Co-existence simulation results for non-synchronized scenarios, Ericsson, RAN4#108-bis

R4-2315904, 3GPP TR 38.876 V0.6.0 (2023-10), RAN4#108-bis

R4-2317719, WF on ATG Co-existence evaluation, CMCC, RAN4#108-bis