3GPP TSG-RAN WG4 Meeting #109	R4-2319739

Chicago, IL, USA, 13 November – 17 November 2023

Agenda Item:	8.11.4

Source:	Ericsson

Title:	UE demodulation requirements for non-colocated NR-CA deployment scenario

Document for:	Discussion

# 1	Introduction

RAN4#108bis agreed with the way forward on the UE demodulation requirements for intra-band non-colocated EN-DC/NR-CA deployments [1], where RAN4 agreed to define new Type 2 UE demodulation requirements with rank 2 for PCell and rank 1 for SCell.

This contribution discusses the MCS indexes configured for PDSCH demodulation requirements for Type 2 UE to complete the Type 2 UE demodulation requirements.

# 2	Discussion

RAN4#108bis agreed with the simulation assumption of PDSCH CA demodulation requirements to verify the feature of Type 2 UE as follows [1].

Agreement: Type 2 UE NR-CA demodulation requirements. Received power difference; 25 dB. Received time difference; 33 µs. Channel model; AWGN (TS38.101-4 B.1.1). Maximum number of HARQ transmission; 1. Signal/Noise setting; Set both Es and Noc to set SNR test points. Throughput measurement procedure; Measure both carriers. Antenna configuration; 2x2 for both carriers. MCS table and MCS index; FFS. Test metric; 70% of the maximum throughput. Rank; MCS table; MCS index. Carrier with lower power; Rank 1; MCS table 1; Option 1: MCS4 (QPSK, 0.30) Option 2: MCS5 (QPSK, 0.37). Carrier with higher power; Rank 2; MCS table 2; Option 1: MCS22 (256QAM, 0.74) Option 2: MCS23 (256QAM, 0.78) Option 3: MCS24 (256QAM, 0.82) Option 4: MCS25 (256QAM, 0.86). Note the required SNR difference between the carriers with lower power and higher power should not exceed 25dB.

The remaining open issue is the MCS configuration for each carrier.

Table 1	Simulation results.

| MCS table | MCS index | Rank | Alignment results (Tx EVM 0%) (For information) | Alignment results (With Tx EVM) | Impairment results |
| --- | --- | --- | --- | --- | --- |
| Table 1 | 4 | 1 | -4.1 | -4.1 | -2.1 |
| Table 1 | 5 | 1 | -3.2 | -3.2 | -1.2 |
| Table 2 | 22 | 2 | 20.2 | 20.7 | 22.7 |
| Table 2 | 23 | 2 | 21.7 | 22.2 | 24.2 |
| Table 2 | 24 | 2 | 22.7 | 23.2 | 25.2 |
| Table 2 | 25 | 2 | 23.5 | 24.3 | 26.3 |

Table 2	Test points analysis.

| SCell test point (dB) | PCell test point (dB) | Difference |
| --- | --- | --- |
| -2.1 (MCS4) | 22.7 (MCS22) | 24.8 |
|  | 24.2 (MCS23) | 26.3 |
|  | 25.2 (MCS24) | 27.3 |
|  | 26.3 (MCS25) | 28.4 |
| -1.2 (MCS5) | 22.7 (MCS22) | 23.9 |
|  | 24.2 (MCS23) | 25.4 |
|  | 25.2 (MCS24) | 26.4 |
|  | 26.3 (MCS25) | 27.5 |

Table 1 summarizes our simulation results for PCell and SCell according to the simulation assumption. Note we assumed TxEVM 6% for QPSK and 3% for 256QAM. Table 2 is the difference of SNR values between PCell and SCell based on our impairment results. Considering the UE RF core requirements, SNR difference between lower-power carrier and higher-power carrier should not exceed 25dB. According to our impairment simulation results, it is observed the possible option is MCS4 (64QAM table) and MCS22 (256QAM table).

Observation: The following MCS pair does not exceed 25dB.

- MCS table 1, MCS4

- MCS table 2, MCS22

As we agreed in RAN4#108bis, however, the MCS pair should be derived based on the companies impairment results. We propose to consider our simulation results to decide the requirements for Type 2 UE PDSCH demodulation requirements.

# 3	Summary

Observation: The following MCS pair does not exceed 25dB.

- MCS table 1, MCS4

- MCS table 2, MCS22

Proposal: Consider our simulation results to decide the requirements for Type 2 UE PDSCH demodulation requirements.

# References

- R4-2316917, “WF on NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson.

# Appendix

## A.1	UE type candidates for non-colocated NR-CA/EN-DC deployment scenario

| UE Type | CC# | antenna / LNA | Mixer | Analog BB | #Rx | NRCA / ENDC | power imbalance | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 (NR) | 4 shared | 4 shared | 4 shared | 4Rx | NR-CA EN-DC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 (LTE/NR) |  |  |  | 4Rx |  |  |  |
| 2 | 1 (NR) | 2 | 2 | 2 | 2Rx | NR-CA EN-DC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 (LTE/NR) | 2 | 2 | 2 | 2Rx |  |  |  |