3GPP TSG-RAN WG4 Meeting #109	R4-2319742

Chicago, IL, USA, 13 – 17 November 2023

Agenda Item:	8.12.3.3

Source:	Ericsson

Title:	UE demodulation requirements for FR2 HST multi-Rx reception

Document for:	Discussion

# 1	Introduction

RAN4#108bis agreed with the WF on the UE demodulation requirements for Rel-18 FR2 HST [1]. This contribution shows our simulation results and discusses the remaining open issues on the simultaneous multi-Rx reception scenario.

# 2	Discussion

## 2.1	Performance impacts due to received timer and power differences

RAN4#108bis discussed the receiver assumption for FR2 HST multi-Rx simultaneous reception scenario and agreed to set the received time difference (RTD) between signals from two TRPs. However RAN4 did not agree with the exact RTD value and there are many options.

Issue 3-1-0: Test requirement to be defined - Test cases to be introduced - Option 1: one case with RTD larger than CP - Option 2: two cases based on UE declaration on supported baseband processing with RTD larger than CP or not - Case 1: RTD = 1.0 CP - Case 2: RTD larger than CP

Issue 3-1-1: UE processing assumption for the FFT window Issue 3-1-2: Necessity to introduce RTD into channel model for CPE FFT processing test Issue 3-2-1: Number of MCS for PDSCH requirement for multi-Rx reception Issue 3-2-2: Number of SNR for PDSCH requirement for multi-Rx reception Introduce RTD in the FR2 HST PDSCH requirement between the different RX panels. Discuss RTD value based on evaluation. - Note RTD: Timing offset of TRP2 from TRP1 Define one fixed MCS value per each Panel for PDSCH requirement with multi-Rx reception with fixed FRC - Encourage companies to evaluate the performance difference under assumption on FFT (single FFT across Rx chains, and separate FFT per RF chain) with the following test setup and test metric, make a decision in RAN4#109 meeting - RTD for evaluation with the following priority order - Option 1: (2CP) 1.2 us - Option 2: (1 CP): 0.57us - Option 3a (1.2CP): 0.7us and Option 3b: (2.5CP) 1.5us - MCS - Set MCS 19 for TRP1 and Set MCS Y for TRP2 for FR2 HST simultaneous multi-Rx scenario. - Candidate MCS Y = {MCS 11, MCS 13} - Other feasible MCS are not precluded. - Expected power imbalance value and RTD value according to the deployment model (depending on UE Location) can be considered to derive suitable MCS pair (MCS 19, MCS Y). For example - Step 1: Derive the UE location based on given RTD value; - Step 2: Derive the expected power imbalance value x dB based on the UE location - Step 3: Run simulation to find SNR pair (SNR1, SNR2) for MCS pair (MCS 19, MCS Y) derived based on test metric 70% of TP for each PDSCH. - Choose the highest MCS Y according to the following condition: - SNR1(MCS19) – SNR2(MCS Y) =< x dB - To be decided whether SNR based on averaged submitted simulation results or individual SNR values reported. - Test metric for SNR derivation - Option 1: 70% Tput for each PDSCH as baseline. - Option 2: 70% Tput across all PDSCH - Step 1: Run simulation to find one SNR = y dB so that SNR pair {y dB, y-x dB} derived based on MCS pair (MCS 19, MCS Y) is fulfilled the following test metric - (Tput1 from TRP1 (MCS19) + Tput2 from TRP2 (MCS Y)) > 70% * (Tput1_max from TRP1 + Tput2_max from TRP2)

| (a) Absolute RTD and received power difference | (b) CDF of absolute RTD (with regard to CP (0.59us)). |
| --- | --- |

Figure 1	Absolute RTD and received power difference for FR2 HST Scenario B-2.

Figure 1 (a) shows the absolute RTD according and corresponding received power difference according to the UE location for FR2 HST Scenario B-2. Figure 1 (b) shows the CDF of absolute RTD in Figure 1 for UE location from 0m to 700m. The figure shows RTD < 1.0 x CP for 54% of the test time, in other words, RTD > 1.0 x CP for 46% of the test time.

Observation 1: For FR2 HST deployment scenario B, RTD > 1.0 x CP for 46% of the test time.

Table 1	Relation among UE location, RTD, and receiver power difference.

|  | RTD | UE location | Received power difference | CDF |
| --- | --- | --- | --- | --- |
| Option 1 | 1.2 x CP | 333m | 4.9dB | 0.62 |
| Option 2 | 0.57 x CP | 47m | 1.2dB | 0.30 |
| Option 3a | 0.7 x CP | 382m | 2.8dB | 0.38 |
| Option 3b | 2.5 x CP | 200m | 10.6dB | 1.00 |
|  | 1.4 x CP | 313m | 5.8dB | 0.70 |
|  | 1.8 x CP | 272m | 7.6dB | 0.80 |
|  | 2.2 x CP | 230m | 9.4dB | 0.90 |

Table 1 summarizes the relation between RTD and corresponding received power difference. For example, RTD is 1.2 x CP at UE location of 333m, and the received power difference is 4.9dB at this location. Moreover, RTD is 1.2 x CP or less for 62% of the test time. For reference, we show the RTD for 70%-ile, 80%-ile, and 90%-ile of the test time.

## 2.2	Evaluation results

This section shows our simulation results for multi-Rx simultaneous reception scenario according to the parameters in Table 2 and simulation assumption [2].

Table 2	PDSCH simulation parameters

| Parameters | TRP#1 | TRP#2 |
| --- | --- | --- |
| Carrier frequency | 30 GHz |  |
| SCS/CBW | 120kHz / 200MHz |  |
| TDD Pattern | DDDSU, S=10D2G2U Schedule PDSCH in special slots |  |
| Antenna configuration | 2x2 | 2x2 |
| TRS periodicity | 10 ms (80 slots) | 10 ms (80 slots) |
| Number of active TCI States | 1 | 1 |
| Number of additional DMRS | 2 | 2 |
| rank | 2 | 2 |
| MCS index (MCS table 1) | 19 | Option 1: 11 Option 2: 13 |
| RTD (us) from TRP#1 | N/A | Option 1: 1.2 x CP Option 2: 0.57 x CP Option 3a: 0.7 x CP 2.5 x CP 1.4 x CP 1.8 x CP 2.2 x CP |

| (a) MCS19 for TRP#1, MCS11 for TRP#2 | (b) MCS19 for TRP#1, MCS13 for TRP#2 |
| --- | --- |

Figure 2	Simultaneous multi-Rx panel reception simulation results with 2+2 (per Rx panel).

Table 3	Summary of HST simulation results with RTD

| RTD | Power difference | MCS11 (SNR diff from TRP#1) | MCS13 (SNR diff from TRP#1) |
| --- | --- | --- | --- |
| 0 (Separate processing) | 0dB | 4.4dB (6.7dB) | 6.2dB (4.9dB) |
| 1.2 x CP | 4.9dB | 6.2dB (4.9dB) | 8.0dB (3.1dB) |
| 0.57 x CP | 1.2dB | 4.5dB (6.6dB) | 6.3dB (4.8dB) |
| 0.7 x CP | 2.8dB | 4.6dB (6.5dB) | 6.4dB (4.7dB) |
| 2.5 x CP | 10.6dB | Not achieve the peak rate | Not achieve the peak rate |
| 1.4 x CP | 5.8dB | 8.3dB (2.8dB) | 9.5dB (1.6dB) |
| 1.8 x CP 2.2 x CP | 7.6dB 9.4dB | Not achieve the peak rate | Not achieve the peak rate |
| Note 1: SNR to achieve 70% of pear rate for TRP1 is 11.1dB |  |  |  |

Figure 2 shows our ideal simulation results for TRP#2 with MCS11/MCS13 and different RTD options assuming the single FFT to process the signals from both TRP#1 and TRP#2 at the same time. For reference, we also show the simulation result for MCS19 from TRP#2. Table 3 summarizes the simulation results with regard to SNR to achieve 70% of the peak rate. Note the RTD=0 corresponds to the results with the separate FFT processing per TRP.

Firstly, it is observed the peak rate is not reached for TRP#2 with RTD > 1.8 x CP with the single FFT processing. We have observed the significant performance degradation for RTD = 1.4 x CP compared with the separate FFT processing. From the simulation results, MCS11 with RTD = 1.2 x CP is most aligned with Scenario B because the SNR difference between TRP#1 and TRP#2 if we consider the single FFT processing.

Observation 2: With single FFT assumption, performance degradation with RTD = 0.7 x CP or RTD = 0.57 x CP is negligible.

Observation 3: With single FFT assumption, performance degradation with RTD = 1.2 x CP is about 2dB for both MCS11 and MCS13, compared with separate FFT assumption.

Observation 4: With single FFT assumption, the peak rake is not reached with RTD = 2.5 x CP.

## 2.3	UE demodulation requirements

As we discussed in RAN4#108bis, in this WI, RAN4 is going to define PDSCH demodulation requirements for UE supporting at least the following capabilities:

- UE Power Class 6 (PC6)

- simultaneousReceptionFR2HST-r18

- maxNumberActiveTCI-PerBWP > 1

- multiDCI-MultiTRP-r16

- overlapPDSCHsFullyFreqTime-r16

According to the UE capability assumption, the demodulation requirements use the mDCI-based full-overlapping transmission with two TRPs, where each TRP can schedule different PDSCH codeword independently. This means it is possible to measure PDSCH throughput per TRP, and it is suitable especially for power imbalanced testing. We therefore propose to define UE demodulation requirements for FR2 HST simultaneous multi-Rx reception for each TRP.

Proposal 1: Define PDSCH demodulation requirements for FR2 HST simultaneous muti-Rx reception case as the SNR to achieve 70% of the maximum throughput for each TRP.

Regarding the requirements, considering the CDF of RTD shown in Figure 1 (b), we prefer to set RTD = 1.2 x CP or more because it covers about 60% of the test time with Scenario B.

Proposal 2: Configure RTD = 1.2 x CP (~0.7us) or more between TRP#1 and TRP#2 for PDSCH demodulation requirements for FR2 HST simultaneous muti-Rx reception case.

Regarding the final demodulation requirements, RAN4 is still discussing the receiver assumption: single FFT across Rx chains or separate FFT per Rx chain. According to our simulation results shown in Table 3, it is observed the performance degradation for the single FFT assumption with RTD = 1.2 x CP is about 2dB compared with the separate FFT assumption. We don’t think it is negligible to define the RAN4 demodulation requirements. From the deployment scenario and RRM agreements, we believe UE supporting FR2 HST simultaneous multi-Rx reception should be capable of separate FFT processing per TRP. Therefore we prefer to set the demodulation requirements based on the assumption of separate FFT processing.

Proposal 3: Define PDSCH demodulation requirements for FR2 HST simultaneous muti-Rx reception case with the assumption UE is capable of the separate FFT processing per TRP.

According to our simulation results in Table 3, MCS13 is suitable for TRP#2 because the SNR difference between TRP#1 and TRP#2 becomes 4.9dB, and this is aligned with RTD = 1.2 x CP.

Proposal 4: Configure MCS19 for TRP#1 and MCS13 for TRP#2.

If companies have strong preference to keep the single FFT processing option, one possible option is to define two requirements: one is based on the separate FFT processing, and another is based on the single FFT processing. Like the FR2 Multi-Rx chain DL reception WI, since gNB configuration does not change regardless of single or separate FFT processing, RAN4 may assume to introduce new UE feature for FR2 HST multi-Rx reception if necessary.

# 3	Summary

Observation 1: For FR2 HST deployment scenario B, RTD > 1.0 x CP for 46% of the test time.

Observation 2: With single FFT assumption, performance degradation with RTD = 0.7 x CP or RTD = 0.57 x CP is negligible.

Observation 3: With single FFT assumption, performance degradation with RTD = 1.2 x CP is about 2dB for both MCS11 and MCS13, compared with separate FFT assumption.

Observation 4: With single FFT assumption, the peak rake is not reached with RTD = 2.5 x CP.

Proposal 1: Define PDSCH demodulation requirements for FR2 HST simultaneous muti-Rx reception case as the SNR to achieve 70% of the maximum throughput for each TRP.

Proposal 2: Configure RTD = 1.2 x CP (~0.7us) or more between TRP#1 and TRP#2 for PDSCH demodulation requirements for FR2 HST simultaneous muti-Rx reception case.

Proposal 3: Define PDSCH demodulation requirements for FR2 HST simultaneous muti-Rx reception case with the assumption UE is capable of the separate FFT processing per TRP.

Proposal 4: Configure MCS19 for TRP#1 and MCS13 for TRP#2.

# References

- R4-2317007, “WF on FR2 HST demodulation requirements”, Samsung.

- R4-2313920, “Simulation assumption for FR2 HST demodulation”, Ericsson.

# Appendix: Channel model used for FR2 HST multi-Rx simultaneous reception

| Panel 1 | fst=fdcosθ(t) cosθt=Ds_offset+Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθ(t)=cost modDsv, t>Dsv |
| --- | --- |
| Panel 2 | fst=fdcosθ(t) cosθt=-Ds-Ds_offset+vtDmin2+Ds-Ds_offset+vt2, 0<t≤2Ds_offsetv--Ds_offset+vtDmin2+-Ds_offset+vt2, 2Ds_offsetv<t≤Dsv cosθ(t)=cost modDsv, t>Dsv |

| Parameters | Description | Scenario B-2 |
| --- | --- | --- |
| Ds | Inter-RRH distance | 700 m |
| Dmin | Distance between rail track and RRH | 150 m |
| v | Train velocity | 350 km/h |
| fd | Maximum Doppler frequency shift | 9722 Hz |
| fc | Carrier frequency | 30 GHz |
| Drx_panel | Distance between two UE Rx panels | 0 m |
| Ds_offset | Switching transmission point between adjacent RRHs | 100 m |